# 3GPP TSG SA2 Meeting #142E	S2-2008379

# November 16 – 20, 2020, Electronic meeting 	(revision of S2-200xxxx)

Source:	NTT DOCOMO, Qualcomm Incorporated

Title:	KI#1: Conclusions update

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: Conclude the Method 4 for BMCA procedure. Add a possibility for locally configured PTP port states, instead of BMCA evaluation.

# 1		Discussion

1) The conclusions for BMCA procedure in KI#1 in clause 8.1 has 2 open Editor’s Notes:

Editor’s note: Whether the DS-TT or NW-TT regenerates the Announce messages for the Master ports in DS-TT(s) is FFS.

Editor's note:	Whether and how the DS-TT filters the receive Announce/Sync messages are FFS.

The rationale for the ENs was to reduce the number of Announce messages that are sent within the 5GS and thereby reduce the UP traffc.

However, such optimization would not help much in terms of UP traffic reduction. Most of the gPTP related traffic comes from the Sync / Follow_Up messages, where the default value in gPTP is to send them once per 125 ms, i.e. 8 times a second.  This means 16 gPTP messages per second per port. Announce messages are sent once per second (default value in gPTP). So even if all Announces messages are removed, 94% (16:17) of the gPTP messages are still left.

When considering the size of the pPTP messages; Follow_Up is  96 bytes, Sync is 44 bytes, and Announce is 68 bytes in minimum. In one second, 140 bytes x 8 = 1120 bytes are sent for Sync + Follow_Up messages against 68 bytes for Announce (Ethernet header is not included here).  Again, 94% of the traffic is left if Announces are removed.

Therefore the proposal is to remove the ENs, and conclude that the NW-TT generates the Announce messages on behalf of the DS-TT ports, and DS-TT does not filter any received Announce messages.

Note that the above does not apply to a scenario when the DS-TT acts as a GM; in this case the DS-TT generates the Sync / Follow_Up messages and should also generate the Announce messages. This is already concluded in 8.1.

2) In Rel-16 the PTP port states are locally configured and not evaluated by BMCA procedure. It it proposed to retain the same functionality also in Rel-17, i.e. the PTP port states can be either locally configured, or evaluated by the BMCA procedure.

3) The conclusion for BMCA does not describe a scenario when the TSN GM is external to 5GS, it is proposed to add this; NW-TT should generate the Announce messages on behalf of the DS-TT master ports.

4) When the DS-TT(s) are configured to act as grandmaster, the NW-TT ports are either in Disabled or Passive state, i.e. do not send Announce messages.

# 2		Proposal

The following change is proposed for TR 23.700-20.

*** Start Change ***

## 7.1	Key Issue #1: Uplink Time Synchronization

Solution #1 is the only solution addressing KI#1 Uplink Time Synchronization.

The solution describes four alternative methods for BMCA procedure for gPTP:

Method 1:

-	Most of the BMCA logic is centralized to the TSN AF (or NEF for non-TSN sessions), but also DS-TT and NW-TT participate to BMCA operation.

-	DS-TT/NW-TT ingress port sends the received Announce message to BMCA function in TSN AF (or NEF) only if the received Announce is equal or better than the current Announce the bridge is sending.

-	Ingress port detects the Announce and Sync reception timeouts and indicates the timeout to TSN AF (or NEF).

-	DS-TT/NW-TT port is aware of the gPTP port state (Master, Slave, Passive)

-	Signalling consideration: since the received Announce messages are sent from the ingress port to TNS AF (or NEF) for BMCA evaluation via PMIC/BMIC (control plane), the solution is feasible only if the ingress port pre-screens the received Announce message and sends the message only if it is better than the current Announce known to the port. In this case no Announce messages need to be sent to the TSN AF after the BMCA procedure has stabilized. Compared to other methods, this one has more signalling between TSN AF and NW-TT/DS-TT, and between NW-TT and DS-TT.

Method 4:

-	BMCA logic is centralized to the NW-TT. Ingress ports forward all Announce messages to BMCA function in NW-TT.

-	Ingress ports forward all Sync messages to NW-TT.

-	NW-TT generates Announce messages on behalf of the egress ports.

-	NW-TT detects the Announce and Sync reception timeouts for each port.

-	DS-TT/NW-TT port is not aware of the gPTP port state.

-	Signalling consideration: since all received Announce messages are sent from the ingress port to NW-TT for BMCA evaluation, and NW-TT generates the Announce messages on behalf of the egress ports, the solution leads to more amount of user plane traffic between ingress and egress ports, even after the BMCA procedure has stabilized but not control plane signalling.

Method 2:

-	The same as Method 4, except that the Master port generates the Announce messages based on the Announce information received from the NW-TT. The Announce information is sent only if there is a change compared to the previous Announce information.

-	The port must be aware whether it is Master or non-Master port. The port learns this via UP from the NW-TT.

-	Signalling consideration: compared to Method 4, since the egress port generates the Announce messages based on the Announce information from NW-TT, and the Announce information is sent only if there is a change compared to the previous Announce information, this Method can reduce the user plane traffic related to the Announce messages. However, since the Announce messages form only ca. 6% of all gPTP traffic that needs to be sent via 5GS user plane (with the default Annouce and Sync interval values in IEEE 802.1AS), the reduction in the overall gPTP traffic is negligible. The trade-off is that the Master port must be aware of its port state in order to generate Announce messages.

Method 3:

-	The same as Method 4, except that the ingress port sends the received Announce message to BMCA function in NW-TT if the received Announce is equal or better than the current Announce known to the port or if the ingress port expired to receive announce message from the NW-TT.

-	The port stores the state of the most recent Announce it has sent for the comparison with the received Announce.

-	Signalling consideration: compared to Method 4, since the ingress port sends the received Announce message to BMCA function in NW-TT if the received Announce is equal or better than the previous received Announce or if the ingress port expired to receive announce message from the NW-TT, the Method is not able to reduce any signalling after the BMCA procedure has stabilized. On the other hand complexity is increased in the ingress ports to pre-screen the received Announce messages.

To apply the Methods 2-4 with Solution #9 (i.e. GM resides in DS-TTs), the NEF (or TSN AF) needs to subscribe for the BMCA result reports from the NW-TT. If due to the BMCA result the GM needs to be activated or deactivated in the DS-TTs, the NW-TT reports the BMCA result to NEF(or TSN AF) so that NEF(or TSN AF) can activate or deactivate the GM in DS-TT.

*** End Changes ***