SA WG2 Meeting #S2-142e	S2-2008429r01

16 – 20 November, 2020, Electronic

Source:	Ericsson, Nokia, Nokia Shanghai Bell ? , NTT Docomo?, Qualcomm?

Title:	KI #1: Conclusion update

Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: This contribution clarifies KI#1’s conclusion.

# Discussion

The objective of this Key Issue is to introduce support for Time Synchronization with TSN GM in the TSN network attached to the device. Therefore, TSN time synchronization with gPTP according to IEEE 802.1 AS is within the scope.

Observation 1: The BMCA procedure to which 8.1 conclusion of KI#1 referred to, is unclear, as 5GS internal time synchronization process may also involve BMCA procedure. KI#1 should only address TSN BMCA procedure.

KI#1’s scope is uplink synchronization, the TSN BMCA procedure involves three cases:

Figure 1. TSN BMCA procedure (uplink case 1): TSN GM is at device side and TSN node(s) with gPTP clock(s) is at network (N6) side

In uplink case 1, as shown in Figure 1, the DS-TT port (port 1 in figure) receives announce messages from TSN GM, the DS-TT forwards announce messages to NW-TT for BMCA processing. The NW-TT port (port 2 in figure) then generates announce messages towards TSN nodes connected at N6 side.

Figure 2. TSN BMCA procedure (uplink case 2): TSN GM is at device side and TSN node(s) with gPTP clock(s) is at another device side

In uplink case 2, as shown in Figure 2, the DS-TT port (port 1 in figure) receives announce messages from TSN GM, the DS-TT forwards announce messages to NW-TT for BMCA processing. The NW-TT determines the port states of another DS-TT port (port 3 in figure).

If the state of the DS-TT port 3 is already “Master”, DS-TT generates announce messages towards TSN nodes connected to DS-TT port 3.

If the state of the DS-TT port 3 is other than “Master”, NW-TT changes the state of DS-TT port 3 to “Master”. Then DS-TT can generate announce messages towards TSN nodes, or NW-TT can generate announce messages on behalf of DS-TT port 3.

Figure 3. TSN BMCA procedure (uplink case 3): when 5GS clock is the best, 5GS acts as TSN GM

In uplink case 3, We do not cover the case where TSN GM is attached to the network side, but we cover the case that 5GS can become the GM for TSN (e.g., after a failure of the original TSN GM deployed in the device side), as shown in Figure 3. The DS-TT port (port 1 in figure) receives announce messages from TSN GM, the DS-TT forwards announce messages to NW-TT for BMCA processing. In the case that 5GS clock is the best (as a result of BMCA procedure), the 5GS acts as the newly elected TSN GM.

The NW-TT changes the port state in DS-TT port (port 1 in figure) from “Slave” to “Master”, and then DS-TT port 1 generates announce messages towards TSN GM.

NW-TT port (port 2 in figure 3) is in “Master” state and generates announce message for the TSN nodes connected at N6 side.

Observation 2: The NW-TT port at N6 side is always set in “Master” state when it is involved in KI#1 uplink synchronization cases.

Observation 3: Neither DS-TT nor NW-TT can act as grandmaster, in the case 3 it is the 5GS as a whole (time-aware system) that acts as a GM for TSN. DS-TT port and NW-TT port are gPTP and implement the gPTP ports of the newly elected grandmaster. DS-TT and NW-TT ports are set to “Master” state. Instead of NW-TT acting as grandmaster or DS-TT acting as grandmaster, it would be more correct to say that NW-TT or DS-TT implement the grandmaster fundamental functions, e.g., generate the Sync and Announce messages.

TR 23.700-20 Clause 8.1 states:

“The TSN AF configures the grandmaster in DS-TT port (if supported) via PMIC; otherwise TSN AF configures the grandmaster in NW-TT via BMIC for PDU sessions where the DS-TT does not support acting as a gPTP GM.”

Observation 4: It is not clear what the grandmaster is in DS-TT and NW-TT.

Observation 5: The role of TSN AF and BMCA procedure in NW-TT is vague, which entity can configure port states is unclear.

Merging with S2-2009072 and S2-2008379

# Proposal

Based on the discussion above, it is proposed to update 23.700-20 as follows.

* * * * Start change * * * *

## 8.1	Key Issue #1: Uplink Time Synchronization

To enable support for Uplink Time Synchronization, following principles are recommended for the basis of normative work:

-	The following principles are applied for transmission of the gPTP event messages (carried within Sync message for one-step operation or Follow_up message for two-step operation):

-	DS-TT which is attached by one or more TSN GMs will perform exactly the same operations for UL gPTP messages as what the NW-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	The PDU session of DS-TT port receiving the source GM's gPTP messages is used to forward the gPTP messages to the UPF/NW-TT:

-	the UPF/NW-TT forwards the gPTP messages to the NW-TT and DS-TT ports in Master state.

-	If the egress port is in NW-TT, the NW-TT port performs exactly the same operations for UL gPTP messages as what the DS-TT performs for the DL gPTP messages as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	If the egress port is in other DS-TT, the other DS-TT(s) perform the operation as specified in clause 5.27.1.2.2 of TS 23.501 [2].

-	As specified in TS 23.501 [2] already for Rel-16 Time Synchronization, 5GS is required to guarantee delivery of gPTP message in less than 10ms as recommended by IEEE 802.1AS[6] clause B2.2, if the operator decides to follow such recommendation.

-	The following principles are applied for TSN BMCA procedure:

-	The NW-TT maintains the PTP port state for each DS-TT port and NW-TT port. The PTP port states may be either locally configured in NW-TT or evaluated by BMCA procedure.

-	When the 5GS Bridge PTP port states are locally configured in NW-TT, the following applies for uplink synchronization:

-	When the TSN GM is external to the 5GS, for one of the NW-TT or DS-TT ports (per each gPTP domain) that PTP port state is “Slave” and for all other NW-TT and DS-TT ports of the same gPTP domain the PTP port state is either “Passive” or “Master” (depending on implementation).

-	When the NW-TT implements the grandmaster function for a gPTP domain for the connected networks, all NW-TT and DS-TT ports are in “Master” state for that gPTP domain.

-	When the DS-TT implements the grandmaster function for a gPTP domain for the connected networks, the PTP port state for this DS-TT port is “Master” and for the other DS-TT ports and NW-TT ports of the same gPTP domain the PTP port state is “Master”.

-	When the 5GS Bridge PTP port states are evaluated by BMCA procedure, the following applies for uplink synchronization:

-	NW-TT needs to process the received Announce messages (over user plane) for BMCA procedure, determine port states within the 5GS, maintain Master-Slave-Passive hierarchy.

-	When the NW-TT implements the grandmaster function or when the TSN GM is external to the 5GS, the NW-TT ports at N6 side are always in “Master” state and the NW-TT generates the Announce messages for the Master ports on the NW-TT.

-	When the DS-TT implements the grandmaster function, the DS-TT port(s) is in “Master” state and the DS-TT generates the Announce messages for the Master port(s) in this DS-TT. The PTP port state for the NW-TT ports is “Disabled” or “Passive” (depending on implementation), i.e. the NW-TT ports do not send Announce messages.

-	Irrespective of whether the DS-TT or NW-TT implement the grandmaster function, all the egress ports acting as Master ports are capable of generating Announce messages. This is to avoid the issue of discarding the Announce messages at the DS-TTs because the clockIdentity is obsolete (e.g. it has been updated before by the NW-TT).

-	If the TSN AF has subscribed for the BMCA result reports, the NW-TT reports BMCA result to the TSN AF via BMIC.

-	DS-TT forwards the received Announce messages to NW-TT over User plane.

-	TSN AF can configure the state of NW-TT and of the DS-TT ports either directly using BMIC and PMIC respectively (with external port configuration as per IEEE 802.1AS 10.3.1.1 option b), or by updating the BMCA setting at NW-TT.

* * * * End change * * * *