SA WG2 Meeting #S2-142e	S2-2008431r02

16 – 20 November, 2020, Electronic

Source:	Ericsson

Title:	KI #2, sol 20: updates to CNC controlled VLAN configuration paper

Document for:	Agreement

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

Abstract of the contribution: The paper resolves outstanding questions on the CNC controlled VLAN configuration solution.

Introduction

The paper resolves outstanding questions on the CNC controlled VLAN configuration solution. The following questions have been raised.

Should the VLAN configuration be sent in the BMIC or in the PMIC? In the IEEE specifications, the VLAN configuration is part of the Bridge VLAN Configuration managed object according to IEEE specifications; there is a single such managed object per Bridge. We propose to keep this principle and send the VLAN configuration inside the BMIC to the UPF/NW-TT. Note that some of the VLAN configuration information is port specific. Whenever possible, VLAN functionality at DS-TT should re-use existing PDR/FAR for the PDU session. If a port specific VLAN functionality is not supported by PDR/FAR at UPF it needs to be checked whether the VLAN configuration information may be provided via PMIC for each port separately.

Should the DS-TT port VLAN processing be performed in the UPF or in the device? Preferred is DS-TT port VLAN processing using PDR/FARs at UPF.

What is the relationship of VLAN configuration with KI#2? By partitioning the network into multiple VLANs, it is possible to control and separate network resources when UEs communicate with each other.. The VLAN support is important to establish multiple network domains (e.g. TSN domains), since otherwise according to Ethernet principles, all end stations in the Ethernet network can communicate with each other. In this way, the VLAN configuration provides a mechanism to structure communication needs of UE-UE communication, which is part of KI#2.

The solution is described in a generic way, i.e., the same solution applies to traffic in all directions, and hence the solution also allows to control communication between UEs and end stations that are reachable on N6.

CNC provided VLAN configuration is necessary to allow the CNC to use the same configuration method for all bridges, including both fixed and 5GS bridges. Without this, separate configuration methods would be needed for communication over fixed bridges and UE to UE communication over 5GS, which would be a serious operational burden and a disadvantage for the 3GPP TSN integration.

Can CNC provided VLAN configuration co-exist with SMF provided VLAN processing rules? Yes, VLAN processing using N4 rules may co-exist with CNC provided VLAN configuration and when configured, both are executed. It is up the operator to determine which set of VLAN configuration is provided by the SMF and which set of configuration is provided by the CNC; this depends on the actual deployment considerations.

Proposal

It is proposed to update the solution in 23.700-20 as follows, which also includes some additional clarifying sentences.

* * * * Start change * * * *

6.20	Solution #20: CNC controlled VLAN configuration

6.20.1	Introduction

The solution adds general support for CNC controlled VLAN configuration. This can be also used to address UE to UE communication, Key Issue #2, in practical deployments, e.g., for logical separation of one or multiple UE to UE TSN streams, for easier management of flows or for easier setup for forwarding paths. By partitioning the network into multiple VLANs, it is possible to control and separate network resources when UEs communicate with each other. The VLAN support is important to establish multiple network domains (e.g. TSN domains), since otherwise according to Ethernet principles, all end stations in the Ethernet network can communicate with each other. In this way, the VLAN configuration provides a mechanism to structure communication needs of UE-UE communication.

Note that the VLAN configuration support proposed here is general for all communications, including TSN and non-TSN flows that are UE to UE as well as other flows that are between UE and network, or between multiple network ports.

It is proposed that the specification supports CNC controlled VLAN configuration only by the existing interface via TSN AF already defined for the fully centralized configuration model specified by IEEE Std 802.1Qcc  [7]; therefore, MVRP is not supported or needed at DS-TT and NW-TT.

The existing release 16 specification supports Ethernet Packet Filters set by the SMF in the PDRs. However, the release 16 specification is limited to SMF controlled cases which assume that the SMF has full knowledge and control over the Ethernet network, which is not the case in many deployments that serves TSN and non TSN traffic in parallel. As another limitation, the release 16 SMF provided rules apply to the N4 sessions corresponding to the PDU Sessions and the associated DS-TT ports, while VLAN handling rules for NW-TT ports apply according to static pre-configuration at NW-TT.

The need to manage and configure the 5GS in the same way as other bridges in the network is essential in practical industrial deployments. This solution allows the CNC to control the VLAN configuration of IEEE 802.1Q bridges in a unified way, and thereby allows the CNC to handle the VLAN configuration for 5GS bridges in the same way as fixed bridges.

NOTE:	CNC provided VLAN configuration co-exists with SMF provided PDR/FAR rules that also act on the VLAN IDs in the Ethernet headers.

The VLAN configuration affects the configuration of bridge forwarding: the forwarding rules may also include the VLAN IDs, and the VLAN configuration may refer to a specific forwarding table. In common deployments, the use and the implementation of the VLAN configuration and bridge forwarding is closely coupled.

The VLAN configuration provided by the CNC is set on a per port granularity. VLAN configuration may be set at any port, including DS-TT as well as NW-TT ports.

VLAN configuration implies a set of functionality listed below based on IEEE 802.1Q.

Table 6.20.1-1

| Configuration | Reference in IEEE 802.1Q clause | Description |
| --- | --- | --- |
| Per Bridge VLAN Configuration |  |  |
| Read Bridge VLAN Configuration | 12.10.1.1 | To obtain general VLAN information from a Bridge. |
| PVID and VID Set values | 12.10.1.2 | Set the VID value to be added to received untagged frames at a given port. |
| Acceptable Frame Types parameters | 12.10.1.3 | Used to configure whether to admit only VLAN-tagged frames, only untagged/priority tagged frames, or all frames. |
| Enable Ingress Filtering parameters | 12.10.1.4 | Whether to enable or disable ingress filtering. With ingress filtering, only ports in the member set for the given VID can admit traffic. |
| Reset Bridge | 12.10.1.5 | To reset all statically configured VLAN-related information in the Bridge to its default state. |
| VID to FID allocation | 12.10.3 | Filtering identifier for a given VID, which determines whether different VLANs use individual or shared forwarding tables. |
| FID to MSTID allocation | 12.12.2 | The FID to MSTID allocation is required to complete the VID to FID to MSTID allocation. For TSN Streams, this is used to allocate their VID to the TE-MSTID for central CNC controlled operation. (See clause 12.32.3.1 in IEEE Std 802.1Qcc [7] for further information). |
| Per VLAN Configuration |  |  |
| The VLAN Configuration | 12.10.2 | For setting the VLAN name and reading VLAN specific configuration. |
| Filtering Database Configuration |  |  |
| Static Filtering Entries | 8.8.1 | Defines forwarding rules based on the combination of destination MAC address and VID. Note that the use of static filtering entries is introduced and elaborated in other solutions as well, which can be used in combination with this solution. |
| Static VLAN Registration Entries | 8.8.2 | Determines whether frames are to be VLAN-tagged or untagged when transmitted; also defines member set for a VID that is used for ingress and egress filtering. |
| Dynamic Filtering Entries | 8.8.3 | Maintains the dynamically learnt addresses respecting the VID to FID allocation as configured above, i.e., whether shared or independent learning is applied among certain VLANs. |
| Default Group filtering behaviour | 8.8.6 | Specifies defaults for group-addressed frames for each VID and outbound Port regarding forwarding/filtering. |

The solution reuses the mechanisms defined in Rel-16 for the transfer of bridge and port configuration between the UPF/NW-TT, DS-TT and the TSN AF using the already specified Management Information Container. Note that VLAN processing functionality may be set up and used even before TSN streams are established or for non-TSN traffic.

VLAN processing is a set of IEEE 802.1Q defined functionalities for Ethernet specific processing may also be realized using N4 rules. The detailed implementation of VLAN processing is not specified; the implementation is not restricted as long as the externally observable behaviour is according to the configuration.

VLAN configuration and processing is a general Ethernet functionality that is not TSN specific; however, TSN deployments often make use of this functionality. Even if there is a dependency between both, the CNC performs the VLAN configuration independent of the TSN stream configuration. For the VLAN configuration that is needed by the TSN streams, the CNC can provide the VLAN configuration before the provision of TSN stream related configuration. VLAN processing using N4 rules before TSN streams are established can be provided based on SMF configuration. VLAN processing using N4 rules may co-exist with CNC provided VLAN configuration and when configured, both are executed.

The solution could be combined with other solutions.

Editor’s note: If VLAN configuration is provided by the third party function CNC, it is FFS whether and how the 5GC SMF/PCF synchronizes the VLAN configuration provided at UPF via BMIC. It is also FFS whether and how the 5GC SMF generates N4 rules (PDR/FAR) before TSN streams are established over the 5GS user plane.

6.20.2	Functional Description

The configuration for VLAN is exchanged between the CNC and the TSN AF as part of the 5GS bridge configuration. The VLAN configuration is part of the Bridge VLAN Configuration managed object according to IEEE specifications; there is a single such managed object per Bridge, however the configuration is flexible to support different VLANs for the different ports. The TSN AF provides the information to the UPF inside the Bridge Management Information Container. Configuration can also be read from the UPF to the TSN AF and CNC in the reverse direction using the existing signalling path.

6.20.3	Procedures

-	CNC provides VLAN configuration to the TSN AF.

-	TSN AF provides the VLAN configuration inside the BMIC/PMIC using the existing signalling path to the UPF/NW-TT and the DS-TT concerned.

-	Based on the configuration, the UPF sets up the VLAN processing. The implementation of VLAN processing is not specified.

-	The VLAN configuration can also be read from the UPF/NW-TT and provided to the TSN AF and CNC.

6.20.4	Impacts on existing services and interfaces

TSN AF:

-	Forward VLAN configuration to/from the UPF NW-TT inside BMIC

UPF/NW-TT:

-		Support for VLAN processing for both network side based on the CNC provided VLAN configuration.

DS-TT:

Support for VLAN processing for the device side ports based on the VLAN configuration.

* * * * End change * * * *