# 3GPP TSG|WG-SA WG2 Meeting #142E	S2-2008472

# Elbonia, 16 – 20 November 2020	(Revision of S2-20xxxxx)

# Source:	Ericsson, AT&T, CableLabs, Charter Communications, China Unicom, Convida Wireless, Intel, Lenovo, Motorola Mobility, OPPO, Sony, Huawei, HiSilicon, ETRI, Samsung, China Telecom, KDDI, Futurewei, DOCOMO, Spreadtrum Communications, China Mobile, LG Electronics, ZTE, Nokia, Nokia Shanghai Bell, MediaTek Inc

# Title:	New WID on enhanced support of Non-Public Networks

# Document for:	Approval

Agenda Item:	9.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Work Item on enhanced support of Non-Public Networks

## Acronym: eNPN

## Unique identifier: 	{A number to be provided by MCC at the plenary}

Potential target Release: Rel-17

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | x | x | x |  |
| No |  |  |  |  |  |
| Don't know | x |  |  |  | x |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

| x | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| AVPROD | SA1 | 840045 | WID on Audio-Visual Service Production |
| IESNPN | SA1 | 860008 | WID on IMS emergency support for SNPN |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 820017 | 5GS Enhanced support of Vertical and LAN Services | Antecedent Stage 2 work item |
| 840024 | Study on enhanced support of Non-Public Networks | Antecedent Stage 2 study item |

## 3	Justification

This work item aims at specifying system enhancements required for 5G system to support service requirements for Audio-Visual Service Production, and service requirements for IMS voice and emergency support for SNPN, as concluded by the Study on enhanced support of Non-Public Networks.

The service requirements are documented in TS 22.261, 22.263, 22.101 and 22.228.

## 4	Objective

The objective is to specify enhancements to 5GS as per conclusions reached within TR 23.700-07 for the following aspects:

-	Enhancements to Support SNPN along with credentials owned by an entity separate from the SNPN (KI#1)

-	To enable scenarios when UE is using a SNPN subscription

-	To enable scenarios when UE is using a PLMN subscription

-	To enable the scenario when separate entity is offering a AAA server as per conclusions in clause 8.1.1

-	To enable the scenario when separate entity is offering a UDM and AUSF

-	To enable mobility between networks as per conclusions in clause 8.1.2

-	To enable update of prioritized list of preferred SNPNs to the UE

-	NPN support for Video, Imaging and Audio for Professional Applications (VIAPA) (KI#2)

-	To enable support for VIAPA as per conclusions in clause 8.2.

-	Informative guideline for how to use existing Rel-16 mechanisms and information to support VIAPA services

-	Informative guideline for mapping between standardized 5QI/ARP and DSCP

-	Support of IMS voice and emergency services for SNPN (KI#3)

-	To enable support for IMS services as per conclusions in clause 8.3.

-	Use of IMC when USIM or ISIM is not available in UEs accessing IMS via an SNPN

-	Reuse of USIM credentials for IMS AKA shall be possible when USIM is available in UEs accessing IMS via an SNPN

-	Functionality for support of voice and emergency services with SNPN, excluding EPS fallback and T-ADS.

-	UE Onboarding and remote provisioning (KI#4)

-	To enable UE onboarding and provisioning for SNPN as per conclusions in clause 8.4.1.

-	UE Onboarding i.e. enabling 3GPP connectivity for UE to realize remote provisioning

-	Enabling restricted PDU Session for UP provisioning

-	To enable UE onboarding and provisioning for PNI-NPN as per conclusions in clause 8.4.2.

NOTE:	Initial access for UE onboarding is provided usng PLMN subscription.

-	Enabling restricted PDU Session for UP provisioning

Coordination with RAN WGs and SA3 is expected, where necessary, in meeting the objectives of this WID.

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Architectural and Conceptual enhancements to support the stated objectives | SA#92 (June 2021) |  |
| 23.502 | Procedural enhancements to support the stated objectives | SA#92 (June 2021) |  |
| 23.503 | Policy and Charging Control Framework for the 5G System | SA#92 (June 2021) |  |
| 23.228 | IMS enhancements to support the stated objectives | SA#92 (June 2021) |  |
| 23.167 | Enhancements to Domain Selection Rules for emergency session | SA#92 (June 2021) |  |

## 6	Work item Rapporteur(s)

Hedman, Peter, Ericsson, peter dot hedman at ericsson dot com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

Security aspects will be covered by SA3.

Management aspects will be covered by SA5.

## 9	Supporting Individual Members

Supporting IM name

AT&T

CableLabs

CATT?

Charter Communications

China Mobile

China Telecom

China Unicom

Convida Wireless

Deutsche Telekom?

DOCOMO

Ericsson

ETRI

Futurewei

HiSilicon

Huawei

Intel

Interdigital?

KDDI

KT?

Lenovo

LG Electronics

MediaTek Inc

Motorola Mobility

NICT?

Nokia

Nokia Shanghai Bell

OPPO

Qualcomm Incorporated?

Rogers Communications?

Samsung

Sandvine?

Siemens?

Sony

Spreadtrum Communications

Telecom Italia?

Verizon UK Ltd?

Volkswagen AG?

ZTE