SA TSG-SA2 Meeting #142e	S2-2008487r01

2020-11-16 – 2020-11-20, electronic meeting

Source:	Ericsson

Title:	KI#2, sol#24 update

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC/Rel-17

Abstract of the contribution: This contribution updates solution #24

1	Introduction

Further clarification and improvements of solution 24

3	Proposal

*******************  Start change  ********************

6.24	Solution #24: Support of edge relocation, triggering of new DNS query by the UE

6.24.1	Solution description

The solutions address Key Issue #2: Edge relocation.

The solution in this clause is a complement to application controlled server relocation, where the application layer triggers the UE to move from one server to another.

The assumption for these solutions is that the UE is using DNS to find an edge application server. As the UE moves, the PSA may change. At this point the UE may need to do a new discovery of the application server.

The three connectivity models, distributed anchor, multiple session and session breakout (using UL-CL or BP). From an edge relocation point of view multiple sessions and distributed anchor are similar.

Distributed anchor and multiple sessions, regardless if SSC mode 2 or 3 is used (clause 4.3.5.1-3 of TS 23.502 [3]):

-	When the UE moves, the network will trigger the UE to establish a new PDU session. This can prevent the edge applications in the UE to use the old EAS information but can not guarantee to issue a new discovery according to solution to KI#1, i.e. the UE to do DNS query to find a suitable edge application server in the new PSA.

NOTE 1:	How the edge application in the UE gets aware that a new discovery is needed and that possible cached DNS results in the UE cannot be used is a UE internal matter.

Session breakout there are 2 cases, IPv6 multi homing with a BP or UL-CL (clause 4.3.5.7 of TS 23.502 [3]):

-	For BP, as the UE moves, the network sets up a new local PSA. The UE gets a new IPv6 prefix (and routing rules) and removal of old prefix. This triggers the UE to remove the old EAS information bound to the old prefix. .

NOTE 2:	How the edge application in the UE gets aware that a new discovery is needed.

-	For UL-CL, the UE is not aware that local breakout is used for a specific application traffic flow. Hence, when the network moves the local PSA, the UE is not aware that change has been made. There are several ways to indicate to the UE that there has been a PSA change and the edge applications need to do a new discovery by means of solution to KI#1

1)	Edge application servers subscribes to PSA change in 5GC, and by application specific means the edge application server indicates to the edge application that a new discovery is needed.

2)	The SMF indicates according to solution #32 and #34 that a new discovery is needed via NAS signalling

3)	The SMF blocks traffic by filtering in UPF to old server(s). Application/transport timers will trigger error that indirectlythis can trigger the edge application to do a new discovery when the application issues another DNS query for EAS.

4)	The SMF blocks traffic by filtering in UPF and has a rule to send a ICMP unreachable code 7 response. This can trigger error to disconnect the socket connection bound to the old EAS.

NOTE 3:	How the edge application in the UE gets aware that a new discovery is needed and that possible cached DNS results in the UE cannot be used is a UE internal matter.

e.

NOTE 4:	SA WG3 needs to evaluate any potential threats by using ICMP and whether and how to enforce the recommendation regarding security aspect of ICMP as described in TS 33.501 [38].

6.24.2	Procedures

6.24.2.1	Distributed anchor and multiple sessions, SSC mode 3

Figure 6.24.2.1-1

0.	UE has a PDU session supporting SCC mode 3. UE has discovered and selected a AS.

1.	Application traffic ongoing with the discovered O-AS.

2.	5GC triggers move of PSA using SSC mode 3, e.g. due to UE mobility. UE sets up a new PDU session for this and gets a DNS server address from 5GC.

3.	When the UE’s application queries DNS request based on the application logic, UE can issue a discovery of a new AS due to SSC mode 2/3 procedure..

4.	Once new AS address is received, the UE can start using the new AS based on UE's application implementation.

NOTE:	Any AS state synchronization is outside the scope of this solution.

6.24.2.2	IPv6 multi homing with a BP

Figure 6.24.2.2-1

0.	UE has a PDU session supporting IPv6 multi homing. The 5GC has installed a IPv6 BP and UE has a prefix for this. UE has discovered and selected an edge AS for this prefix (this using any method as per KI#1 solutions using IPv6 BP and DNS).

1.	Application traffic ongoing with the discovered O-AS.

2.	5GC triggers move of PSA using IPv6 BP, e.g. due to UE mobility. This by sending a new prefix and removing old prefix to UE, as part of this the UE gets a DNS server address from 5GC for the new prefix. UE can drop traffic to O-AS that is bound to the old prefix.

3.	The application of UE can trigger a discovery of a new AS due to new IPv6 prefix and removal of old prefix based on its implementation, and since new DNS configuration is received as part of the IPv6 prefix provisioning, the UE should not use the DNS cache so that the UE can issue a new DNS request to discover the AS to be used on the new PDU session when the UE's application queries the new DNS.

4.	Once new AS address is received, the UE can start using the new AS.

NOTE:	Any AS state synchronization is outside the scope of this solution.

6.24.2.3	Session breakout using UL-CL, UE is not aware of change of PSA for a flow

Figure 6.24.2.3-1: ICMP triggered discovery

The pre-requisite for the flow is that UE has a PDU session, it has discovered an edge application server (AS) and the network has inserted an UL-CL with a local PSA that will break out the application traffic to the AS.

1.	UE has ongoing application traffic with Old AS via Old PSA and UL-CL.

2.	SMF insert a new UL-CL and a new local PSA. It also prepares for generating an ICMP message (happening in step 4) by e.g. one of the following mechanisms:

-	SMF installs a forwarding rule in UPF to forward any IP packet destine to Old AS to be forwarded to SMF, and SMF generates the ICMP packet to be sent to the UE via UPF.

-	N4 is updated by e.g. SMF installs a PDR to match any IP packet destined to Old AS. This is linked to a FAR with an Application Action that indicates that ICMP shall be sent.

3.	UE sends an IP packet, which is caught in UPF and depending on how step 4 is done the packet may be forwarded to SMF.

4.	SMF or UPF generates an ICMP Destination unreachable message with code 3 (Port Unreachable) or 7 (destination host unknown).

5.	When UE has received the ICMP Destination unreachable message, it is expected that the UE follows TTL and follows RFC 1122 [15] for hard errors.

6.	UE uses the new AS for the application traffic.

NOTE:	SA WG3 needs to evaluate any potential threats by using ICMP and whether and how to enforce the recommendation regarding security aspect of ICMP as described in TS 33.501 [38].

6.24.3	Impacts on services, entities and interfaces

UE’s application need make sure that a new discovery is done for edge application servers based on different triggers and not use any cached result

Depending on solution for UL-CL, SMF to generate ICMP, or UPF to generate ICMP based on new N4 functionality.

********************  End  change  ********************