3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008511

Elbonia, November 16 – 20, 2020, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#2 : Session continuity PLMN/SNPN

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN/ [Rel-17]

Abstract: This contribution clarifies the difference between service continuity and session continuity issue and propose to add that in the conclusion. Meanwhile, it proposes some description for SR UE case to bind with an Editor’s note.

# 1. Introduction

There were extensive discussion on service continuity and session continuity in the SA2#141E meeting. This contribution discuss the relationship between service continuity and session continuity and propose to clarify that in the conclusion of KI#2 regarding service continuity support for VIAPA services.

Service continuity between the two networks can be supported with two options:

- Using session continuity mechanism in 3GPP network

- Using the mobility procedure between the two networks in 3GPP with some other mechanism (e.g., MPTCP, multi-homing support in the application layer).

Session continuity in option 1 works only in case of a common anchor.. Option 2 works also in the case of no common anchor. Therefore, this contribution proposed further consideration and to include clarifications in the conclusion of service continuity support for VIAPA.

Meanwhile, this contribution propose to clarify the issue for SR UE regarding service continuity support and potential enhancement. This is bind to the “Editor's note:	Whether the network trigger the UE register to the target network via N3IWF before it lose the radio coverage is FFS.” in the conclusion.

In last meeting, we agreed that for very low latency VIAPA when the UE is connected to the PLMN the VIAPA services can be obtained via PLMN directly.

Let consider the figure 1, where the UE is PLMN and it is accessing VIAPA via PLMN and there is no roaming interfaces between the PLMN and SNPN, in particular:

- The UE is in PLMN (position A) with the PDU session for PLMN services (PDU session #1, blue line), and VIAPA services via UPF in PLMN (PDU session #2, red line)

- UE moves into SNPN position B

- Per current interim agreement the UE can access PLMN’s services from SNPN via N3IWF==> the PDU session #1 (Blue line) can be HO via N3IWF from SNPN to PLMNWhen the UE moves to position B the following considerations applyNeither Session continuity nor PDU session continuity can be supported due to lack of roaming interfaces between PLMN and SNPN, since PDU session #2 (red line) can not be moved, it is lost and it shall be established as a new one (also IP address is changed)

- If the VIAPA services are provided to UE via PDU session #1 when in PLMN, by definition or they stay anchored in PLMN via N3IWF or they needs to be moved in to a new PDU session established when the UE moved to SNPN.

- The same scenario occurs when UE moves from SNPN to P:MN with VIAPA services active.

Figure 1: UE moves from PLMN to SNPN with VIAPA services

Figure 2: UE moves from SNPN to PLMN with VIAPA services

Conclusion:

- When the UE is in PLMN if the services are provided via PLMN, then session continuity is not supported

- When the UE is in PLMN if the services are provided via SNPN’s N3WIF, then  session continuity is supported

## 1.2	evaluations updates

Evaluations updates taking into account the new solution in S2-2007948 approved last meeting and impacts of solution #50.

# 2. Text Proposal

It is proposed to capture the following changes in the TR.

1) revisions of evaluations clause

2) address the Service Continuity and Session continuity

# * * * * First change * * * *

## 7.2	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

The evaluation is based on major principles from the KI#2 solutions. The evaluation separates to different areas for each principle as below table:

Table 7.2-1: Evaluation of KI#2 related principles

| Principle | What it solves | Benefit, drawback | System impact | Open issues | Reference |
| --- | --- | --- | --- | --- | --- |
| N3IWF architecture | Service continuity aspect. Simultaneous data service aspect. | Benefit: Drawback: | Based on the existing release 16 architecture, No new architecture impact |  | Solution #13, #14, #15, #17, #18, #46, #47, #48, #49, #50, #51, #52 |
| N3IWF architecture with dual radio capability (2RX/2TX) UE enhancement. | Same as N3IWF architecture. | Benefit: Drawback: |  |  | Solution #13, #15, #48, #50 |
| N3IWF architecture with application layer service continuity support | Same as N3IWF architecture | Benefit: Drawback: | Solution #49: The handover or the deregistration procedure is triggered by the AF; |  | Solution #48, #49 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in overlay network. | Paging aspect. | Benefit: Drawback: | UE logic to keep one PDU session in underlay network. Existing liveness check tool to be used over NWu interface to keep the NWu connection open. Apply to single radio UE |  | Solution #13, #14, #50 |
| N3IWF architecture with enhancement to keep UE in CM-Connected state in both overlay network and underlay network. | Paging aspect. | Benefit: Drawback: | RAN and CN impact to keep UE always in RRC-Inactive in the underlay network. PDU session in underlay network remains active. Support single radio UE | Whether it is necessary to keep UE in CM-Connected in underlay network is FFS. | Solution #14 |
| N3IWF architecture with enhancement to introduce "inactive" mode on NWu interface. | Paging aspect. | Benefit: Drawback: |  | IKEv2 impact is FFS. How does the DL IPSec data reach UE when NAT timeout is FFS. | Solution #17 |
| N3IWF architecture with enhancement to split DL traffic to Uu interface and UL traffic to NWu interface for 2RX/1TX UE. | Simultaneous data service aspect. | Benefit: Drawback: | RAN impact for uplink control communication with single Tx UE/AMF impact to support split indication. UE/SMF/UPF impact to support user data plane traffic path steering. | It will be up to RAN WG2 and RAN WG3 to evaluate and decide what the RAN impact will be or if reusing the existing RAN mechanism is feasible | Solution #18 |
| N3IWF architecture with enhancement to assist UE to choose either SNPN or PLMN to use Uu interface for better QoS. | Simultaneous data service aspect. | Benefit: Drawback: | Solution #46: The USRP rule is extended or a new policy is used for the UE to select the subscription. Solution #47: The solution provides supported PLMN list to the UE to decide whether to access SNPN service via SNPN N3IWF using PLMN PDU Session |  | Solution #46, #47, #50 |
| Inter-PLMN like architecture between SNPN and PLMN. | Service continuity aspect. | Benefit: Drawback: | To be concluded from KI#1 evaluation. |  | Solution #1, #2, #50. |
| Inter-PLMN like architecture between SNPN and PLMN with enhancements to enable SNPN operator to control access to PLMN services | Simultaneous data service aspect | Benefit: enable the SNPN control the access to PLMN services via SNPN Drawback: | Network impacts for control the access to PLMN services using a HR PDU session or a secondary registration at PLMN. For secondary registration at PLMN, the 5GC can indicate to UE whether the secondary registration at PLMN is allowed and further the PLMN’s UDM will determine whether to accept UE's secondary registration. | How to achieve the functionality of secondary registration on the same access type in 5GS is FFS. | Solution #50 |
| N3IWF architecture with MUSIM like implementation | Concurrent paging and data service for single radio UE. | Benefit: Drawback: |  | The principle is not clearly stated in the solution. | Solution #15, #50, #52. |
| N3IWF architecture with QoS degradation subscription/notification between overlay and underlay network. | Because of the traffic characteristics of VIAPA (e.g. High QoS KPI, potentially bringing heavy load to the networks, etc) the overlay network need to get QoS notification from the underlay network and adjust its QoS accordingly. | Benefit: Drawback: | UE to provide overlay network with information about the underlay network, so the overlay network could find the underlay network to make a QoS degradation subscription request. |  | Solution #51 |
| N3IWF architecture with paging/downlink data event subscription/notification between overlay and underlay network | Paging aspect | Benefit: none Drawback: support N3IWF architecture as well as NEF exposure between SNPN and PLMN | UE, AMF, SMF, NEF impacts to enable UE receive the paging message from another network |  | Sol#55 |

# * * * * End of changes * * * *