SA WG2 Meeting #142E (e-meeting)	S2-2008560r03

November 16 – November 20, 2020, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics, LG Uplus

Title:	KI #2, Conclusion update to resolve EN on Uu subscription

Document for:	Approval

Agenda Item:	8.2 Study on enhanced support of Non-Public Networks

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This paper proposes KI#2 conclusion update to resolve editor's note.

Background

In the last SA2 meeting, SA2 sent LS to SA1 to ask whether there is a requirement that SNPN or PLMN can instruct the UE to use PLMN or SNPN subscription to select a PLMN or SNPN for VIAPA service and following editor's note is added in the conclusion.

Editor's note:	Whether the network can assist UE to select the proper network for Uu is FFS.

This paper assumes that the SA1 response LS will say there is no requirement.

According to Rel-16 N3IWF architecture, when a UE receives SNPN service via PLMN, QoS differentiation can be supported based on proper configuration e.g. DSCP and QoS flow mapping, non-alteration of DSCP marking, etc which may be governed by SLA. So SNPN network may have preferred PLMNs to guarantee QoS differentiation. Providing such information is helpful to the UE so that the UE may decides whether N3IWF architecture can be used to receive VIAPA service. So it is proposed that the SNPN may provide assistance information (e.g. a list of preferred network) to the UE.

Proposal

It is proposed to capture the following proposal to TR 23.700-07.

* * * * Start 1st Change * * * *

8.2	Key Issue #2: NPN support for Video, Imaging and Audio for Professional Applications (VIAPA)

Editor's note:	These are INTERIM conclusions for Key issue #2.

When UE only has single subscription, the data service from both V-SNPN and Home SP (PLMN or SNPN), as well as service continuity is to be evaluated and concluded by KI#1.

When UE have both subscriptions for SNPN and PLMN, following interim agreements are adopted.

For the issue of service continuity for VIAPA,

-	It is concluded that the existing Rel-16 N3IWF-architecture is used as the basis to address data service from both networks and session/service continuity between the two networks.

-	For single radio UE, PDU session continuity can be realized by utilizing the existing handover procedure between non-3GPP access and 3GPP access for single access PDU session, where one network is acting as non-3GPP access of the other network.

Editor's note:	Whether the network trigger the UE register to the target network via N3IWF before it lose the radio coverage is FFS.

-	For dual radio UE the UE can use one radio operating in SNPN access mode and the other operating the normal PLMN selection, in order to avoid SNPN access mode switch. PDU Session continuity and service continuity may e.g. be provided as follows:

-	UE registers to both SNPN and PLMN the procedure described in clause 4.9.2 in TS 23.502 [6] is followed as necessary.

-	Register to the same 5GC via both Uu and NWu interface and possibly establish MA-PDU session. Upon mobility, UE and UPF could switch the user plane resource to the corresponding access type.

Editor's note:	Dual radio may have radio limitation when operated simultaneous with two independent service providers. It is FFS whether is further enhancements is needed.

For the issue of QoS support for VIAPA:

Editor's note:	Whether the network can assist UE to select the proper network for Uu is FFS.

-	After the UE selects the SNPN or PLMN, the UE obtains VIAPA service with or without Rel-16 N3IWF architecture specified in clause D.3 of TS 23.501 [4].

-	It is proposed to add an informative guideline for mapping between standardized 5QI/ARP and DSCP marking value in TS 23.501 [4], Annex D, so that the PLMN and SNPN may use the same mapping values for UL and DL user plane traffic within SNPN and PLMN.

-	According to the existing Rel-16 N3IWF architecture, SLA between the two networks is helpful to ensure the QoS differentiation in the underlay network e.g. by configuring DSCP and QoS flow mapping, non-alteration of DSCP marking, etc. When the SLA exists, the SNPN network may provide assistance information (e.g. a list of preferred PLMN network that can support QoS, and the service(s) for which the UE can get QoS support) to the UE during the Registration in SNPN network. When the SNPN network provides assistance information, the network may considers UE location and time to check SLA with PLMN network.. When a UE wants to receive SNPN service and PLMN service simultaneously, the UE may receive SNPN service via SNPN N3IWF over the PDU Session of the PLMN if the PLMN is included in the preferred PLMN network. Otherwise, the UE may receive PLMN service via PLMN N3IWF over the PDU Session of the SNPN.

To improve the latency to resume a service provided by the overlay network, the following optimization is concluded. The RAN node in the underlay network can receive an indication that the NG-RAN can use as input to decide whether it is preferred to release a UE to RRC-Inactive.

Editor's note:	Further details of the indication and the conditions for the 5GC sending the indication to NG-RAN is FFS, and whether existing QoS flow information can be used to derive whether it is preferred to release a UE to RRC-Inactive is FFS.

To support UE to receive data services from one network (e.g. NPN), and paging as well as data services from another network (e.g. PLMN) simultaneously, the following principles will be progressed in the normative phase:

-	For single radio UE, keep overlay network connection always in CM-CONNECTED by using mechanisms available in Rel-16 .

NOTE:	mechanisms available such as the TIMEOUT_PERIOD_FOR_LIVENESS_CHECK of IKEv2 defined in TS 24.502 [10] allows to keep alive the PDU session over N3IWF avoiding the deregistration from the network where the UE is connected via N3IWF. The TIMEOUT_PERIOD_FOR_LIVENESS_CHECK, CM_IDLE timer and N3GPP UE Deregistration timer need to be configured properly in order to efficiently reach the goal of increasing UE reachability and reduce the lack of paging in PDU session carried over IKEv2.

-	When N3IWF based solution is used, the overlay network and its service AF can use existing NEF notification procedures, such as of subscribing the "QoS monitoring" or "QoS sustainability" via the interface between NEF and AF, to subscribe and receive the notification from underlay network regarding the connectivity QoS status or QoS mapping changes that are associated with the IPsec of the overlay network. With the QoS update information from the underlay network, the overlay network can adjust its connectivity QoS accordingly. The opposite way is also applicable that the underlay network and its service AF can use existing NEF notification procedures, such as of subscribing the "QoS monitoring" or "QoS sustainability" via the interface between NEF and AF, to subscribe and receive the notification from overlay network regarding the connectivity QoS status or QoS mapping changes that are associated with the IPsec of the overlay network. With the QoS update information from the underlay network, the underlay network can adjust its connectivity QoS accordingly.

Editor's note:	It is FFS if any new information is needed or not for the QoS notification between NPN and PLMN

Concurrent access to very low latency VIAPA services and PLMN services can be supported as follows:

-	The single radio UE may register on the SNPN and accesses VIAPA services directly via the SNPN and accesses PLMN services via the SNPN and the PLMN's N3IWF.

-	The single radio UE may register on the PLMN and accesses VIAPA services directly via the PLMN (e.g. based on a local UPF and direct peering between the PLMN and the venue's VIAPA services) and also accesses PLMN services directly via the PLMN.

* * * * End of Change * * * *