SA WG2 Meeting #142E e-meeting	S2-2008605r01

Elbonia, November16 – 20, 2020	(revision of S2-20xxxx)

Source:	China Mobile

Title:	KI#2, Sol #25 Update to enable more use cases for exposing UE mobility analytics

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes to clarify to enable more use cases for Solution #25.

1. Text Proposal

It is proposed to capture the following clarification in TR 23.700-91.

* * * * Start of Changes * * * *

6.25	Solution #25: Exposing UE mobility analytics for multiple NWDAFs case

6.25.1	Introduction

This solution addresses Key Issue #2: Multiple NWDAF instances.

The UE mobility information is regarded as useful information for e.g. some certain commercial usage for deriving commercial footprint between shopping centres.

The awareness of users who even stayed in some areas may become useful e.g.:

Case 1: The AFs inside Source Area of Interest A may need to enquiry the users who once stayed in Target Area of Interest B in target period 1 as depicted in the below figure.

Case 2: The AFs inside Source AOI A may need to enquiry the users who once stayed in Target AOI B in target period 1 and once stayed in Target AOI C in target period 2 as depicted in the below figure.

Figure 6.25.1-1: UE mobility analytics provided to NF/AF

To address the above use case, the solution is to enhance the existing mechanism of data collection and analytics generation for the scenario of multiple NWDAFs.

6.25.2	Input data

Input Data is defined in Table 6.7.2.2-1, Table 6.7.2.2-2, clause 6.7.2.2, TS 23.288[5] respectively.

6.25.3	Output Analytics

Output Data is defined in Table 6.7.2.3-1 and Table 6.7.2.3-2, TS 23.288 clause 6.7.2.3, TS 23.288[5] respectively.

6.25.4	Procedures

Figure 6.25.4-1: Procedure for UE mobility analytics provided to NF/AF

1a.	To obtain the SUPI list and their related mobility analytics, e.g. user location and time slot, who currently stays in Source Area of Interest and once stayed in Target Area of Interest(s) in target period(s) or not stayed in Target Area of Interest(s) in target period(s) , the NF/AF sends:

- For case 1, an Analytics subscription or request (Analytics ID = UE mobility, Analytics Filter information = Source AOI A, Target AOI B, Analytics Reporting information = Analytics target period 1) to the NWDAF1 serving the Source AOI A by Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request.

- For case 2, an Analytics subscription or request (Analytics ID = UE mobility, Analytics Filter information = Source AOI A, Target AOI B, Target AOI C, Analytics Reporting information = Analytics target period 1, Analytics target period 2) to the NWDAF1 by Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request.

For these untrusted NFs, the NF sends the subscription or request via a NEF, where the NF invokes NEF service Nnef_AnalyticsExposure_Subscribe.

1b-1c.	Authorization for the AF is performed in the NEF in case the NEF is used. The NEF will then convey the request to the NWDAF1 as defined in the clause 6.1.2.2, TS 23.288 [5].

2a-2e	Upon receiption of the request from the NF directly or via the NEF for untrusted NF, the NWDAF1 may subscribe/request to events from AMF/AF/OAM for UE location information, as the steps 2a-2e described in the clause 6.7.2.4, TS 23.288 [5].The NWDAF figures out a candidate SUPI list who currently stays in Source Area of Interest A.

2f-2g.	For case 1, to obtain UE mobility analytics in Target Area of Interest B and related target period 1, the NWDAF1 sends Analytics subscribe/request (Analytics ID = UE mobility, Target of Analytics Reporting = list of SUPIs, Analytics Filter information = Target AOI B, Analytics Reporting information = Analytics target period 1) to the NWDAF2 serving the Target Area of Interest. The NWDAF2 provides data analytics to the NWDAF1 including SUPI list 2 and their related mobility analytics, who once stayed in Target Area of Interest B in target period 1.

For case 2, besides the same interactions between NWDAF1 and NWDAF2 to obtain SUPI list 2 and their UE mobility analytics in Target Area of Interest B and related target period 1, in addition,to obtain SUPI list 3 and UE mobility analytics in Target Area of Interest C and related target period 2, the NWDAF1 sends Analytics subscribe/request (Analytics ID = UE mobility, Target of Analytics Reporting = list of SUPIs, Analytics Filter information = Target AOI C, Analytics Reporting information = Analytics target period 2) to the NWDAF3 serving the Target Area of Interest C. The NWDAF3 provides data analytics to the NWDAF1 including SUPI list 3 and their related mobility analytics, who once stayed in Target Area of Interest C in target period 2.

NOTE:	It is assumed that the NWDAF2 could collect data and keep for a certain period of time, e.g. 2 weeks, which could be configured by the operators.

3.	The NWDAF1 derives requested analytics, i.e.:

- For case 1, based on SUPI list 2 and their UE mobility analytics in Target AOI B provided by NWDAF2 and the UE mobility analytics in Source AOI A obtained by NWDAF1 itself, NWDAF1 derives the SUPI list and their aggregated UE mobility analytics, who currently stay in Source AOI A and once stayed in Target AOI B in target period 1.

- For case 2, based on SUPI list 2 and their UE mobility analytics in Target AOI B provided by NWDAF2, SUPI list 3 and their UE mobility analytics in Target AOI C provided by NWDAF3, and the UE mobility analytics in Source AOI A obtained by NWDAF1 itself, NWDAF1 derives the intersection SUPI list and their aggregated UE mobility analytics, who currently stay in Source AOI A and once stayed in Target AOI B in target period 1 and in Target AOI C in target period 2.

4a-4b.	The NWDAF1 sends the requested UE analytics to the NF/AF. The details for UE mobility analytics provided by the NWDAF are defined in the clause 6.7.2.3, TS 23.288 [5]. Based on the analytics provided by the NWDAF1, the NF/AF derives the mobility related information for these UEs.

For these untrusted NFs, the NWDAF1 provides requested UE analytics to the NF via a NEF.

6.25.5	Impacts on services, entities and interfaces

NWDAF:

-  	For UE mobility analytics, extend the filter information of the existing Nnwdaf_AnalyticsSubscription_Subscribe service or Nnwdaf_AnalyticsInfo_Request service, by including a new analytic filter = Source Area of Interest and Target Area of Interest(s), and extending Analytics Reporting information to contain multiple Analytics target periods if necessary.

NEF:

-	For UE mobility analytics, extend the filter information of the existing Nnef_AnalyticsExposure_Subscribe service, by including a new analytic filter = Source Area of Interest and Target Area of Interest(s), and extending Analytics Reporting information to contain multiple Analytics target periods if necessary.

* * * * End of Changes * * * *