3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008654r01

Elbonia, November 16 – 20, 2020, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI #13, Evaluation: Update to add the configuration information

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution provides updates to the evaluation of solution #75 clarifying the configuration information used for the triggers in KI#13.

# 1. Introduction/Discussion

This contribution provides clarifications to the description of Sol#75 highlighting the main principle of Sol#75 related to the definition of the configuration information about the triggers to analytics subscription. Such configuration information is referenced as analytics subscription information in Sol#75.

Additionally, the analytics subscription information proposed in Solution #75 is complementary to the solutions #45 and #74, as it can support the refinement of the configuration of the target of analytics IDs that is used by the NFs to subscribe/request analytics IDs to NWDAF.

In this contribution we propose the following:

- Update the description of the solution #75;

- Update descriptions of solution #75 in the Table 7.13-1 with clarifications in further comments column and removing the solution #75 as focusing on the triggers.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-91

# * * * * First change * * * *

## 7.13	Key Issue #13: Triggering conditions for analytics

For Key Issue #13 "Triggering conditions for analytics", there are solutions mainly as follows:

-	Solution #10: Handling of mixed and distributed NWDAF deployments

-	Solution #37: Reduction of Frequency from NFs

-	Solution #44: Analytics for Session Management Congestion Control Experience

-	Solution #45: Triggers for requesting analytics

-	Solution #65: Triggers for data collection from the UEs

-	Solution #73: Trigger configuration for data collection and analytics

-	Solution #74: Triggers for requesting analytics

-	Solution #75: AF influencing NFs triggers for interactions with NWDAF

Solution #10 proposes mixed and distributed NWDAF deployment when multiple NWDAFs are deployed in a network. Solution #10 mainly addresses KI#2 "Multiple NWDAF instances" and KI#11"Increasing efficiency of data collection", and therefore can be further evaluated and concluded under KI#2 and/or KI#11.

Solution #37 proposes that the NWDAF can activate or deactivate the reception of notifications from the NFs, which allows NWDAF to collect data from NFs in bulks and reduce the signalling for notifications between the NFs and NWDAF. Solution #37 mainly addresses KI#11 "Increasing efficiency of data collection" and therefore can be further evaluated and concluded under KI#11.

Solution #65 proposes that the NWDAF decides when to trigger data collection from UEs based on the notification from the AMF on the ratio/number of UEs supporting data for analytics in e.g. a given area or a slice. Solution #65 addresses also KI#8 "UE data as an input for analytics generation" and is highly dependent on the assumption and conclusion of KI#8, so it can be further evaluated and concluded under KI#8.

Solution #44 introduces a new Analytics ID "Session Management Congestion Control Experience (SMCCE)", where the SMF uses potential congestion condition as trigger to request for the SMCCE analytics to the NWDAF.

Solution #45 proposes that the analytics consumers (e.g. PCF, AMF, SMF) request for or subscribe to analytics from the NWDAF, based on triggers (e.g. event reports or requests) from other NF(s), local event, analytics information received, and etc.

Solution #73 proposes that the PCF provides the configuration on triggering conditions for data collection and analytics as network analytics control information to the NWDAF and to analytics consumers (e.g. AMF, SMF), and the AF may provide service specific analytics control information to the PCF for setting the network analytics control information for specific applications/services.

Solution #74 highlights that NF selection and Slice selection procedures may trigger analytics consumers to request for or subscribe to analytics from the NWDAF.

Solution #75 proposes that the AF provides the Analytics Subscription Information (ASI), which defines the restriction and/or permission of analytics IDs for specific UEs or group of UEs, as configuration for triggers used by analytics consumers to interact with NWDAF for analytics when triggering analytics ID subscriptions or analytics ID requests.

Table 7.13-1 gives the comparison of candidate solutions for Key Issue#13.

Table 7.13-1: Comparison of solutions for Key Issue#13

| Solutions | Aspects addressed for the Key Issue |  |  |  | Impacted | Other comments |
| --- | --- | --- | --- | --- | --- | --- |
|  | Triggers for requesting or subscribing to analytics by analytics consumers | Triggers for data collection by NWDAF | Triggers for analytics generation by NWDAF | Configuration for triggers |  |  |
| 10 | √ |  |  |  | NWDAF, analytics consumer NFs. New parameters defined for existing services. | Mainly addressing KI#2 and #11. Can be evaluated and concluded under KI#2/#11. |
| 37 |  | √ |  |  | NWDAF, data provider NFs. New parameters defined for existing services. | Mainly addressing KI#11. Can be evaluated and concluded under KI#11. |
| 44 | √ |  |  | yes | SMF, NWDAF. New analytics ID defined. |  |
| 45 | √ | internal | internal | yes | analytics consumer NFs. No changes to existing services. |  |
| 65 | N/A | N/A | N/A | N/A |  | Also addressing and highly depends on KI#8. Can be evaluated and concluded under KI#8. |
| 73 | √ | √ | √ | √ Provision of analytics control information to AMF/SMF, or Network Analytics policy to NWDAF, by the PCF. AF may be involved. | PCF, NWDAF, analytics consumer NFs, AF. For provision of analytics control information to AMF/SMF, new parameters defined for existing services. For provision of Network Analytics policy to NWDAF, new services defined. | Addressing also KI#18 on data collection/analytics enhancements of real-time communication with NWDAF |
| 74 | √ |  |  |  | analytics consumer NFs. No changes to existing services. |  |
| 75 |  |  |  | √ Provision of analytics subscription information (ASI) via the UDM. AF involved. | AF, UDM, analytics consumer NFs. New services defined. | Complementary to the solutions #45 and #74, supporting the refinement of the configuration of the target of analytics IDs used by the NFs to subscribe/request analytics IDs to NWDAF. |

# * * * * End of changes * * * *