SA WG2 Meeting #142E e-meeting	S2-2008659r04

Elbonia, November 16 – 30, 2020	(Revision of S2-200xxxx)

# Source:	China Mobile, Huawei

# Title:	Proposed New WID on Enablers for Network Automation for 5G - phase 2

# Document for:	Approval

Agenda Item:	9.1/8.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

# Title:	Enablers for Network Automation for 5G – phase 2

## Acronym:	eNA_Ph2

## Unique identifier:

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don’t know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | Antecedent study item (TR 23.700-91) |
| 760047 | Study of enablers for Network Automation for 5G | Antecedent study item (TR 23.791) |
| 830047 | Enablers for Network Automation for 5G | Antecedent Work item (TS 23.288) |

## 3	Justification

Rel-15 and Rel-16 specify framework to enable data collection and provide analytics to consumers i.e. defining extensions to existing Nnwdaf services to support the analytics that are required for e.g. QoS Profile Provisioning, Traffic Routing, Future Background Data Transfer, Slice SLA, Performance Improvement and Supervision of mIoT Terminals, Support of Northbound Network Status Exposure and Customizing Mobility Management.

In order to improve the NWDAF related work initiated in Rel-15 and Rel-16, it looks beneficial to further specific solutions for supporting network automation leveraging 5GC information exposure and network data analytics.

This work item aims at specifying system enhancements that have been studied in FS_eNA_Ph2 and will allow 5GS to support network analytics enhancement. The work will specify the architecture enhancement, the necessary data to expose to NWDAF and the necessary NWDAF outputs (i.e. statistics / predictions) in order to support  use cases listed in TR 23.700-91, and solutions and procedures defined in the study phase will be provided in this normative work.

## 4	Objective

This work item aims at further specifying system enhancements for NWDAF, based on what has been specified in Rel-15 and Rel-16, and will allow 5GS to support network automation.. The objective of this work item is to:

-	Specify architecture or framework enhancements for 5G System to support network data analytics service:

-	Logical function decomposition of NWDAF (Model Training logical function, Analytics logical function) and the possible interactions between these logical functions.

-    Increasing efficiency of data collection

-   Trained data model sharing between multiple NWDAF instances

-	Scenarios for multiple NWDAF instances: Hierarchical NWDAF architecture on interaction of NWDAFs or Analytics/Data aggregation, Re-selection of NWDAF and some other specific aspects (Time coordination, Abnormal Behaviour interactions and Exposing UE Mobility analytics).

-	UE data as an input for analytics generation (via AF).

-	User consent for UE data collection/analysis.

NOTE: Normative work on User consent for UE data collection/analysis depends on the LS reply from SA3.

-	Triggering conditions for the Data Analytics.

-	Enhancement for real-time communication between the NWDAF Service Consumer and the NWDAF.

-	Define extensions to existing Nnwdaf services to support the analytics as described in clause 7.0 of TR 23.700-91, required for e.g. Slice SLA enhancement, Dispersion Analytics, NWDAF-assisted UP optimization, NWDAF-assisted RFSP policy, NWDAF-assisted Application Detection, UP optimization for edge computing, adding application attributes to User Data congestion Analytics..

The solutions for the aforementioned objectives will be standardized based on the conclusions reached within TR 23.700-91 (clause 8).

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| 23.501 | Architectural and Conceptual enhancements to support the stated objectives | SA#93, September, 2021 |
| 23.502 | Procedural enhancements to support the stated objectives | SA#3, September2021 |
| 23.503 | Policy enhancements to support the stated objectives | SA#93, September, 2021 |
| 23.288 | Architecture enhancements and use cases for 5G System (5GS) to support network data analytics services | SA#93, September, 2021 |

## 6	Work item Rapporteur(s)

Aihua Li, China Mobile

liaihua@chinamobile.com

Xiaobo Wu, Huawei Technologies

wuxiaobo@huawei.com

## 7	Work item leadership

SA2

## 8	Aspects that involve other WGs

Coordination with SA3, SA4 and SA5 is expected during the work.

## 9	Supporting Individual Members

Supporting IM name

China Mobile

China Telecom

Huawei

China Unicom

China Southern Power Grid Co., Ltd

AT&T

CATT

Telecom Italia

KDDI

CAICT

NEC

Alibaba

Lenovo

ZTE

Verizon UK Ltd

Intel

Deutsche Telekom

Convida Wireless

Motorola Mobility

Ericsson

Orange

Sprint

NTT DoCoMo

ETRI

Tencent

Cisco Systems

OPPO

Vivo

Nokia

Nokia Shanghai Bell

LG Electronics

Samsung

Spirent

Sandvine

KT

InterDigital