SA WG2 Meeting #142E e-meeting	S2-2008659r12

Elbonia, November 16 – 30, 2020	(Revision of S2-200xxxx)

# Source:	China Mobile, Huawei

# Title:	Proposed New WID on Enablers for Network Automation for 5G - phase 2

# Document for:	Approval

Agenda Item:	9.1/8.1

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title:	Enablers for Network Automation for 5G – phase 2

Acronym:	eNA_Ph2

Unique identifier:

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X |  | X |  |
| No | X |  | X |  |  |
| Don’t know |  |  |  |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 840022 | Study on Enablers for Network Automation for 5G - phase 2 | Antecedent study item (TR 23.700-91) |
| 760047 | Study of enablers for Network Automation for 5G | Antecedent study item (TR 23.791) (Rel-16) |
| 830047 | Enablers for Network Automation for 5G | Antecedent Work item (TS 23.288) (Rel-16) |

3	Justification

In Rel-15 and Rel-16 SA2 specified framework to enable data collection, and provide analytics to consumers via defining extensions to existing Nnwdaf services to support the analytics that are required for e.g. QoS Profile Provisioning, Traffic Routing, Future Background Data Transfer, Slice SLA, Performance Improvement and Supervision of mIoT Terminals, Support of Northbound Network Status Exposure and Customizing Mobility Management.

In order to enhance the NWDAF related work initiated in Rel-15 and Rel-16, it looks beneficial to further specify solutions for supporting network automation leveraging 5GC information exposure and delivering network data analytics.

This work item aims at specifying system enhancements that have been studied in FS_eNA_Ph2 and will allow 5GS to support network analytics enhancement. The work will specify the architecture enhancement, the necessary input data to expose to NWDAF and the necessary NWDAF outputs (i.e. statistics / predictions) in order to support the use cases listed in TR 23.700-91, and solutions and procedures concluded in the study phase will be provided in this normative work.

4	Objective

This work item aims at further specifying system enhancements for NWDAF, based on what has been specified in Rel-15 and Rel-16, and will allow 5GS to support network automation.. The objective of this work item is to:

-	Specify architecture or framework enhancements for 5G System to support network data analytics service:

-	Logical function decomposition of NWDAF (Model Training logical function, Analytics logical function) and the possible interactions between these logical functions, as per conclusions in clause 8.1;

-    Increasing efficiency of data collection as per conclusions in clause 8.11: Signalling reduction via parametrization and services changes, signalling reduction on Tracking and Discovery of Entities, signalling reduction via Event Exposure service enhancements and signalling reduction via architectural changes;

-   Trained data model sharing between multiple NWDAF instances, limited to single vendor environments, as per conclusions in clause 8.19;

-	Scenarios for multiple NWDAF instances as per conclusions in clause 8.2: interaction of NWDAFs for Analytics/Data aggregation, re-selection of NWDAF and some other specific aspects (Time coordination, Abnormal Behaviour analytics interactions and Exposing UE Mobility analytics).

-	UE data as an input for analytics generation (via AF) , as per conclusions in clause 8.8;

NOTE 1: An alignment on UE data as an input for analytics generation will be carried out during normative work in coordination with SA3 and SA4.

-	User consent for UE data collection/analysis, as per conclusions in clause 8.15;

NOTE 2: Normative work on User consent for UE data collection/analysis needs to be coordinated with SA3.

-	Triggering conditions for the Data Analytics, as per conclusions in clause 8.13;

-	Enhancement for real-time communication between the NWDAF Service Consumer and the NWDAF, as per conclusions in clause 8.18;

-	Define extensions to existing Nnwdaf services as well as other NF services required to support the analytics approved for normative work,  e.g. for:

-	Slice SLA enhancement as per conclusions in clause 8.4,

-	Dispersion Analytics as per conclusions in clause 8.9,

-	NWDAF-assisted UP optimization as per conclusions in clause 8.10,

-	NWDAF-assisted RFSP policy as per conclusions in clause 8.12,

-	NWDAF-assisted Application Detection as per conclusions in clause 8.14,

-	UP optimization for edge computing, as per conclusions in clause 8.16

-	adding application attributes to User Data congestion Analytics as per conclusions in clause 8.7.

When UPF data is needed at NWDAF, the mechanisms to collect UPF data will not be specified.

The solutions for the aforementioned objectives will be standardized based on the conclusions reached within TR 23.700-91 (clause 8).

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | Series | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |
| 23.501 | Architectural and Conceptual enhancements to support the stated objectives | SA#93, September, 2021 |
| 23.502 | Procedural enhancements to support the stated objectives | SA#3, September2021 |
| 23.503 | Policy enhancements to support the stated objectives | SA#93, September, 2021 |
| 23.288 | Architecture enhancements and use cases for 5G System (5GS) to support network data analytics services | SA#93, September, 2021 |

6	Work item Rapporteur(s)

Aihua Li, China Mobile, who is responsible for Logical function decomposition of NWDAF, Increasing efficiency of data collection,Trained data model sharing between multiple NWDAF instances and Scenarios for multiple NWDAF instances.

liaihua@chinamobile.com

Xiaobo Wu, Huawei Technologies, who is responsible for all the others aspects.

wuxiaobo@huawei.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

Coordination with SA3, SA4 and SA5 is expected during the work.

9	Supporting Individual Members

Supporting IM name

China Mobile

China Telecom

Huawei

China Unicom

China Southern Power Grid Co., Ltd

AT&T

CATT

Telecom Italia

KDDI

CAICT

NEC

Alibaba

Lenovo

ZTE

Verizon UK Ltd

Intel

Deutsche Telekom

Convida Wireless

Motorola Mobility

Ericsson

Orange

Sprint

NTT DoCoMo

ETRI

Tencent

Cisco Systems

OPPO

Vivo

Nokia

Nokia Shanghai Bell

LG Electronics

Samsung

T-Mobile USA

Sandvine

KT

InterDigital

Qualcomm

KPN

Matrixx