3GPP TSG-WG SA2 Meeting #142E e-meeting	S2-2008679r02

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Samsung, CATT, Ericsson

Title:	KI#4, update on conclusion

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract of the contribution: This contribution proposes to update conclusion on KI #4.

# Discussion

Broadcast communication services have no different aspects from multicast services especially about QoS level control, except that Join/leave procedures and individual delivery mode etc. are not considered for broadcast services.

In current conclusion for KI#4, there is a NOTE as follows which is related to KI#1 conclusion: "

NOTE:	Depending on KI#1 conclusions, MB-SMF may obtain QoS information in different ways, e.g.:

-	MB-SMF may get QoS info from PCF: MB-SMF uses policy rules provided by the PCF for the MBS session QoS determination, which includes QoS parameters for multicast/broadcast mode and sends these to MB-SMF.

-	MB-SMF may also get QoS info from AF (via NEF/MBSF)."

Regarding QoS information provision, in KI#1 conclusion, it states that “For multicast service parameters storage, the UDR shall be able to store the AF provisioned or preconfigured service parameters per MBS session, the PCF shall be able to provide policy and QoS requirement per MBS session to the MB-SMF.” So it shall be supported that the PCF provides MBS related policy rules which includes QoS parameters for MBS flows to the MB-SMF, if dynamic PCC is deployed for MBS services. Considering that the multicast service data can be transported via 5GC Shared MBS traffic delivery method and 5GC Individual MBS traffic delivery method, the policy rules provided by the PCF for the MBS session shall include QoS parameters for 5GC Shared MBS traffic delivery method.

The MB-SMF may get MBS QoS info from the AF via the NEF/MBSF. If dynamic PCC is deployed, the AF shall send MBS service information and QoS information to the PCF via the NEF/MBSF, and then the MB-SMF receives the QoS parameters for MBS flows from the PCF.

R02: The following are from S2-2008525:

AF specific priority for 5MBS group member

Solution #35 allows a 5MBS group member to get priority (over the other members) within a multicast MBS Session. This is basically compliant with the SA1 requirement in TS 22.280, e.g. clause 5.6 (MCX service priority requirements) contains the following:

The MCX Service Emergency Alert is initiated from an MCX UE to inform the MCX Service of the MCX User's immediate need of assistance due to the MCX User's personal, life-threatening situation.

The above requirement applies to unicast delivery and is equally applicable for group member involved in an MBS Session.

[Proposal] Therefore, it’s proposed to adopt AF specific priority for 5MBS group member as an optional feature.

MB Session-AMBR

Sol#36 introduces MBS Session-AMBR which intends to limit the aggregate bit rate that can be expected to be provided across all Non-GBR QoS Flows for an MBS Session.

In our view, the concept of Session-AMBR for PDU Session does not apply to MB Session, reasoning:

-	when non-GBR MB QoS Flow is set up to differentiate QoS for different MB service data flows of an MBS Session, the QoS parameters for an MB service data flow is either provided by the AF or by PCF (if deployed for MBS Session), and the QoS enforcement will be applied to that MB service data flow. In particular,

-	MBR policing for the MB service data flow (which is DL only) will be enforced in the MB-UPF, therefore there is no need for NG-RAN to do MBR policing again, therefore the bit rate for non-GBR MB QoS Flow does not need to be sent to the NG-RAN.

[Proposal] It’s proposed to remove MBS Session-AMBR.

# Proposal

It is proposed to add the following changes into TR 23.757.

******************************** START OF CHANGES*********************************

## 8.4	Key Issue #4: QoS level support for Multicast and Broadcast communication services

The following principles are applied for normative work for multicast and broadcast communication services:

-	The network shall support QoS control per MBS session instead of per user.

-	The network shall support one or multiple QoS flow for an MBS session.

-	The network may use dedicated QoS flows for multicast sessions in a PDU session if 5GC individually delivery is use to deliver the 5MBS data packet.

-	The 5G QoS model and parameters as defined in TS 23.501 [2] clause 5.7 also apply to MBS service with the following differences:

-	Reflective QoS is not applicable;

-	Wireline access network specific 5G QoS parameters do not apply to MBS services;

-	Alternative QoS Profile is not applicable;

-	QoS Notification Control is not applicable;

-	UE AMBR is not applicable;

-	Session-AMBR is not applicable.

-	There is support for both GBR and non-GBR MBS flows.

-	AF provides the MBS session information description including QoS requirements to the 5GC.

-	The MB-SMF obtains QoS information and configures the MB-UPF accordingly.

-	If dynamic PCC is used, the MB-SMF obtains QoS information for MBS session in policy rules provided by the PCF, for MBS session QoS determination. The QoS information includes QoS parameters for 5GC Shared MBS traffic delivery method.

-	The MB-SMF may get MBS QoS information from the AF via the NEF/MBSF. If dynamic PCC is used, the NEF/MBSF provides QoS information to the MB-SMF via the PCF.

-	Interactions between MB-SMF and SMF and signalling of QoS profile to NG-RAN have dependencies with other Key Issues e.g. key issue #1 and the overall architecture and will be determined based on conclusions for Key Issue #1 and the overall architecture.

The following principle is applied for normative work for multicast communication services:

-	Optionally, for multicast communication, the AF may set specific QoS requirement (i.e. priority) of a specific UE within an MBS Session to ensure that the UE within an MBS Session can get priority when needed.

******************************** END OF CHANGES **************************************