3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008683r05

Elbonia, November 16 – 20, 2020, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Evaluation and conclusion update of pre-configuration

Document for:	Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2 / Rel-17

Abstract: This contribution updates the evaluation part for PC5 DRX schedule levels.

1. Introduction

This contribution evaluates the potential options for provisioning of PC5 DRX schedules based on following criteria:

-	Possibility to set up a proper DRX configuration (i.e. the radio resource).

-	Efficiency of the DRX configuration, e.g. fragmentation of the radio resource.

For each class of solution evaluation is added, taking the above criteria into account.

.2. Text Proposal

It is proposed to capture the following changes vs. TR 23.776.

# * * * * First change * * * *

7.1.1	Evaluation for the level of Provisioned PC5 DRX schedules

Provisioned default PC5 DRX schedules (or cycles, as they are termed by some solutions) can be associated with the PC5 RAT type (i.e., one default cycle for all NR PC5 communications, as suggested by Solution#6) or they can be more fine-granular, e.g., associated with either V2X service types (as suggested by Solution#2) or PQIs (as suggested by Solution#5). If V2X services apply their own, independent, and uncoordinated PC5 DRX cycles, this could indeed lead to increased ON durations of the UEs that run many services.

The main advantages and disadvantages of the different approaches are listed in the following:

-	PC5 DRX cycles/schedules per PC5 RAT

-	Advantages:

-	Simpler solution leading to less network traffic when re-provisioning/re-configuration is required..

-	Disadvantages:

-	It is not possible to predefine an appropriate DRX configuration (i.e. the radio resources) in a RAT based granularity, as it is not possible to estimate how much traffic will be transmitted on each RAT.

- 	 It does not provide much benefit in case the transmitting UEs cannot be configured to transmit in the right intervals (note that this is a probably disadvantage of all approaches that use PC5 DRX for PC5 broadcast communications).

-	May cause congestion and fragmentation of the preconfigured radio resources.

-	This option requires (pre-)configuration in all the UEs, including vehicles and pedestrian. In addition, if any change happens (e.g., change the existing DRX configuration or add new DRX configuration for new V2X service type), many UEs (including vehicles and pedestrian) may have to update the configuration. Otherwise, the UEs have misalignment of DRX configuration.

-	Too coarse PC5 DRX cycles that do not reduce the ON durations as much as it could be done for certain V2X services or V2X service combinations

-	PC5 DRX cycles/schedules per V2X service type(s)

-	Advantages:

-	Alignment of the PC5 DRX cycles of the UEs that run the same V2X service types or V2X service type combinations. Shorter ON durations in case the UE is interested in limited V2X service types due to the high granularity of the set of applicable PC5 DRX cycles, thus helping to save energy.

- 	This option enables the UEs in groupcast and broadcast to have a predefined DRX configuration per V2X service type, so that the UEs can transmit/receive V2X service data at a predefined time for its interested service(s).

-	Disadvantages:

-	It does not provide much benefit in case the transmitting UEs cannot be configured to transmit in the right intervals (note that this is a probably disadvantage of all approaches that use PC5 DRX for PC5 broadcast communications).

-	May cause congestion and fragmentation of the (pre-)configured radio resources

-	This option requires (pre-)configuration in all the UEs, including vehicles and pedestrian. In addition, if any change happens (e.g., change the existing default DRX schedules or add new ones for new V2X service type), all the UEs (including vehicles and pedestrian) have to be updated with the new information. Otherwise, the UEs have misalignment of DRX configuration.

-	The more V2X service types the UE is interested in, the lower the chances for power savings.

-	In case V2X applications have different QoS requirements for the same V2X service type a longer PC5 DRX ON duration will be required to support the highest QoS reducing the power efficiency of the UE.

-	PC5 DRX cycles/schedules per PQI

-	Advantages:

-	Disadvantages:

Editor's note:	The further analysis of advantages and disadvantages or the drawing of any conclusions based on them is FFS and shall be based on RAN WG feedback.

# * * * * End of changes * * * *