3GPP TSG-WG SA2 Meeting #142E 	S2-2008709r02

# 16 November - 23 November 2020, Elbonia	(revision of S2-200xxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	Sol #23: Updates to clarify on SBI based interface for A&A

Document for:	Approval

Agenda Item:	8.7

Work Item / Release:	FS_ID_UAS-SA2 / Rel-17

Abstract of the contribution: This paper updates solution #23

# 1 Introduction

Updates are related to removing the optional EAP based option for A&A and only keep SBI based approach for A&A

Since in this release of specification UAV controller is considered as a normal UE, the paper is updated to provide procedure for UAV authentication and authorization only. However, if in future there is a need to perform A&A for UAV controller also, the same procedures as explained for an UAV can also be used for the UAV controller.

Further updates include clearly depicting in the procedure that the A&A is initiated by the SMF, but the interaction with USS/UTM is done via a different NF (it could be a new NF as depicted in the figure or extension of an existing NF like NEF) so that USS/UTM has a single interface towards the 3GPP system.

# 2 Proposal

It is proposed to approve the below content. All changes are marked with revision marks.

* * * * Begin of Changes * * * *

## 6.23	Solution #23: UAV/UAVC authentication/authorization by USS/UTM and C2 communication establishment

### 6.23.1	Introduction

This solution addresses Key Issue #2, #3, #5, #6 and #7. This solution is applicable to both 5GC and EPC. The solution facilitates below capabilities:

-	Authentication/authorization of UAV by USS/UTM during PDU session establishment

-	Obtain UAV and networked UAVC IP address for C2 communication (from 3GPP network) and perform necessary C2 communication authentication/authorization and pairing.

-	Initiate re- authentication/ re-authorization when deemed necessary by the USS/UTM

-	Revoke authentication/authorization and tear down the PDU sessions

### 6.23.2	Functional Description

#### 6.23.2.1	Assumptions

The solution is based on below assumptions:

-	An UAS consists of one UAV controller (UAVC) and one UAV. A single USS/UTM is serving both the UAV and the UAVC associated within an UAS

-	The process for associating the UAV and UAVC to form an UAS is outside the scope of 3GPP and is assumed to be done at the USS/UTM.

-	The UAV and UAVC forming the UAS may be served by different MNO. The identification, authentication / authorization of the UAVC and UAV can happen independently.

-	The registration of the UAV with aviation authority (CAA) is outside the scope of 3GPP. Once the UAV is registered to the CAA, a CAA-Level UAV ID is assigned to the UAV.

-	At 3GPP level the UAV is publicly identified by an MNO assigned GPSI in the form of an External Identifier.

-	The UAV may establish separate PDU sessions with dedicated DNN/S-NSSAI (or dedicated APNs in EPC) for

-	Communication with USS/UTM (e.g. OAM purpose, Networked Remote Id, Tracking etc)

-	Command and control (C2) communication between UAV and mutually exclusively an UAVC or a TPAE or an USS/UTM

-	Possibly One or more PDU sessions (PDN connection in EPC) for other communication purposes (e.g. for video streaming etc).

-	The UAV gets via means out of scope of 3GPP a USS/UTM address and this is included by the UAV in the request for the PDU session establishment. How the USS/UTM address is configured in UAV is outside the scope of 3GPP.

-	Mapping between CAA-Level UAV ID and External Identifier is maintained by the USS/UTM.

-	For EPC, the PDN connections used by UAV are served by a combo SMF+PGW-C regardless of whether the UAV support 5G NAS or whether their subscription allows access to 5GC. This allows reusing common procedures between EPC and 5GC; and of course, allows UAV to freely move between 5GC and EPC coverage.

-	The solution re-uses (with necessary adaptation) Secondary authentication/authorization by a DN-AAA.

NOTE 1:	Re-using (with necessary adaptation) of Secondary authentication/authorization by a DN-AAA allows to reuse an existing feature (limiting the cost of introduction of the UAS support by 3GPP systems) that can equally apply over EPC and 5GC.

NOTE 2:	APN used by the UAV for contacting the USS/UTM resolves to SMF+PGW-C only.

NOTE 3:	The 3GPP system shall be able to verify that the USS/UTM address provided by the UAV/UAVC UE can be trusted. Security solutions for verification of the USS/UTM address and establishing trust relationship by 3GPP network shall be covered at SA WG3.

#### 6.23.2.2	Overview of the solution

An UAS consists of one UAV controller (UAVC) and one UAV. The UAVC can be a networked meaning the UAVC is connected to a 3GPP network and connected to the UAV via the 3GPP network or the UAVC can be non-networked meaning the UAVC is not connected to the 3GPP network and connected to UAV via a transport outside the scope of 3GPP e.g. internet. In this release of specification, a networked UAVC is considered as a normal UE, without having any aerial subscription and without having the need for secondary authentication & authorization by the USS/UTM. However, the concepts defined in the paper for authentication & authorization of an UAV by the USS/UTM can be utilised for a networked UAVC also, if in future there is a need to do so.

Based on the type of UAVC (networked or non-networked) used in an UAS, the complete solution can be viewed in two different representation from 3GPP point view.

An overview of the solution is first provided for the case of a non-networked UAVC;

NOTE 1:	Despite the fact that a 5GC wording is used in the figure, it applies also to EPC with e.g. PDU sessions replaced with PDN connections

Figure 6.23.2.1-1: Overall solution for an UAV with non-networked UAVC

The UAV may establish multiple PDU sessions.

In Figure 6.23.2.1-1 above, PDU session1 is used for communication with USS/UTM. This PDU session may be used by the UAV to register with the USS/UTM and provide any further information that the USS/UTM may need to register the UAS. During the PDU session establishment the USS/UTM may perform Authentication & authorization of the UAV. This PDU session may further be used for any communication with USS/UTM that are not related to command and control (C2) operations. For example: for Networked Remote ID (NRID) or for tracking purpose or even for USS/UTM to update certain C2 communication related configuration in the UAV.

For C2 communication the UAV may establish a second PDU session. The USS/UTM authenticates/authorizes this PDU session establishment request for C2 communication by the UAV. The USS/UTM also performs authentication/authorization of the UAV if it was not performed earlier. The USS/UTM may also allocate a dynamically generated CAA-Level UAV ID (e.g. session Id) to the UAV. After the second PDU session establishment, the USS/UTM receives the C2 communication IP address(es) for the UAV from the 3GPP network. The USS/UTM may use this IP address to configure the authorized UAVC to control the UAV, however, how this configuration is done is outside the scope of 3GPP.

When the UAVC IP address for C2 communication is available with the USS/UTM, it configures this IP address in the UAV and may also further do some additional configuration in the UAV to enable C2 communication. PDU Session1 of the UAV may be used for such communication between UAV and USS/UTM.

The USS/UTM also provides (as part of authorization data) the C2 communication IP address of the UAVC to the 3GPP network anchoring the PDU session for C2 communication of the UAV. This IP address in configured in Access Control List (ACL) for the PDU session such that, using the second PDU session the UAV is able to access only the configured IP addresses meant for C2 communication that are authorized by the USS/UTM.

The UAV can now connect on the C2 communication link over the PDU session2

NOTE 2:	SMF are configured to associate the PDU Session for C2 (corresponding to a specific (DNN, S-NSSAI)) with an empty default ACL; this means that as long as the USS/UTM has not authorized an IP address in the ACL, no communication is possible on such PDU session.

NOTE 3:	It is possible that by default the USS/UTM puts in the ACL it configures in the network its own IP in order to be able to take control of the UAV in case of issue ; this depends on USS/UTM policies that are out of scope of 3GPP.

NOTE 4:	The UAV may establish additional one or more other PDU sessions for various other purposes and are not shown in above.

Although in this release of specification a networked UAVC is considered as a normal UE without any aerial subscription and without the need for any A&A by USS/UTM Figure 6.23.2.1-2 below shows the solution overview with a networked UAVC, if in future such need arises to also perform A&A for UAVC by the USS/UTM. The UAV UE behaviour is independent of whether the UAVC is a networked or non-networked. In the case of a networked UAVC, as can be seen from the Figure 6.23.2.1-2, the UAVC UE also may establish multiple different PDU sessions e.g. one for communication with USS/UTM and another one specifically for C2 communication.

The UAV and networked UAVC registers to 3GPP network and establishes PDU sessions independent of each other. It is also possible that the UAV and UAVC are served by different MNOs. However, from UAS operations point of view both are served by a single USS/UTM system.

In the case of a networked UAVC, the authentication/authorization and C2 communication pairing configuration can be facilitated by the 3GPP, similar to how it works for an UAV.

Figure 6.23.2.1-2: Overall solution for an UAV with networked UAVC

#### 6.23.2.3	Usage of secondary DN AAA authentication / authorization

For 5GC the solution enhances the feature "Secondary authentication/authorization by a DN-AAA server during the establishment of a PDU Session" provided by 3GPP (TS 23.501, clause 5.6.6 and TS 23.502, clause 4.2.3.2) to expose a service based interface from SMF to facilitate the USS/UTM to authenticate/authorize the UAV using web based mechanism instead of an EAP based mechanism..

For EPC the solution introduces similar procedure as for 5GC, in the SMF+PGW-C, to facilitate the USS/UTM to authenticate/authorize the UAV.

It is proposed to use a new network function UAV NF (or alternatively enhance the existing NFs like NEF), that will provide a single interface towards the USS/UTM for authentication and authorization and for USS/UTM to access other 3GPP exposed services. The UAV NF may perform the USS/UTM selection (as described in clause 6.5.2.3) and will work as an intermediary between SMF (SMF+PGW-C) and USS/UTM for supporting the authentication & authorization of the UAV.

The UAV provides the necessary identifiers and information needed by the USS/UTM system. These may include (and not limited to and is out of scope of 3GPP, thus not further described):

-	UAV SN to identify the UAV HW

-	CAA-Level UAV ID

-	Other USS/UTM specific Identifiers/information to identify, authenticate/authorize the UAV controller, the pilot etc.

-	Additional aviation specific information like Current location, Flight information, flight authorization specific information, location (latitude, longitude, altitude) of the UAV controller etc.

PDU session (PDN connection in EPC) establishment for C2 communication is continued only after the USS/UTM has successfully authenticated/authorized the UAV. The authorization data is also provided to the 3GPP network.

The 3GPP system or the USS/UTM may initiate a re-authentication/re-authorization of the UAV at any point of time.

The USS/UTM may at any point revoke the authorization, forcing 3GPP network to tear down the PDU session or change the ACL associated with the PDU Session for C2 to e.g. remove the UAVC IP address from the list of authorized IP address and e.g. replace it with the IP address of a TPAE.

### 6.23.3	Procedures

#### 6.23.3.1	Authentication/authorization and C2 communication establishment of UAV/UAVC by USS/UTM via 5GC network

The procedure for UAV authentication & authorization and further authentication & authorization of C2 communication establishment by the USS/UTM during PDU session establishment in a 5GC network is depicted in Figure 6.23.3.1-1 below.

Figure 6.23.3.1-1: Authentication/authorization and C2 communication establishment for UAV/UAVC by USS/UTM via 5GC

1.	The UAV registers to the 3GPP network. The UE's subscription information obtained by AMF from the   UDM has an indication for support for Aerial UE function and the AMF provides the subscription information on Aerial UE authorization to the gNB.

NOTE 1:	It is assumed that the UAV has been registered to the CAA level authority, a CAA-Level UAV ID is available with the UAV/UAVC and the UAV/UAVC has a USS/UTM address configured.

2.	The UAV initiates a PDU session establishment request for communication with USS/UTM.

3.	Based on the UAV UE's subscriber profile, DNN and S-NSSAI provided in the request the AMF selects the SMF and sends Nsmf_PDUSession_CreateSMContext Request to the SMF. It is assumed that the UAV is configured with the appropriate S-NSSAI and DNN to use or that the 3GPP Network has configured the Default S-NSSAI and Default DNN to be dedicated for UAV-USS/UTM connectivity.

4.	The SMF obtains subscription data from the UDM for the given SUPI obtained from the AMF.

The SMF checks the subscription data whether a secondary authentication is required for the PDU session request and whether the UE request is allowed according to the user subscription and local policies. If the secondary authentication is required and the UE request is allowed, the SMF may further send a request to the UAV to fetch the USS/UTM specific identifier (e.g. CAA-Level UAV ID) and (optionally) a USS/UTM address for secondary authentication if it was not already provided by the UAV in step 2 above.

The SMF send UAV authentication & authorization request to the UAV NF including the CAA-Level UAV ID received from the UAV UE, the GPSI (e.g. External Identifier of the UE) and USS/UTM address if received. The SMF may also reserve an IP address for the PDU session and include the UE IP address information in the UAV authentication & authorization request to the UAV NF.

5.	UAV NF shall verify that the USS/UTM address provided by the UAV can be trusted. If the USS/UTM address provided by the UAV is an FQDN, UAV NF uses DNS resolution to find out the IP address of the USS/UTM. The UAV NF forwards the UAV authentication & authorization request to the selected USS/UTM.

6.	Depending on the security mechanis, multiple roundtrip messages may be exchanged between the UAV and USS/UTM.

NOTE 2:	Security details will be determined by SA WG3.

7.	If the authentication & authorization was successful at USS/UTM, the USS/UTM provides the successful response with authorization information to the UAV NF. The USS/UTM may maintain a mapping of the CAA-Level UAV ID and the 3GPP level UAV ID (i.e. the GPSI) to further access services exposed by the 3GPP network using the 3GPP level UAV ID.

8.	The UAV NF provides the UAV A&A response to the SMF.

9.	After successful authorization by the USS/UTM, further PDU session establishment procedure is continued.

10.	SMF may notify the USS/UTM the IP address(es) allocated to the UAV via the UAV NF if it was not provided in step 4. The USS/UTM may store the correlated GPSI and PDU session IP address with the authorized CAA-Level UAV-ID to enable further service requests via network exposure APIs to the 3GPP system related to the authorized UAV. The GPSI may e.g be used to activate monitoring of the UAV/UE location and the PDU session IP addess for requests for dedicated QoS and gating to be applied for the authorized PDU session. It may also be used to revoke the established connectivity/PDU-session in case it is deemed necessary by the USS/UTM.

11.	[Optional] USS/UTM may provide access control list information, so that the PDU session is used only for accessing UAS services (e.g. connecting to the USS/UTM or other UAV operations). For this the USS/UTM interacts with the PCF via UAV NF to configure dedicated polices.

12.	The UAV may use this PDU session for communicating with the USS/UTM e.g. for Networked Remote ID (NRID) and tracking related communication with USS/UTM.

13.	Once the UAV is ready for flight operation, it initiates a new PDU session establishment request for C2 communication if separate PDU sessions are used for USS/UTM communication and for C2 communication. If a single PDU session is used for both, the UAV UE requests for PDU session modification indicating C2 related operation.

14.	The SMF verifies whether a secondary authentication is required for the PDU session establishment/modification request and performs authentication of the request by the USS/UTM using the same procedures as explained in steps 4 – 8 for UAV authentication and authorization.

15.	SMF continues with the PDU session establishment/modification only after successful authentication & authorization done the by USS/UTM.

16.	If a separate PDU session was used for C2 communication and the UE IP address allocation for the PDU session was not included in the authentication & authorization request towards USS/UTM in step 14, the SMF notifies the IP addresses allocated to the UAV UE to the USS/UTM via the UAV NF. If the same PDU session is used for C2 communication this step is skipped as the UE IP address is already notified in step 4 or 10. The USS/UTM may store the correlated GPSI and PDU session IP address with the authorized CAA-Level UAV-ID to enable further service requests via network exposure APIs to the 3GPP system related to the authorized UAV. The PDU session IP addess may be used for requests for dedicated QoS and gating to be applied for the authorized PDU session. It may also be used to revoke the established connectivity/PDU-session in case it is deemed necessary by the USS/UTM.

17.	USS/UTM provides necessary access control list information, so that using the PDU session the UAV can be controlled only by authorized UAV controller or TPAE. If this pairing information is not available, the USS/UTM may provide its own IP address as a UAV controller. And later when the UAV controller IP address is known to the USS/UTM, it may update the access control list, using the same SBI interfaces, to allow C2 communication between the UAV and UAV controller using this PDU session. The USS/UTM interacts with the PCF via UFES/NEF to configure dedicated polices required for setting up C2 communication and/or UAV/UAVC pairing as "negotiated" between UAV(C) and USS/UTM. SMF updates the ACL in UPF with the IP addresses provided by the USS/UTM.

18.	USS/UTM may now further configure the C2 communication related information on the UAV. This is done at the application level using PDU session that was established in Step 2-9, and the complete communication is transparent to 3GPP.

--

#### 6.23.3.2	Authentication/authorization and C2 communication establishment of UAV/UAVC by USS/UTM via EPC network

Figure 6.23.3.2-1: Authentication/authorization and C2 communication establishment for UAV/UAVC by USS/UTM via EPC

The procedure for UAV authentication & authorization and further authentication & authorization of C2 communication establishment by the USS/UTM during PDN connection establishment in a EPC network is depicted in Figure 6.23.3.2-1 above.

[1-3].	The UAV UE sends an attach request to the EPC network. MME performs the primary authentication and downloads subscription data from HSS. The subscription data obtained by MME from the HSS has an indication for support for Aerial UE function and the MME provides the subscription information on Aerial UE authorization to the eNB. The MME initiates default bearer establishment procedure during the attach procedure according to TS 23.401 [10] Figure 5.3.2.1-1.

[4-7].	The SMF+PGW-C checks the subscription data whether a secondary authentication is required for the PDN connection establishment request and whether the UE request is allowed according to the user subscription and local policies and establishes a N4 session with UPF+PGWu (same behaviour as in 5GC case). If the secondary authentication is required, the SMF+PGW-C sends back Create Session Response. If the CAA-Level UAV ID and USS/UTM address were not included in step 3, the SMF+PGW-C includes a PCO in the Create Session Response requesting for the CAA-Level UAV ID and signed USS/UTM address. The SMF+PGW-C also ensures (same behaviour as in 5GC case), with appropriate ACL (Access Control List) configuration at the UPF+PGW-U, that the UE is not able to access any data network using this PDN connection until a secondary authentication is performed by the USS/UTM. The MME sends attach accept and the NAS message containing the PCO requesting CAA-Level UAV ID.

8.	The UE sends attach complete and also provides the CAA-Level UAV ID to the MME.

[9-10].	MME sends a Modify Bearer Request including the PCO containing the CAA-Level UAV ID to the SMF+PGW-C. The SMF+PGW-C then sends back a Modify Bearer Response to the MME.

11.	The SMF+PGW-C now performs the UAV A&A using a services based interface towards UAV NF as explained in steps 4 – 8 in clause 6.23.3.1.

NOTE 1:	Requesting CAA-Level UAV identifity in steps 6, 7 are completely optional. If CAA-Level UAV Id is always provided by the UAV UE in attach request, then the addional PCO requesting CAA-Level UAV Id is not included in step 7 and Step 9, 10 are completely skipped.

NOTE 2:	It is proposed to complete the attach procedure before the SMF+PGW-C completes authentication & authorization with USS/UTM to avoid situations like the attach procedure getting timed out at the UE side while the network awaits completion of authentication & authorization with USS/UTM.

12[a-c].	The SMF+PGW-C sends an Update Bearer Request to the MME including the authorization data in the PCO. The MME sends the PCO in a Downlink NAS transport message to the UAV UE and confirms the bearer update procedure by sending an Update Bearer Response to the SMF+PGW-C. SMF+PGW-C also notifies the IP address(es) allocated for the PDN connection of the UAV, to the USS/UTM via the UAV NF, if it was not included in the authentication & authorization request towards USS/UTM in step 11. The USS/UTM may store the correlated External Identifier and UE IP address with the authorized CAA-Level UAV-ID to enable further service requests via network exposure APIs to the 3GPP system related to the authorized UAV. The External Identifier may e.g be used to activate monitoring of the UAV/UE location and the UE IP addess for requests for dedicated QoS and gating to be applied for the authorized PDN connection. It may also be used to revoke the established PDN connection in case it is deemed necessary by the USS/UTM.

13.	[Optional] USS/UTM may provide access control list information, so that the PDN connection is used only for accessing UAS services (e.g. connecting to the USS/UTM or other UAV operations). For this the USS/UTM interacts with the PCF via UAV NF to configure dedicated polices. The SMF+PGW-C then configures the Access Control List into the UPF+PGW-C

14.	The UAV may use this PDN connection for communicating with the USS/UTM e.g. for Networked Remote ID (NRID) and tracking related communication with USS/UTM.

15.	Once the UAV is ready for flight operation, it initiates a new PDN connection establishment request for C2 communication if separate PDN connections are used for USS/UTM communication and for C2 communication. If a single PDN connection is used for both, then the UAV UE requests for Bearer modification indicating C2 related operation.

16.	The SMF+PGW-C verifies whether a secondary authentication is required for the PDN connection establishment/modification request and performs authentication of the request by the USS/UTM using the same procedures as explained in steps 4 – 8 of clause 6.23.3.1 for UAV authentication and authorization.

17.	SMF+PGW-C continues with the PDN connection establishment/bearer modification only after successful authentication & authorization done the by USS/UTM.

18.	If a separate PDN connection is used for C2 communication and the UE IP address allocation for the PDN connection was not included in the authentication & authorization request towards USS/UTM in step 16, the SMF+PGW-C notifies the IP addresses allocated to the UAV UE for the PDN connection, to the USS/UTM via the UAV NF. If the same PDN connection is used for C2 communication this step is skipped as the UE IP address is already notified to the USS/UTM. The USS/UTM may store the correlated External Idenfitier and UE IP address with the authorized CAA-Level UAV-ID to enable further service requests via network exposure APIs to the 3GPP system related to the authorized UAV. The UE IP addess may be used for requests for dedicated QoS and gating to be applied for the authorized PDN connection. It may also be used to revoke the established PDN connection in case it is deemed necessary by the USS/UTM.

19.	USS/UTM provides necessary access control list information, so that using the PDN connection the UAV can be controlled only by authorized UAV controller or TPAE. If this pairing information is not available, the USS/UTM may provide its own IP address as a UAV controller. And later when the UAV controller IP address is known to the USS/UTM, it may update the access control list, using the same SBI interfaces, to allow C2 communication between the UAV and UAV controller using this PDN connection. SMF+PGW-C updates the ACL in UPF+PGWu with the IP addresses provided by the USS/UTM.

20.	USS/UTM may now further configure the C2 communication related information on the UAV. This is done at the application level using PDN connection that was established in Step 3-13, and the complete communication is transparent to 3GPP.

--

#### 6.23.3.3	Authentication/authorization revocation by USS/UTM

Figure 6.23.3.3-1: Authentication/authorization revocation or update

When the USS/UTM determines the need for updating the authorization data (e.g. update the access control list) or revoke the UAV authorization (or C2 communication authorization), it uses the service based interface exposed by UAV NF to send the authorization update request or authorization revocation request. The request is sent on the 3GPP Level ID that the USS/UTM had received during the A&A procedure. The SMF (or SMF+PGW-C) updates the access control list for the PDU session (or PDN connection) based on the information received in the authorization data update request. If the request was for revocation of the authorization, the SMF may decide to terminate the PDU session (or PDN connection). The SMF then sends a response acknowledging the successful execution of the authorization data update or revocation.

### 6.23.4	Impacts on existing entities and interfaces

This solution may have the following impacts to existing entities and interfaces:

For 5GC:

-	N1: UE provides the signed USS/UTM address and CAA-Level UAV ID in the PDU session establishment request

-	SMF: Retrieves signed USS/UTM address and the CAA-Level UAV ID from the PDU session establishment request. Define a new SBI interface (if needed) for secondary DN authentication/authorization during PDU session establishment. When the USS/UTM provides a success response for the secondary authentication result (via UAV AF), use the authorization data to configure the ACL in UPF, such that the UE can access only the configures list of IP addresses using the PDU session.

For EPC:

-	SMF+PGW-C: Supports "Secondary authorization/authentication by an DN-AAA server during the PDU Session establishment" using a SBI interface in the same way as the SMF in case of 5GC. Retrieves the USS/UTM address and CAA-Level UAV ID included in the PCO by the UE. Additionally, same changes as described above for SMF in 5GC case.

-	S1: Provides USS/UTM address and CAA-Level UAV ID to the SMF+PGW-C in a PCO

* * * * End of Changes * * * *