3GPP TSG-SA WG2 Meeting #142E 	S2-2008730r07

Elbonia, November 16 – 20, 2020

Source:	Huawei, HiSilicon, Intel

Title:	KI#2, Update to Conclusions on UE DNS cache

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes update to conclusions on UE DNS cache.

1	Introduction

In last meeting, it has concluded that the SMF may provide DNS cache clear indication within NAS message to UE. However, there are still two FFSs in the conclusion:

Editor’s Note: It is FFS when SMF sends the DNS cache clear indication to the UE, i.e. before the EAS relocation or after EAS relocation.

Editor’s Note: It is FFS whether the SMF send the associated information with DNS cache clear to the UE so the UE can clear a part of DNS cache or all DNS cache.

As already indicated in the conclusion that the indication do not impact the application client, and in NOTE1 that “If connectivity is available to the source EAS, then the application can select a new EAS after the DNS caching is flushed by the applications or cache timer expires”. It is clear that the indication is not related to the ongoing EAS relocation, i.e. the indication is not intended to be used to trigger the UE to reselect EAS for ongoing connections. It is used for the UE to reselect EAS when it triggers a new connection with the EAS, i.e. the connection to the EAS has not been established. In solution 32 and 34 during locla PSA-UPF insertion/change/removal procedure, based on information received from AF or stored locally the SMF sends the DNS cache clear information to UE via PDU Session Modification procedure. In case of EAS load balance, the AF may provide EAS information update indication to 5GC. This may trigger SMF to send EAS rediscovery indication too. Hence, the first Editor’s Note can be removed with clarification that the indication is provided to UE during UP path change (i.e. ULCL insertion/change/removal) or triggered by EAS information update due to AF load balance.

For the second editor’s note, it is possible that the SMF does not include information to identify which DNS records need to be removed. If so, the UE will remove all records stored in DNS cache. However, this option is not friendly. For example if application servers are in central DN, the DNS records stored in the DNS cache are related to central DN. For these records, the removal of them will cause AS re-discovery even if the AS server does not need to be changed. This will cause application layer delay (due to re-discovery) and more DNS traffic. It is recommended that the associated impacted field information, i.e. DNS Suffix, FQDNs or IP range are included in the DNS cache clear indication sent to UE, so that UE can identify the DNS records to be removed based on the associated impacted field information.

2	Proposal

It is proposed to add the following changes in TR 23.748.

**************************** Start Change ***************************

9. 2.2	Conclusions for Key Issue #2:  UE based EAS rediscovery

A new DNS query may be performed to reselect a new EAS. For reselection of a new EAS, the cached old EAS information in the UE should not be reused. For UE based EAS rediscovery, it is concluded:

- For session breakout case, the SMF may provide EAS rediscovery indication and optional its associated impact field within NAS message to UE as described in Solution #32 and #34 (only DNS cache related part). The SMF makes the decision based on information provided by AF or based on local configuration. The impacted field includes information DNS Suffix, FQDNs or IP ranges , which is used to identify the impacted DNS records. Without the impacted field the UE applies the EAS rediscovery indication to all applications associated with the PDU Session. Based on the received EAS rediscovery indication, when a new connection to EAS need to be established, the UE re-discovers the EAS according to KI#1 without reusing the cached old EAS information. This does not impact the application client.

NOTE 1:  	If connectivity is available to the source EAS, then the application can select a new EAS after the  DNS caching is flushed by the applications or cache timer expires

NOTE 2:	In case the UE does not support EAS rediscovery triggered by 5GC, the application will continue to be served by the old EAS until the UE Application Layer DNS cache timer expires or the applications determines to flush the DNS cache.

NOTE 3:	This EAS rediscovery indication does not impact the UE Application Layer DNS caching.

NOTE 4:	This EAS rediscovery indication does not apply to the scenario that UE agnostic to EAS IP replacement.

NOTE 5:  EAS rediscovery indication is used as the DNS cache clear indication in Solution #32, and EAS rediscovery indication is recommended to be used in the normative phase.

-	For SSC mode 2/3 case, the UE removes the cached DNS records associated with the released PDU Session and can reselect a new EAS after it is allocated with a new IP address.

********************************* End Change ****************************