3GPP TSG-SA WG2 Meeting #142E 	S2-2008730r07

Elbonia, November 16 – 20, 2020

Source:	Huawei, HiSilicon, Intel

Title:	KI#2, Update to Conclusions on UE DNS cache

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	FS_enh_EC /Rel-17

Abstract of the contribution: This contribution proposes update to conclusions on UE DNS cache.

1	Introduction

In last meeting, it has concluded that the SMF may provide DNS cache clear indication within NAS message to UE. However, there are still two FFSs in the conclusion:

Editor’s Note: It is FFS when SMF sends the DNS cache clear indication to the UE, i.e. before the EAS relocation or after EAS relocation.

Editor’s Note: It is FFS whether the SMF send the associated information with DNS cache clear to the UE so the UE can clear a part of DNS cache or all DNS cache.

As already indicated in the conclusion that the indication do not impact the application client, and in NOTE1 that “If connectivity is available to the source EAS, then the application can select a new EAS after the DNS caching is flushed by the applications or cache timer expires”. It is clear that the indication is not related to the ongoing EAS relocation, i.e. the indication is not intended to be used to trigger the UE to reselect EAS for ongoing connections. It is used for the UE to reselect EAS when it triggers a new connection with the EAS, i.e. the connection to the EAS has not been established. In solution 32 and 34 during locla PSA-UPF insertion/change/removal procedure (see 6.34.2.1) , based on information received from AF (see 6.34.2.3), PCF controlled local PCF insertion (see 6.34.2.2) or stored locally the SMF sends the local DN (?) information to UE via PDU Session Modification procedure. In case of EAS load balance, the AF may provide EAS information update indication to 5GC. This may trigger SMF to send EAS rediscovery indication too. Hence, the first Editor’s Note can be removed with clarification that the indication is provided to UE during UP path change (i.e. ULCL insertion/change/removal) or triggered by EAS information update due to AF load balance. During this procedure, the SMF may also send local DN identifer (i.e. DNAI), IP address range, domain name to the UE.

For the second editor’s note, it is possible that the SMF does not include information to identify which DNS records need to be removed. If so, the UE will remove all records stored in DNS cache. However, this option is not friendly. For example if application servers are in central DN, the DNS records stored in the DNS cache are related to central DN. For these records, the removal of them does not prevent the UE from re-discovery of new EAS even if the AS server does not need to be changed. This will cause application layer delay (due to re-discovery) and more DNS traffic.

2	Proposal

It is proposed to add the following changes in TR 23.748.

**************************** Start Change ***************************

9. 2.2	Conclusions for Key Issue #2:  UE based EAS rediscovery

A new DNS query may be performed to reselect a new EAS. For reselection of a new EAS, the old EAS information in the UE should not be reused. For UE based EAS rediscovery, it is concluded:

- For session breakout case, the SMF may provide EAS rediscovery indication and optional its associated impact field within NAS message to UE as described in Solution #32 and #34 (only DNS cache related part). The SMF makes the decision based on information provided PCC rules provided by PCF, which can be PCF’s decision or influenced by AF or based on SMF’s local configuration. The impacted field includes information DNS Suffix (i.e. domain name), FQDNs or IP address ranges of the local DN. Without the impacted field the UE applies the EAS rediscovery indication to all applications associated with the PDU Session. Based on the received EAS rediscovery indication, the old connection can be tone down and when a new connection to EAS need to be established, the UE can re-discover the new EAS  This does not impact the application client, but can not guarantee the triggering the application client to re-discover the new EAS.

NOTE 1:  	If connectivity is available to the source EAS, then the application can select a new EAS after the  DNS caching is flushed by the applications or cache timer expires

NOTE 2:	The application can continue to be served by the old EAS until the UE Application Layer DNS cache timer expires or the applications determines to tear down the old EAS connection or DNS client in the UE flushes the DNS cache.

NOTE 3:	This EAS rediscovery indication does not impact the UE Application Layer DNS caching.

NOTE 4:	This EAS rediscovery indication does not apply to the scenario that UE agnostic to EAS IP replacement.

-	For SSC mode 2, the UE can removes the old EAS information associated with the released PDU Session and can reselect a new EAS after with the UE receives a new IP address in a PDU Session Release Command with 5GSM cause “reactivation requested”.

-   For SSC mode 3, the UE can remove the old EAS information associated with the released PDU Session and can reselect a new EAS after the UE receives a new IP address in a PDU Session Modification Command with 5GSM cause "reactivation requested".

********************************* End Change ****************************