3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008733

Elbonia, November 16 – 20, 2020	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4: Updates of conclusion for KI#4

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2/ Rel-17

Abstract: This contribution provides the updates of conclusion for KI#4.

1. Introduction/Discussion

As described in KI#4, this key issue covers Network Slice related quotas defined in KIs, in particular, KI#1 and KI#2. The selected solutions of KI#1 and KI#2 should support the event subscription and notification of the registered UEs and the established PDU sessions within an S-NSSAI, accordingly.

This contribution proposes to remove the highlighted part of the following Editor’s note related to KI#1 and KI#2 by providing further information for KI#1 and KI#2.

Editor’s note:  Key issue #4 depends on other KIs for quota management. Therefore, the 5GC NF (including whether the 5GC NF is the same NF providing the quota management functionality) and the NF service(s) to receive data from other 5GC NFs will be determined based on the conclusion of KI#1, KI#2, KI#3 and KI#5 in a subsequent meeting.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-40.

# * * * * First change * * * *

8.4	Conclusion for Key Issue #4

For Key Issue #4 (Support for network slice quota event notification in a network slice):

-	The 5GC NF (Control Plane NF within the 5G Core Network) has an interface with AF (optionally via NEF) and manages event subscription/notification from the AF for all network slice related quota such as number of registered UEs, number of established PDU sessions and limitation of data rate per network slice.

-	The 5GC NF determines whether event notification triggering condition is met based on data received from other 5GC NFs via service based interface.

Editor's note:	Key issue #4 depends on other KIs for quota management. Therefore, the 5GC NF (including whether the 5GC NF is the same NF providing the quota management functionality) and the NF service(s) to receive data from other 5GC NFs will be determined based on the conclusion of KI#1, KI#2, KI#3 and KI#5 in a subsequent meeting.

For managing event subscription and notification of the registered UEs in an S-NSSAI, it shall be supported by the 5GC NF selected for the NW Slice quota management functionality in KI#1.

For managing event subscription and notification of the established PDU Sessions in an S-NSSAI, it shall be supported by the 5GC NF selected for the NW Slice quota management functionality in KI#2.

# * * * * End of changes * * * *