3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008735r01

Elbonia, November 16 – 20, 2020

Source:	Huawei, HiSilicon, CATT

Title:	KI#7: Conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution proposes to updater the conclusion for KI#7.

- Introduction

In S2#141E e-meeting, regarding KI#7 the conclusions has been achieved with three Editor’s Notes captured. Per the analysis in S2-2008736, this paper proposes to update the conclusion of KI#7.

In S2-2006044 "LS on RAN impact of FS_5MBS Study", one question is asked on PTP/PTM delivery method switching: "4.	Some solution suggests the 5GC sends MBS assistance information to RAN for PTP/PTM delivery method decision and switching.	SA2 would appreciate RAN2 and RAN3 feedback on the above and comments, if any." To this question, RAN WG2 give the following response in S2-2009094 (R2-2011271) "Reply LS on RAN impact of FS_5MBS Study": "RAN2 agreed that at least information of MBS services/groups subscribed by the UE (e.g. TMGI) and QoS requirements of a MBS service should be provided to RAN for MBS operation in general. RAN2 has not concluded whether any information from CN is needed, e.g. for PTP/PTM delivery method decision and switching." Based on the response, an EN is proposed to the conclusion for KI#7.

Regarding "switching between 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery method", currently it is only addressed in the conclusion for "delivery method switching due to mobility". However, switching between 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery method shall also be performed based on network policy, service requirements, etc., triggered by the MB-SMF, PCF and/or MBSF, as proposed in solution #23, #24, #25, #28 and #31. So it is proposed to also address this in the conclusion for KI#7.

2. Proposal

It is proposed to capture the following changes in TR 23.757.

# * * * * First change * * * *

# 8	Conclusions

## 8.7	Key Issue #7: Reliable delivery method switching between unicast and multicast

For delivery method switching due to mobility, the following principle are agreed,

- When the UE moves from a NG-RAN node that supports 5MBS to a RAN node that does not support 5MBS, the network and UE shall support switch from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- When the UE moves from a RAN node that does not support 5MBS to a NG-RAN node that supports 5MBS, the network and UE shall support switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- Before the inter RAN node handover procedure is triggered, the PDU session, which is used for related MBS Session traffic individual delivery, need be activated. The 5GC need provide the information of linkage between the Multicast QoS flow and the corresponding mapped unicast QoS flow to the UE and NG-RAN.

- During the handover from RAN not supporting 5MBS to NG-RAN supporting 5MBS, PDU sessions, including the one associated with MBS session and used for individual MBS traffic delivery, are handed over to target RAN. After the handover, the switch is triggered at the 5GC from the 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- During the handover from NG-RAN supporting 5MBS to RAN not supporting 5MBS, the Multicast QoS flow information and its corresponding mapped unicast QoS flow information are all sent from source NG-RAN node to the target RAN node. The session continuity is supported via the data forwarding tunnel between the source NG-RAN node and target RAN node. To support lossless packet transferring the mechanism defined in clause 6.27.2.3 is used as baseline for normative work.

- During the handover from NG-RAN supporting 5MBS to RAN not supporting 5MBS, the MBS Session flow is converted to the QoS flow within the associated PDU session at the target RAN. 5GC switch the MBS traffic delivery method from 5GC Shared MBS traffic delivery to 5GC Individual MBS traffic delivery.

- The 5GC NF, which select the MBS delivery method in the MBS session establishment procedure, triggers the MBS traffic delivery method switch between 5GC Individual MBS traffic delivery method and 5GC Shared MBS traffic delivery method.

For delivery method switching not due to mobility, the following principle are agreed,

- Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for of switching the PTP and PTM delivery methods.

Editor’s Note:	Information of MBS services/groups subscribed by the UE (e.g. TMGI) and QoS requirements of a MBS service are provided to RAN for MBS AN resources handling. Whether any assistance information from CN is needed, e.g. for PTP/PTM delivery method decision and switching, needs further confirmation when the relevant conclusion is reached in RAN WGs.

- If the NG-RAN node support 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

NOTE:  	When a UE is handed over from an NG-RAN node not supporting 5MBS, Individual MBS traffic delivery is still used during the handover process. After the handover, 5GC triggers the switch from the 5GC Individual MBS traffic delivery to 5GC Shared MBS traffic delivery.

- Switching between 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery method shall be supported based on network policy, service requirements, etc.

Switching between multicast delivery and unicast delivery using individual UE and application server addresses is not specified in normative work.

# * * * * End of changes * * * *