3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008735r01

Elbonia, November 16 – 20, 2020

Source:	Huawei, HiSilicon, CATT

Title:	KI#7: Conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution proposes to updater the conclusion for KI#7.

- Introduction

In S2#141E e-meeting, regarding KI#7 the conclusions has been achieved with three Editor’s Notes captured. Per the analysis in S2-2008736, this paper proposes to update the conclusion of KI#7.

In S2-2006044 "LS on RAN impact of FS_5MBS Study", one question is asked on PTP/PTM delivery method switching: "4.	Some solution suggests the 5GC sends MBS assistance information to RAN for PTP/PTM delivery method decision and switching.	SA2 would appreciate RAN2 and RAN3 feedback on the above and comments, if any." To this question, RAN WG2 give the following response in S2-2009094 (R2-2011271) "Reply LS on RAN impact of FS_5MBS Study": "RAN2 agreed that at least information of MBS services/groups subscribed by the UE (e.g. TMGI) and QoS requirements of a MBS service should be provided to RAN for MBS operation in general. RAN2 has not concluded whether any information from CN is needed, e.g. for PTP/PTM delivery method decision and switching." Based on the response, an EN is proposed to the conclusion for KI#7.

2. Proposal

It is proposed to capture the following changes in TR 23.757.

# * * * * First change * * * *

# 8	Conclusions

## 8.7	Key Issue #7: Reliable delivery method switching between unicast and multicast

For delivery method switching due to mobility, the following principle are agreed,

- When the UE moves from a NG-RAN node that supports 5MBS to a RAN node that does not support 5MBS, the network and UE shall support switch from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- When the UE moves from a RAN node that does not support 5MBS to a NG-RAN node that supports 5MBS, the network and UE shall support switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- When the UE moves from a source NG-RAN node that supports 5MBS to a target RAN node that does not support 5MBS, the UE shall support establishing/activating the PDU Session associated with MBS session at the target RAN node for 5GC Individual MBS traffic delivery.

- Before the handover regarding to MBS from NG-RAN supporting 5MBS to RAN not supporting 5MBS, the PDU session, which is associated with MBS session, needs to be activated for 5GC Individual MBS traffic delivery at target RAN node. The 5GC needs to provide the information of linkage between the Multicast QoS flow and the corresponding mapped unicast QoS flow to the UE and the NG-RAN node.

- During the handover regarding to MBS from RAN not supporting 5MBS to NG-RAN supporting 5MBS, PDU sessions, including the one associated with MBS session and used for 5GC Individual MBS traffic delivery, are handed over to target RAN. After the handover, the switch is triggered at the 5GC from the 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- During the inter RAN node handover regarding to MBS, the lossless session continuity is supported via data forwarding. The 5GC shall use same sequence number for same packet of the MBS session to archive the lossless session continuity.

- During the handover regarding to MBS from NG-RAN supporting 5MBS to RAN not supporting 5MBS, the MBS Session flow is converted to the QoS flow within the associated PDU session at the target RAN. The 5GC triggers the switching from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- The 5GC NF, which selects the MBS traffic delivery method in the MBS session establishment procedure, triggers the switching between 5GC Individual MBS traffic delivery method and 5GC Shared MBS traffic delivery method.

- If lossless delivery is required for a MBS session, the UE receiving MBS traffic of the MBS session shall be in CM-CONNECTED state.

For delivery method switching not due to mobility, the following principle are agreed,

- Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for of switching the PTP and PTM delivery methods.

Editor’s Note:	Whether any assistance information from CN is needed, e.g. for PTP/PTM delivery method decision and switching, needs further confirmation when the relevant conclusion is reached in RAN WGs.

- If the NG-RAN node support 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

NOTE:  	When a UE is handed over from an NG-RAN node not supporting 5MBS, Individual MBS traffic delivery is still used during the handover process. After the handover, 5GC triggers the switch from the 5GC Individual MBS traffic delivery to 5GC Shared MBS traffic delivery.

Switching between multicast delivery and unicast delivery using individual UE and application server addresses is not specified in normative work.

# * * * * End of changes * * * *