3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008735r07

Elbonia, November 16 – 20, 2020

Source:	Huawei, HiSilicon, CATT

Title:	KI#7: Conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution proposes to updater the conclusion for KI#7.

- Introduction

In S2#141E e-meeting, regarding KI#7 the conclusions has been achieved with three Editor’s Notes captured. Per the analysis in S2-2008736, this paper proposes to update the conclusion of KI#7.

In S2-2006044 "LS on RAN impact of FS_5MBS Study", one question is asked on PTP/PTM delivery method switching: "4.	Some solution suggests the 5GC sends MBS assistance information to RAN for PTP/PTM delivery method decision and switching.	SA2 would appreciate RAN2 and RAN3 feedback on the above and comments, if any." To this question, RAN WG2 give the following response in S2-2009094 (R2-2011271) "Reply LS on RAN impact of FS_5MBS Study": "RAN2 agreed that at least information of MBS services/groups subscribed by the UE (e.g. TMGI) and QoS requirements of a MBS service should be provided to RAN for MBS operation in general. RAN2 has not concluded whether any information from CN is needed, e.g. for PTP/PTM delivery method decision and switching." Based on the response, an EN is proposed to the conclusion for KI#7.

# 2. Proposal

It is proposed to capture the following changes in TR 23.757.

# * * * * First change * * * *

# 8	Conclusions

## 8.7	Key Issue #7: Reliable delivery method switching between unicast and multicast

For delivery method switching due to mobility, the following principle are agreed,

- When the UE moves from a NG-RAN node that supports 5MBS to a RAN node that does not support 5MBS, the network and UE shall support switch from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- When the UE moves from a RAN node that does not support 5MBS to a NG-RAN node that supports 5MBS, the network and UE shall support switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- When the UE moves towards a target NG-RAN node not supporting 5MBS, the UE shall support establishing/activating the PDU Session associated with MBS session at the target NG-RAN node for 5GC Individual MBS traffic delivery.

- When the UE joins an MBS session and handover the MBS session towards NG-RAN nodes not supporting 5MBS is required, mapping information for association between 5GC Shared MBS traffic delivery and 5GC Individual MBS traffic delivery is provided to the UE and to the NG-RAN node to enable data reception of the MBS session via 5GC Individual MBS traffic delivery mode during handover.

- At the handover regarding to MBS towards NG-RAN node not supporting 5MBS, the user plane resources for 5GC Individual MBS traffic delivery need to be activated.

- During the handover from RAN not supporting 5MBS to NG-RAN supporting 5MBS, PDU sessions, including the one associated with an MBS session and used for 5GC Individual MBS traffic delivery, are handed over to target RAN. After the handover, the switch is triggered at the 5GC from the 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- During the inter NG-RAN node handover regarding to MBS, the lossless session continuity is supported via data forwarding.

- During the handover from NG-RAN supporting 5MBS towards NG-RAN not supporting 5MBS, the MBS Session flow is converted to the QoS flow within the associated PDU session at the target RAN. After the handover, MBSF and MB-SMF are notified and the 5GC triggers the switching from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- If lossless delivery is required for a MBS session, the UE receiving MBS traffic of the MBS session shall be in CM-CONNECTED with RRC-CONNECTED state.

For delivery method switching not due to mobility, the following principle are agreed,

- Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for of switching the PTP and PTM delivery methods.

Editor’s Note:	Whether any assistance information from CN is needed, e.g. for PTP/PTM delivery method decision and switching, needs further confirmation when the relevant conclusion is reached in RAN WGs.

- If the NG-RAN node support 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

NOTE:  	When a UE is handed over from an NG-RAN node not supporting 5MBS to an NG-RAN node supporting 5MBS, 5GC Individual MBS traffic delivery is used at the target NG-RAN node until 5GC delivery method switch to shared delivery is done.

Switching between multicast delivery and unicast delivery using individual UE and application server addresses is not specified in normative work.

# * * * * End of changes * * * *