3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2008735r16

Elbonia, November 16 – 20, 2020

Source:	Huawei, HiSilicon, CATT

Title:	KI#7: Conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution proposes to updater the conclusion for KI#7.

- Introduction

In S2#141E e-meeting, regarding KI#7 the conclusions has been achieved with three Editor’s Notes captured. Per the analysis in S2-2008736, this paper proposes to update the conclusion of KI#7.

In S2-2006044 "LS on RAN impact of FS_5MBS Study", one question is asked on PTP/PTM delivery method switching: "4.	Some solution suggests the 5GC sends MBS assistance information to RAN for PTP/PTM delivery method decision and switching.	SA2 would appreciate RAN2 and RAN3 feedback on the above and comments, if any." To this question, RAN WG2 give the following response in S2-2009094 (R2-2011271) "Reply LS on RAN impact of FS_5MBS Study": "RAN2 agreed that at least information of MBS services/groups subscribed by the UE (e.g. TMGI) and QoS requirements of a MBS service should be provided to RAN for MBS operation in general. RAN2 has not concluded whether any information from CN is needed, e.g. for PTP/PTM delivery method decision and switching." Based on the response, an EN is proposed to the conclusion for KI#7.

# 2. Proposal

It is proposed to capture the following changes in TR 23.757.

# * * * * First change * * * *

# 8	Conclusions

## 8.7	Key Issue #7: Reliable delivery method switching between unicast and multicast

For delivery method switching due to mobility, the following principle are agreed,

- When the UE moves from a NG-RAN node that supports 5MBS to a RAN node that does not support 5MBS, the network and UE shall support switch from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- When the UE moves from a RAN node that does not support 5MBS to a NG-RAN node that supports 5MBS, the network and UE shall support switch from 5GC Individual MBS traffic delivery method to 5GC Shared MBS traffic delivery method.

- When the UE joins an MBS session and handover to NG-RAN nodes not supporting 5MBS is required, mapping information is provided to the UE and to the NG-RAN node supporting MBS, which enables data reception of the MBS session via 5GC Individual MBS traffic delivery mode.

- Before handover to an NG-RAN node not supporting 5MBS, the N3 tunnel of the PDU Session, which is used for 5GC Individual MBS traffic delivery, need to be activated.

- During the handover from RAN not supporting 5MBS to NG-RAN supporting 5MBS, PDU sessions, including the one associated with the MBS session and used for 5GC Individual MBS traffic delivery, are handed over to target RAN.

NOTE 1:  	How 5GC Shared MBS delivery is enabled for the UE will be developed with RAN WGs.

- During the mobility from NG-RAN supporting 5MBS to NG-RAN not supporting 5MBS, the 5GC triggers the switching from 5GC Shared MBS traffic delivery method to 5GC Individual MBS traffic delivery method.

- The 5GC NF, which selects the delivery method during the MBS session establishment procedure, triggers the switching between 5GC Individual MBS traffic delivery method and 5GC Shared MBS traffic delivery method.

NOTE 2:	The 5GC NF that triggers the switching between 5GC Individual MBS traffic delivery method and 5GC Shared MBS traffic delivery method depends on conclusion about SMF vs AMF centric approach in key isuue#1.

NOTE 3: It is FFS whether the support for lossless handover with data forwarding from source NG-RAN to the target NG-RAN is needed, which needs confirmation by RAN.

- It is commonly understood that if service requirements result in applying ‘lossless handover’ (see TS 38.300), UEs receiving MBS traffic of that MBS session need to be in CM-CONNECTED with RRC-CONNECTED state.

NOTE 4: 	When the UE detects it is moving out of the coverage of 5MBS capable NG-RAN, it may activate a PDU session at target for traffic delivery per service logic (i.e. break-before-make).

For delivery method switching not due to mobility, the following principle are agreed,

- Switching between PTP and PTM delivery methods for 5GC Shared MBS traffic delivery shall be supported. NG-RAN is the decision point for of switching the PTP and PTM delivery methods.

NOTE 5:	Whether any assistance information from CN is needed, e.g. for PTP/PTM delivery method decision and switching, needs further confirmation when the relevant conclusion is reached in RAN WGs.

- If the NG-RAN node supports 5MBS, the network shall use the 5GC Shared MBS traffic delivery method for MBS Session packet transfer.

NOTE 6:  	When a UE is handed over from an NG-RAN node not supporting 5MBS to an NG-RAN node supporting 5MBS, 5GC Individual MBS traffic delivery is still used at the target NG-RAN node until delivery method switch is done.

Switching between multicast delivery and unicast delivery using individual UE and application server addresses is not specified in normative work.

# * * * * End of changes * * * *