SA WG2 Meeting #142E	S2-2008775r1

Nov 16 – Nov 20, 2020, Elbonia

Source:	OPPO, Convida Wireless

Title:	KI#8, conclusion for KI#8 –eNA

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: conclusion of UE data for reporting

1. Discussion

Based on evaluation in S2-2007015, in solution in eNA KI#8, for the data collected from UE, it can be put into three groups:

- Application layer parameters:

Route, Destination, Average speed, Time Interval, Application status (in soln#29); QoS and Service experience contribution (in solution#63)

- NAS layer parameters:

enforcement of URSP rules, WLANSP rules;

- AS layer parameters

none

Actually, the AS layer parameters can be reported using MDT and Application layer parameters can be reported by OTT application server (3rd party AF), however there is no way to provide such NAS layer parameters and they are important to let the network provision proper policies to UE and know that they are being applied as intended. For example, Solution #28 explains that the UE can report the time when URSP Rules are applied. This information may be used by the NWDAF to check if policies are being applied as intended and help the network monitor what slices are being used by applications.

Therefore, it is proposed to agree the parameters described in solution#28 (enforcement of URSP rules) and solution#62 (performance of WLANSP) for the UE data to be collected by NWDAF.

2. Proposal

* * * * Start of Change * * * *

8	Conclusions

Editor's note:	This clause is intended to list conclusions that have been agreed during the course of the study item activities. This should also capture the guiding principles and documentation approach for creating CRs to normative specifications within the responsibility of SA WG2.

A new analytics Id on the performance of URSP rules is agreed for normative work. To achieve this, NWDAF may collect the input data from:

- PCF: the distributed URSP rule (Traffic Descriptor + RSD) for a SUPI;

-  AF: the UE IP address/prefix serving the UE;

-  SMF: acquire SM Context related to RSD parameters (e.g. S-NSSAI, DNN).

NOTE: NWDAF can acquire UE IP address/prefix serving the UE from an AF and get the SUPI corresponding to the UE IP address/prefix, then the SM context (RSD related parameters) from SMF. The SM context can be mapped to the RSD of the URSP rule, then NWDAF knows which URSP rule is being used for the AF (application).

* * * * End of Change * * * *