SA WG2 Meeting #S2-142E 	S2-2008784

16 - 20 November, 2020, Electronic meeting

Source:	China Mobile

Title:	KI#5 evaluation and conclusion update

Document for:	Approval

Agenda Item:	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution includes updating the evaluation part for key 5 and make a conclusion.

# 1. Discussion

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-40.

# * * * * First change * * * *

# 7	Evaluation

Editor's note:	This clause will provide a general evaluation of the solutions.

## 7.5	Evaluation on solutions of KI#5

From all 8 solutions proposed for KI#5 (Solution #12, #14, #16, #18, #19, #20, #24, #25), some may have a complete solution and some not. But overall, we can summarize that there are three main functionalities for supporting limitation of data rate per network slice in UL and DL as described below.

- NW Slice max. data rate Storage functionality: This functionality is responsible for storing a NW Slice maximum data rate quota information

- NW Slice Quota Control functionality: This functionality is responsible to check that the aggregated data rate for the S-NSSAI in UL/DL complies with the maximum rate for the S-NSSAI in the UL/DL.

- NW Slice Quota Enforcement functionality: This functionality is responsible to ensure that the data rate per slice is not exceeded. If exceeded actions are taken to a) redistribute the data rates, or b) reject new PDU session establishments or new UE registration or some other action such as report to OAM to log the event..

Editor's note:	It is FFS for NW Slice Quota Enforcement functionality whether option a) or option b) or both options should be supported.

Table 7.5-1 illustrates 5GS system impacts for all solutions proposed for the KI#5. Moreover, it also compares where to place the afore-mentioned network slice quota functionalities in the 5G system.

Table 7.5-1: Key impacts of the solutions

|  | UE | RAN | Notes |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Impact | Impact | NW Slice max. data rate Storage | NW Slice Quota control | NW Slice Quota Enforcement | Amount of signalling for enforcement | Amount of signalling for adjustment | Description |
| Sol#12 | No | No | NSQ | NSQ | N/A | The centralized new NF checks the quota per each PDU session. Additional signalling to PDU session est, and modification or when a new QoS is estab/updated/release. Additional signalling to update PDU session bit rates. | If the Network Slice max data rate is reached the NSQ contacts all (or a subset) of SMFs in the slice to update Session-AMBR or MFBR for GFBR data flows. | For each PDU session SMF provides to NSQ the authorized Session-AMBR and MFBR for each GBFR flow. If any updates, the SMF updates the NSQ. |
| Sol#14 (depends on KI#3) | Yes | Yes | PCF | PCF/NWDAF | RAN | - | When the number of UE changes and new analytics comes from NWDAF, PCF adjust UE-Slice-AMBR for each UE. | PCF decides the UE-Slice-AMBR based on NWDAF analytics on network slice data rate. PCF provides the UE-Slice-AMBR to AMF. The AMF provides the UE-Slice-AMBR to RAN. RAN controls the UE-Slice-AMBR. |
| Sol#16 | No | No | PCF | PCF | UPF | - | The periodic reporting proposed by UPF/SMF to PCF helps dynamic adjustment of local quota allocation. It can increase the amount of signalling between the PCF and SMF. | The UPFs get quotas of UL/DL bitrate allowed to transfer and enforce them to meet the limitation of the slice. The PCF of the slice (i.e. Slice PCF) collects the actual bitrate in N6 interface per S-NSSAI from the UPFs and periodically adjusts the bitrate quota allocation according to the latest service distribution within the slice. |
| Sol#18 (depend on KI#3) | No | No | SQM | SQM/ AMF | AMF | When SQM standalone, additional signalling for each UE registration to provide a quota per Slice if not available. | . When the local quota is reached the AMF notifies the SQM for further quota or other actions | The SQM allocates a quota per Slice and provides it to AMF. The AMF sums the Slice-MBR of all UEs in Connected mode which have established PDU Session in the network slice and ensures the total data rate does not exceed the allowed value in the quota for the network slice. SQM is either stand alone or a functionality of PCF, NSSF, NRF, NWDAF, OAM, CHF. |
| Sol#19 | No | N/A | QCF | QEF | N/A | - | The QCF may initiate adjustment of quota distribution among the QEFs when the NDWAF notifies new analytics. | QCF determines the local slice quota based on the analytics from NWDAF. The QEF enforces the network slice local quota. |
| Sol#20 | No | No | PCF | PCF | N/A | - | Additional signalling to update PDU session bit rates. | The PCF (per slice) aggregates the Session-AMBR and MBR per PCC Rule in the slice and check if the quota is reached. When the quota is reached the PCF (per slice) decides if a new PDU session is established or if other PDU session needs reconfiguration based on internal policies. |
| Sol#24 | No | No | NSQ | NSQ | N/A | - | User plane adjustment is initiated by SMF with reduced Session-AMBR/MFBR values if the data rate quota runs out. | Similar as solution #12 |
| Sol#25 (depends on KI#3) | Yes | Yes | PCF | PCF | RAN | - | Adjusting the UE-Slice-MBR of the UEs as more UEs join or leave a network slice with at least one PDU session. | The PCF (per slice) allocates a Slice-MBR per UE that is sent to RAN for enforcement. |

From Table 7.5-1 above, one can derived the following three categories:

-	Category A with enforcement of Slice max bit rate for each UE in RAN (#14,#25).

-	Category B with enforcement of Slice max bit rate in control plan function to control that the accumulate bit rate for all PDU sessions within the Slice do not exceed the Slice max bit rate.(#12,#18, #19, #20, #24).

-	Category C with enforcement of slice max bit rate in the user plane by distributing a quota to UPF for enforcement.(#16).

For Category A and Category B, the control is based on the authorized maximum bitrate of each UE or each PDU session. Considering that not every UE or every PDU session can reach the maximum data rate allowed simultaneously, the data rate of the slice (i.e. the sum of the data rate of each UE or each PDU session) may be treated as exceeding the quota even the actual data rate of the slice is still lower than the quota. For Category C, it is feasible to implement UPF enforcement than estimating a proper Slice-MBR value for each UE which can make the total actual data rate of the slice close to meeting the quota, and this solution(#16) has less impact on UE.  The UPFs ensure that the aggregated bitrate across all GBR and Non-GBR QoS Flows is not exceeding the bitrate quota of the slice by shaping the traffic of non-GBR QoS Flows and/or dropping packets exceeding the GFBR from GBR QoS Flows with MFBR>GFBR. The enforcement is based on the actual data rate of the slice, so solution #16 is suggested to be within normative work.

The solutions, both #14 and #25, that propose RAN enforcement are linked to KI#3 and their solutions, as they propose enforcement of UE-Slice-MBR in RAN.

There is no clear benefit that shows if using analytics as input to the decision point adds value for slice quota enforcement.

From signalling point of view, the centralized NSQ in Sol#12 and Sol#24, acting as both slice max data rate storage functionality and quota control functionality has to check the quota per each PDU session which brings heavy signalling load of centralized NF. Quota distribution from slice max data rate storage functionality to quota control functionality can avoid the adverse impacts to the scalability and reliability of the system. Introducing a new NF causes increased signalling from AMF/SMF to these new NFs in every PDU session establishment or at new UE registrations. As such, the new functionality for control of the quota is included in an existing NF.

As for dynamic adjustment, considering that the number of NF is much less than the number of involved UEs and PDU Sessions, the signalling load for dynamic adjustment per NF is less than adjustment per UE or per PDU session.

For Sol#18, AMF is responsible for the slice-level MBR calculation and once the quota reaches the slice-level MBR, new PDU session will not be allowed or the existing PDU sessions are not allowed to be activated. However AMF cannot detect the real data rate usage of a network slice. Therefore, the data rate of the slice may be calculated as exceeding the quota even the real data rate of the slice is still lower than the quota. The slice-level MBR control in this solution is not precise, and it should not be within normative work.

For solution 20, the PCF is responsible to calculate the slice-level MBR quota value and ensure that the aggregate MBR of the SDFs of the PDU sessions of all UEs that use the slice do not exceed the Slice-MBR value, and SMF is responsible for the slice-level MBR quota limitation enforcement e.g. terminating a PDU session establishment. There is no new impact introduced in this solution, but the PCF may calculate the aggregate MBR of the SDFs of the PDU sessions in the slice as exceeding the Slice-MBR value even the actual data rate of the slice is still lower than the quota. Solution 20 is suggested to be within the normative work if the PCF takes the actual data rate usage of a network slice reported by the UPFs as the aggregate MBR of the SDFs of the PDU sessions in the slice.

# * * * * Second change * * * *

# 8	Conclusions

Editor's note:	This clause will capture conclusions from the study.

For key issue5:

It is suggested that solution 16, and solution 20 with addition that the PCF uses the actual data rate of a network slice reported by the UPFs for calculation of the aggregated MBR of the slice and ensure that it does not exceed Slice MBR, are within normative work.

# * * * * Second change * * * *