SA WG2 Meeting #142e	S2-2008792

# Nov 16th – Nov 20th, 2020 ; Elbonia

Source:	Nokia, Nokia Shanghai Bell

Title:	Conclusions for PC5 DRX operation

Document for:	Approval

Agenda Item:	8.12

Work Item / Release:	FS_eV2XARC_Ph2 / Rel-17

Abstract of the contribution: This paper proposes to change the interim conclusion to conclusion.

# 1 Introduction

As RAN2 progressed discussion and captured some agreements on SL DRX in R2-2010708 (Report from session on LTE V2X and NR V2X) at RAN2#112e (November 2020), the interim conclusion made at SA2#141E can be changed to the conclusion.

Agreements on SL DRX:

1: 	Sidelink DRX needs to support sidelink communications for both in and out of network’s coverage scenarios.

2:	RAN2 will prioritize normal use case without consideration of relay UE use case in Rel-17.

3:	Support SL DRX for all casting types.

4:	If a UE is in SL active time, UE should monitor PSCCH. FFS on PSSCH. FFS for sensing impacts.

5:	RAN2 is not going to introduce SL paging and SL PO for SL DRX.

6:	As baseline, for Sidelink DRX for SL unicast, it is proposed to inherit and use timers similar to what are used in Uu DRX. FFS for SL broadcast/groupcast. FFS on detailed timers.

7:	Working assumption: SL DRX should take PSCCH monitoring also for sensing (in addition to data reception) into account if SL DRX is used.

8:	Support of long DRX cycle for SL unicast should be assumed as a baseline. FFS on the need of short DRX cycle.

9:	Deprioritize SL WUS from RAN2 point of view in Rel-17.

2 Proposal

It is proposed to update TR 23.776-030 as follows:

* * * * First Change * * * *

7.2	Conclusions for PC5 DRX operations

For Key Issue #1 (Support of QoS aware NR PC5 power efficiency for pedestrian UEs), regarding NR PC5 DRX operations the following principles are taken as the conclusion:

-	The V2X application layer provides inputs for V2X layer to determine PC5 DRX assistance information.

Editor's note:	It is FFS whether the inputs described in TS 23.287 [5] (e.g. V2X Application Requirements) are sufficient or additional inputs (e.g. traffic pattern) are needed from the V2X application layer.

-	The V2X layer may provide the PC5 DRX assistance information to the AS layer.

Editor's note:	It is FFS whether PC5 DRX assistance information, which is provided to the AS layer by the V2X layer, is defined in 3GPP and what is the content of the PC5 DRX assistance information.

Editor's note:	It is FFS if the PC5 DRX assistance information is per mode of communication (i.e. unicast/broadcast/groupcast).

-	The Access Stratum (AS) layer determines the PC5 DRX for V2X communication over PC5 reference point to enable pedestrian UE power saving.

-	The AS layer provides the applied PC5 DRX information to the V2X layer.

-	For unicast, the PC5 DRX may be negotiated between the UEs in AS layer, pending on the feedback from RAN2.

Editor's note:	The design principles need to be evaluated by RAN WGs.

Editor's note:	It is FFS whether DRX related parameters are needed and what is the contents, which is (pre-)configured on the UE to assist the V2X layer determine the PC5 DRX assistance information.

* * * * End of Changes * * * *