3GPP TSG-WG SA2 Meeting #142 e-meeting 	S2-2008888

November 16 – 20, 2020, Electronic meeting

Source:	Nokia, Nokia Shanghai Bell

Title:	KI#6 evaluation and conclusions

Document for:	Agreement

Agenda Item: 	8.4

Work Item / Release:	FS_eNS_Ph2 / Rel-17

Abstract of the contribution: This contribution proposes evaluation and conclusions for KI#6

1	Proposal

The paper analyses solutions for KI #6 and proposes conclusions.

Start of Change

7	Overall Evaluation

7.x	Evaluation of Solutions for Key Issue #6 - Constraints on simultaneous use of the network slice

The related solutions are #26, #27, #28, #39, #40, #41, #42

| Solution #26 | Network controlled enforcement of simultaneous usage of network slices based on user preference | - it enables the UE to determine the priority for all the requested slices, or a subset of them, based on the services/applications active in the UE. - It enables the CN to determine the Allowed NSSAI even if the UE does not provide any prioritization (basically in this case there is no particular behaviour and in fact the solution does nothing compared to rel-16 – so it is unlikely to effectively enforce any policy) - it requires coordination between the UE and the network, and that the network interprets the prioritization provided by the UE. - priority may conflict with compatibility-based selection in the network, so it may be that the UE is limited to use very small set of compatible slices at a time |
| --- | --- | --- |
| Solution #27 | Network slices simultaneous usage incompatibility support | Somehow similar to solutions #28, #41, and #42 in the sense that the UE is provided configuration information wrt network slice incompatibility attribute per allowed S-NSSAI for simultaneous use. : - It is deterministic in the CN knowledge of whether the UE can support network slices simultaneous usage incompatibility feature, due to the UE explicit indication - The UE is provided explicit incompatibility information per allowed S-NSSAI. - It allows the UE to register to slices that are incompatible while using UP resources only for compatible sets of slices. The incompatible Slices, however, still need to be supported in all the TAs in the RA. - it relies on UE or network to release or deactivate PDU sessions if UE needs to establish or activate a PDU session for an incompatible slice - it does not require UE to re-register to get user plane resources for incompatible slices wrt the current slices with active PDU Sessions - it allows an option for light mode of operation where the incompatibility attribute is not provided to the UE and the incompatibility enforcement is done in the network only. (this aspect makes this solution then not friendly to supporting UEs, and also it makes it not possible for the UE to understand which PDU sessions it can use at the same time, so it is prone to several trial and error attempts when the UE uses several slices.) - the solution cannot prevent simultaneous registration to incompatible slices (the attribute sent to the UE is used only by SM) and therefore it may not make it possible to select specialized AMFs for a small set of compatible slices, thus it would defeat the purpose of deploying these. - The incompatibility for simultaneous use attribute is a subscription information, so it is based on HPLMN policies - The "light mode", if applied to supporting UEs, it misses out on the benefit of avoiding PDU session set up, but it may send to the UE a new cause code. All in all, there is no motivation to use this for Supporting UEs - The "light mode", if applied to non-supporting UEs, the incompatibility enforcement is done by the network only and an existing reject cause is returned to the UE. This may result in a non-predictable behaviour end to end. - This departs from the assumption that the PDU sessions related to network slices in the allowed NSSAI can all be used at the same time. |
| Solution #28 | Compatibility Class based support for simultaneous use of the network slice | Somehow similar to solutions #27, #41, and #42 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. It proposes that the network provides to the UE for each S-NSSAI a "simultaneous use information" compatibility class defining the compatibility with other Network Slices, and that the UE shall not include in the Requested NSSAI S-NSSAIs that map to HPLMN S-NSSAIs which are incompatible with each other - The UDM provides the AMF with compatibility class info if any applies to a S-NSSAI in subscription, in other words, The incompatibility for simultaneous use attribute is a subscription information, so it is based on HPLMN policies.. - This is provided to the UE together with the Configured NSSAI - it allows the UE to be aware of the Compatibility Class information during the UE registration, so it avoids registering with incompatible Network Slices. - If a VPLMN does not support the feature, only compatible slices can be sent as subscription data to that VPLMN. So, it has no impact on existing networks. - If the UE does not support the feature, the Configured NSSAI the AMF sends only includes compatible slices, based on HPLMN decision (they are highlighted in subscription data as the compatible set to be provided in this case) - It prevents the UE to attempt to register with incompatible slices upfront as the UE is aware of what slices are compatible according to the policy it receives from the network |
| Solution #39 | Support registration for incompatible network slice(s) in UE's current serving RA or different one | The solution does not address all the scenarios. - It imposes minimal impact to the UE, by adding a cause code indicating a rejected s-NSSAI is rejected due to compatibility with Network Slices in the allowed NSSAI - It does not differentiate the Rejected S-NSSAIs which are outside of the UE’s current serving TA/RA. |
| Solution #40 | Separate SUPI/GPSI per isolated set of S-NSSAIs | This creative solution enables different “UE profiles” associated to different SUPIs/GPSIs and that identify specific set of S-NSSAIs that are compatible. - It assumes roaming agreements ensures that compatibility between S-NSSAIs is coordinated. - It does not require UE to declare what it supports to the network nor requires any coordination between VPLMN and HPLMN for upgrades since it is backward compatible with existing procedures and mechanism for allowed/configured/subscribed slices. What is not understood is just ignored. - it has NAS impacts in terms of new registration procedure and new NAS mechanisms internal to the UE (e.g. switch between SUPIs) - it seems to have USIM impact as the UE is said to register with the SUPI of each independent slice group. |
| Solution #41 | Simultaneous use of the network slice via Configured NSSAI | The solution is similar to #27, #28, and #42 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. - It can inform the UE of all the deployment and business constraints related to the serving network's network slices (based on network configuration, SLA with HPLMN and possibly dynamic information). - It requires UE to be aware of the SUG (Simultaneous Usage Group) information during the UE registration. - the supporting UE is not expected to register at the same time for incompatible slices - as since Rel-15, it requires UE to re-register to get data resources for incompatible slices wrt the current slices with active PDU Sessions - the HPLMN constraints are not part of subscription data, so every change of policy needs roaming agreement update. - It addresses VPLMN constraints that are already supported by the current system and it is unclear how this is related to GSMA NG.116 attribute that is related to HPLMN only |
| Solution #42 | UE handling of constraints on simultaneous use of network slices based on network assistance | The solution is similar to #27, #28, and #41 in the sense that the UE is provided configuration information wrt the sets of slices that are compatible. - It requires UE to be aware of the category information during the UE registration. Furthermore, this solution proposes multi-level network slice compatibility. - the UE can never register at the same time for incompatible slices - it requires UE to re-register to get data resources for incompatible slices wrt the current slices with active PDU Sessions |

Next Change

8.6	Conclusion for Key Issue #6

Editor's note:	The KI conclusion will be made based on GSMA feedback.

End of Changes