3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009018r01

Elbonia, November 16 – 20, 2020	(revision of S2-xxxxx)

Source:	Nokia, Nokia Shanghai Bell

Title:	KI #2: Solution #55 Evaluation and conclusion

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This paper provides evaluation and conclusion of solution #55

- Proposal

It is proposed to include the following in the TR 23.748.

* * * * Start of First change * * * *

6.55.4	Evaluation

Soluiton#55 in TR 23.748 v1.1.0 addresses scenarios where Edge Application Server(s) and Application Function are located at the Edge Data Network, thus implying multiple AF instances i.e. one (or more) local AF(s) and also a central AF. Application Function here acts as control plane entity managing one or more EAS and responsible to interact with 5GC. In such deployment scenarios: as part of initial PDU session establishment, a central AF may be involved initially, however, due to relocation to Edge DNAI and corresponding EAS relocation, a new local AF serving the Edge Applications may be selected.

To address multiple AF requirements, certain aspects were addressed as part of URLLC work item in Release-16, however during EC study phase certain additional aspects are discussed in TR 23.748 v1.0.0 solution#55. For example, updates to procedure on AF requests to influence traffic routing, indication to SMF of AF migration to a target AF and AF relocation information transfer between AFs. For the enhancement of AF request influence procedure, there are two alternatives, and one is the target AF triggers AF influence procedure based on the context received from source AF, the other is the source AF triggers the AF influence procedure and transfer the UP management information to the target AF.

Additionally, other solutions in the TR also considers or assume multiple AFs deployment, for example solution#28 added note: It is assumed to find out AF based on the pre-configuration or the information received through step 4 and step 5, in case of multiple AFs. Solution #55 overcomes the issue of this pre-configuration mentioned in solution#28.

Solution#55 does not discuss any application Context transfer details, which is assumed to be out of SA2 scope and may be discussed other places e.g. in SA6.

* * * * Start of second Change * * * *

9. 	Conclusions

9.2.6	Conclusions for Key Issue #2: Other sub-issues for edge relocation

-	Solution #26 are not recommended in normative work.

-	Whether AF context transferring can be done via NEF is to be decided during normative phase after SA WG6 solution is stable.

-  Solution #55 is applicable to multiple AF scenarios and provides updates to procedure on AF request to influence traffic routing for scenarios where EAS relocation also results in corresponding AF relocation. Solution #55 is therefore recommended for normative work.

* * * * End of Change * * * *