3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009020r09

Elbonia, November 16 – 20, 2020	merging S2-2008444, 8650, 8651, 8652 ,8798,8905, 8617, 8625 and 8963

Source:	Orange

Title:	KI #11: Update of conclusion

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract of the contribution: This contribution updates the evaluation and conclusions on KI#11 “Increasing efficiency of data collection”.

# Discussion

This contribution updates the evaluation and conclusions for Key issue #11 “Increasing efficiency of data collection”

The four aspects of KI#11 are divided into corresponding sections.

For each section we provide the following updates.

| a | Signalling reduction via architectural changes | Sol #1, #9, #10 (case considering DCCF), #12, #15, #16, #35, #39. | Summary of solutions and evaluation |
| --- | --- | --- | --- |
| b | Signalling reduction via parametrization and services changes | #58, #70, #72. | Summary of solutions and evaluation Conclusion |
| c | Signalling reduction on Tracking and Discovery of Entities | #10, #32, #36, #69. | Summary of solutions and evaluation |
| d | Signalling reduction via parametrization and services changes specific to Event Exposure framework | #33, #34, #37, #38, #71. | Summary of solutions Conclusion |

# 2.	Proposal

It is proposed to apply the following changes to the TR 23.700-91.

* * * * Start of changes * * * *

## 8.11	Key Issue #11: Increasing efficiency of data collection

### 8.11.1 Signalling reduction via parametrization and services changes

For service changes the following principles are proposed:

- Make use of bulk data collection an option for data collection from NWDAF.

.

-	Persistent data collection:

-	Based on the NWDAF event subscription, UDM determines and recreates the event subscription with the new NF to which the UE is attached.

### 8.11.2 Signalling reduction on Tracking and Discovery of Entities

For tracking and discovery of entities, the following principles are proposed:

-	Improve the NF profile stored in the NRF, as well as NRF services with supplementary information in order to facilitate the discovery of relevant NFs, in the case of data collection for any UE.

-	Improve AMF event exposure with supplementary information in order to facilitate the discovery of available network slice information relevant in an area of interest, in case of data collection for any UE.

-	Improve SMF event exposure with supplementary information in order to facilitate the discovery of relevant PDU sessions, in case of data collection for any UE.

NOTE 1: The last Event exposure is dependent on having location available in SMF. UE Location parameter is still to be optional but may be switched on per operator configuration.

-	 Make use of UDM as a means to store relevant per UE information to find serving NFs .

-	As having the possibility to mute the event reporting from NWDAF also introduce for Analytics the notion of maximum duration for the storage of analytics in an NWDAF in conjunction with appropriate parameters at NWDAF service interface (e.g. time when analytics are needed, suppression of notifications).

### 8.11.3 Signalling reduction via Event Exposure service enhancement

For Event Exposure enhancement, the following principles are agreed for normative work.

Event Exposure for Analytics service:

- Additional Analytics filters shall be standardized on a per-analytics basis, in order to narrow down the scope of analytics, thus reducing signalling both for analytics provision and data collection.

- Additional parameters on Analytics service shall be standardized to reduce the amount of information in Analytics reports: time granularity, criteria for ordering of results (together with maximum number or results), aggregation for some analytics (e.g. load of an NF set).

Event Exposure for data collection:

- In case of permanent data collection (during model training or for other specific use cases), additional mechanisms shall be standardized in order to limit signalling while still enabling a relevant access to collected data and general trends. Therefore, the mute storage of events in the DCCF/messaging framework, the distributed NWDAF, or NFs  (i.e. without event reporting) can be performed during periods where no analytics are requested, within a limited size. Because of the limited size of the data collection queue, basic metrics may be performed as a complement on Event-IDs and collected information, in order to gather historical trends.

- Additional parameters on Event Exposure service shall be standardized to reduce the amount of information per Event Report, e.g. value granularity.

- Because a better fairness in sampling contributes to reduce the size of samples, hence reducing signalling load, additional parameters may be added for this purpose, only static criteria is to be progressed. A Static criteria is a criteria where a UE belongs to the same criteria as long as it is served by the NF

* * * * End of changes * * * *