# 3GPP TSG-SA WG2 Meeting #142E e-meeting	S2-2009029r01

Elbonia, Nov 16 – 20, 2020	(revision of S2-200xxxx)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-07: KI#4: Update on conclusions for component #2 (CP based provisioning)

Document for: 	Approval

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This contribution proposes to update the conclusions for KI#4 component #2 when Control Plane is used for provisioning.

# 1 Discussion

One of the goals defined KI#4 description is to study solutions how the UE NPN credentials and other information to enable UE to get 3GPP connectivity are provisioned.

NPN credentials are defined as information that the UE uses for authentication to access an NPN. NPN credentials may be 3GPP credentials or non-3GPP credentials.

In addition, KI#4 description notes that it covers devices with and without a UICC (NOTE3).

It is essential to adequately protect the NPN credentials to be provisioned on the UE in all cases in scope of the KI#4, that is for NPN credentials on devices with and without a UICC.

For User Plane solution, the protection of NPN credentials is part of the provisioning protocol and supported regardless of the type of the secure hardware component (see TS 33.501 clause 5.2.4).

For Control Plane solution, the provisioning is proposed to be based on the UPU procedure. However, as per above TS 33.501 UPU requires ME to forward secured packet explicitly to the USIM.

Conclusion: Current UPU based solution does not cover devices without UICC, thus we propose to update KI#4 conclusion for the CP based solution and add a corresponding Editor’s Note.

# 2 Proposal

It is proposed to update TR 23.700-07 as follows:

***************** Start of Change ****************

## 8.4	Key Issue #4: UE onboarding and remote provisioning

Editor's note:	These are *INTERIM* conclusions for Key issue #4.

### 8.4.1	Conclusions for SNPN case

UE onboarding for SNPN (Component 1 of KI#4)

-	It should be possible to support a registration procedure that enables support for UE onboarding using Default UE credentials and with an O-SNPN as the Onboarding Network (ON).

Editor's note:	In order to support UE onboarding using Default UE credentials and O-SNPN as the Onboarding Network (ON) the distribution of security functions when primary authentication is used should be decided by SA WG3, e.g. whether and how to support the primary authentication based on default credential in case DCS is deployed or not.

-	It should be possible that one SNPN can take the role of both Onboarding Network (ON) and SO (Subscription Owner), and it should be possible that the ON and SO are different SNPNs i.e. O-SNPN and SO-SNPN.

Editor's note:	DCS is potentially introduced to authenticate a UE with default UE credentials or provide means to another entity to do it. There are two potential mechanisms for DCS to authenticate the UE. 1) DCS interacts with O-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is not involved in the authentication procedure. As a result, the SO-SNPN is not directly involved with the authentication procedure but gets informed of its result and then performs remote provisioning. 2) DCS interacts with SO-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is involved in the authentication procedure. As a result, the SO-SNPN is directly involved and aware of the result of authentication procedure and performs remote provisioning. SA WG3 needs to evaluate the two above mechanisms from security perspective and provide feedback.

Editor's note:	The decision on whether primary authentication is required during initial access to the O-SNPN is dependent on SA WG3 feedback; until this feedback is received, it is assumed that such authentication is required.

-	The SNPN, which directly interacts with DCS, may determine the corresponding DCS identity or address/domain based on the input from the UE or when primary authentication is used based on information (e.g. DCS address) locally provisioned at SNPN.

Editor's note:	SA WG3 should provide feedback on whether the UEs permanent identifier (SUPI or SUCI) may be used for finding the DCS identity or address/domain that can authenticate the UE, as well their security properties.

-	The DCS can be an entity external to the 5GC of O-SNPN.

-	The NG-RAN of the Onboarding network includes information in the SIB so that the UE can discover and select an appropriate O-SNPN. The UE may or may not be pre-configured with O-SNPN network selection information (e.g. O-SNPN network identifiers).

-	Upon registration to an SNPN for Onboarding, the UE provides the information at RRC level which indicates the registration is for onboarding. This information will be specified only for SNPN and allows NG-RAN to select an appropriate AMF that supports onboarding procedures.

Editor's note:	Handling of RAN-level congestion is FFS.

-	Upon registration to an SNPN for Onboarding, the UE provides the information at NAS level that the registration request is for onboarding to allow AMF to, e.g., select an appropriate SMF and perform other onboarding-related configuration.

-	Using PLMN credentials for UE onboarding and PLMN as Onboarding Network (ON) is already possible.

-	Onboarding network should support functionality to restrict usage to only on-boarding service.

-	When Onboarding network is O-SNPN, the information required to restrict the usage to only onboarding service is locally configured in the AMF, and the AMF restricts the usage when the UE indicates that the registration is for Onboarding (e.g., onboarding registration type) or NG-RAN indicates that the access is for Onboarding.

-	When Onboarding network is a PLMN, the functionality to restrict usage is activated for the UE by AMF based on received operator profile from the UDM.

Editor's note:	Whether there is a need to extend subscription information to support a restriction to only on-boarding service is FFS.

-	The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration after successful completion of onboarding or based on timer configured for on-boarding service.

Remote provisioning for SNPN credentials (Component 2 of KI#4)

-	Usage of a PLMN as Onboarding Network for a UE equipped with a USIM shall be possible. The SO-SNPN credentials can be transmitted to UE via UP connectivity. The UE shall be configured with Default credentials in USIM to register with a PLMN where the UE can register with the Default credentials in order to communicate with the provisioning server;

-	When User Plane is used for provisioning of SO-SNPN credentials, the User plane remote provisioning protocol used and how the UE downloads the SO-SNPN credential from the Provisioning Server (PS) after PDU session establishment in the O-SNPN is out of scope of SA WG2;

-	Control Plane remote provisioning based on UE Parameters Update Procedure as defined in TS 23.502 [6] can be used for provisioning of credentials and other information to enable access to SO-SNPN. After Registration Complete, while onboarding is still in progress, UPU is imminent, thus, the NAS signalling connection shall not be released. It shall be possible that the SO-SNPN credentials being provisioned are not accessible (e.g. by using an additional credential in the UE) by the onboarding network;

NOTE 1:	The UE may support the Control Plane remote provisioning or the User Plane remote provisioning.

Editor's note:	SA WG3 feedback will need to be taken into account for including of the CP based provisioning.

Editor's note:	When the PS is a stand-alone entity, how the PS is selected is for FFS.

Editor's note:	For remote provisioning via CP, when the PS is a stand-alone entity the role of PS (as UDM of SNPN or AF with respect to O-SNPN) is FFS.

-	Control Plane remote provisioning procedure assumes a Provisioning Server that communicates with the 5GC using 3GPP-defined protocols.

-	For the provisioning of IMSI accompanied by AKA credentials, GSMA RSP is used, Provisioning Server (PS) can provision the credential to UE over User Plane (UP) connectivity;

-	For the provisioning of Non-3GPP credentials, the credentials can be provided to UE over UP connectivity;

-	It shall be possible to pre-configure the Provisioning Server (PS) address, SO-SNPN identity on the UE, and it shall also be possible that the O-SNPN provides the PS address to the UE after successful authentication and authorization. The PS address from the O-SNPN shall be integrity protected. The PS address provided by the network is prioritized, if configured and overrides any PS address stored in the UE. Configuration of PS address to the UE can be supported using one of the following methods:

a.	SMF may deliver onboarding configuration data as part of extended Protocol Configuration Options (PCO) in PDU Session Establishment Response to UE. This is similar to use of PCO to configure Autoconfiguration server for UE in Wireless and Wireline Convergence (TR 23.716 [28] clause 6.10).

b.	Alternatively, onboarding configuration data may be configured in the UE during Registration Procedure.

c.	In addition, onboarding configuration data may be configured in the UE using service specific policies subject to UE capabilities similar to what is used for V2X communications as specified in TS 23.287 [29] clause 5.1.1 for ways how parameters may be made available to the UE and TS 23.287 [29] clause 6.2.5 for AF-based service parameter provisioning and TS 24.587 [30] clause 5.2.4 for configuration parameters such as validity timer, server address and geographical area.

-	It is assumed that the UDM (or AAA) of SO-SNPN is provisioned with UE credentials/subscription data when remote provisioning is successfully performed. Existing mechanism for provisioning of UDM/UDR can be reused.

-	When User Plane is used for provisioning of SO-SNPN credentials, a restricted PDU session is supported to be dedicated for the remote provisioning. Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address, the device uses a well-known FQDN to perform PS discovery.

NOTE 2:	SA WG3 may evaluate these mechanisms and provide guidance on appropriateness of use for SNPNs.

Editor's note:	SA WG3 to determine how Control Plane based provisioning using UE Parameters Update (UPU) procedure and User Plane provisioning can support devices without UICC i.e. how to secure the provisioned credentials between the PS and an endpoint in ME.

***************** End of Change ****************