# 3GPP TSG-SA WG2 Meeting #142E e-meeting	S2-2009031

Elbonia, Nov 16 – 20, 2020	(revision of S2-200xxxx)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	23.700-07: KI#4: Update on conclusions for component #1 (DCS selection)

Document for: 	Approval

Agenda Item: 	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: This contribution proposes to update the conclusions for KI#4 component #1 related to DCS selection.

# 1 Discussion

In KI#4 the concept of using a Default Credential Server (DCS) hosted by an entity external to the onboarding network (O-SNPN) has been introduced. The entity operating DCS is called Default Credential Owner (DCO).

In KI#1 of the enhancement to support SNPNs along with credentials owned by a separate entity are studied.

Observation Onboarding with the help of a DCO can be regarded as a special case of KI#1, if the DCO is regarded as the “separate entity”. Therefore, the same architectural principles as used in KI#1 should be used for the DCO integration as well. The external entity could be a Subscription owner or a Credential owner.

DCO acting as Subscription Owner

For the case of a DCO providing authentication services to onboarding networks, the concept of a Subscription Owner is preferred. That is the DCO has a network level identifier which can either be an SNPN Id or a SNPN realm or both. Since a DCO typically does not operate an own NG-RAN the DCO might only own a DCO realm.

During manufacturing the DCO installs an onboarding SUPI and cryptographical material (i.e. default credentials) on the device. The default credentials can be used by the device to execute mutual authentication with the DCS. If the DCS does not offer a 5G Nausf interface, the DCO should deploy a 5G AUSF and an NRF.

Proposal The DCO acts as a Subscription Owner. The device stores onboarding SUPI and default credentials issued by the DCO and generates the onboarding SUCI from the onboarding SUPI.

The discovery and selection of the DCO’s AUSF and the access token acquisition can be executed using the mechanism of the 5G Service Based Architecture (SBA), which includes:

- During the Registration Request the onboarding device sets the Home Network Identifier of the onboarding SUCI to the DCO’s network level identifier derived from the onboarding SUPI.

- The AMF uses NRF of the O-SNPN to discover the DCO’s AUSF providing the content of the Home Network identifier as query parameter. The O-SNPN's NRF invokes the DCO’s NRF by existing means to resolve the request.

- The AMF uses NRF of the O-SNPN to retrieve an access token for DCO’s AUSF using the O-SNPN's and the DCO’s network level identifiers as query parameters. The O-SNPN's NRF invokes the DCO’s NRF by existing means to resolve the request.

Proposal Service Discovery of 5G NFs in DCO network shall be based on the existing SBA mechanisms.

5G NF Deployment Options

One deployment option could be that a DCO is only running a DCS, which does not expose 5G SBIs.

The main difference between the two deployment variants (i.e., DCO runs only DCS or also 5G NF’s such as AUSF, NRF) is related to the interfaces between DCO and O-SNPN that need to be supported. If the DCO exposes 5G NFs, the interface towards the DCS is an internal DCO interface. Otherwise, the interface to the DCS is an external one (e.g. a AAA interface) and requires extra integration effort between DCO and O-SNPN.

Proposal: The DCO should deploy relevant 5G NFs (AUSF, NRF) and expose corresponding interfaces to the O-SNPN.

# 2 Proposal

It is proposed to update TR 23.700-07 as follows.

************ Start of 1st Change ************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for next generation new services and markets".

[3]	3GPP TS 22.263: " Service requirements for Video, Imaging and Audio for Professional Applications (VIAPA)".

[4]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[5]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[6]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[7]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[8]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[9]	Internet Assigned Numbers Authority (IANA): "Private Enterprise Numbers"; https://www.iana.org/assignments/enterprise-numbers/enterprise-numbers (retrieved March 26, 2020).

[10]	3GPP TS 24.502: "Access to the 3GPP 5G System (5GS) via non-3GPP access networks; Stage 3".

[11]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[12]	3GPP TS 33.203: "3G Security; Access Security for IP-based services".

[13]	3GPP TS 23.632: "User Data Interworking, Coexistence and Migration; stage 2".

[14]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[15]	3GPP TS 23.003: "Numbering, addressing and identification".

[16]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[17]	3GPP TS 22.228: "Service requirements for the Internet Protocol (IP) multimedia core network subsystem (IMS); Stage 1".

[18]	3GPP TS 22.101: "Service aspects; Service principles".

[19]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[20]	SP-191038: "IMS emergency support for SNPN" (IESNPN).

[21]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security".

[22]	GSMA SGP.01: "Embedded SIM Remote Provisioning Architecture", Version 4.0.

[23]	GSMA SGP.02: "Remote Provisioning Architecture for Embedded UICC Technical Specification", Version 4.0.

[24]	GSMA SGP.21: "eSIM Architecture Specification", Version 2.2.

[25]	GSMA SGP.22: "eSIM Technical Specification", Version 2.2.1.

[26]	3GPP TS 31.115: "Secured packet structure for (Universal) Subscriber Identity Module (U)SIM Toolkit applications.

[27]	3GPP TS 31.111: "Universal Subscriber Identity Module (USIM), Application Toolkit (USAT)".

[28]	3GPP TR 23.716: "Study on the Wireless and Wireline Convergence for the 5G system architecture".

[29]	3GPP TS 23.287: "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services".

[30]	3GPP TS 24.587: "Vehicle-to-Everything (V2X) services in 5G System (5GS); Stage 3".

[31]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[32]	IETF draft: "Remote Attestation Procedures Architecture" ( draft-ietf-rats-architecture-04).

[33]	IETF draft: "The Entity Attestation Token (EAT)" (draft-ietf-rats-eat-03).

[34]	MulteFire Alliance (MFA), Architecture for Neutral Host Network Access Mode Stage 2 (Release 1.1), MFA TS MF.202, https://www.multefire.org/, Release 1.1.

[35]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[36]	3GPP TS 23.973: "User data interworking, coexistence and migration".

[37]	3GPP TS 29.544: "5G System; Secured Packet Application Function (SP-AF) services; Stage 3".

[38]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3".

[39]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[40]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[x]	3GPP TR 33.857: "Study on enhanced security support for Non-Public Networks (NPN)".

************ End of 1st Change ************

************ Start of 2nd Change ************

## 8.4	Key Issue #4: UE onboarding and remote provisioning

Editor's note:	These are *INTERIM* conclusions for Key issue #4.

### 8.4.1	Conclusions for SNPN case

UE onboarding for SNPN (Component 1 of KI#4)

-	It should be possible to support a registration procedure that enables support for UE onboarding using Default UE credentials and with an O-SNPN as the Onboarding Network (ON).

Editor's note:	In order to support UE onboarding using Default UE credentials and O-SNPN as the Onboarding Network (ON) the distribution of security functions when primary authentication is used should be decided by SA WG3, e.g. whether and how to support the primary authentication based on default credential in case DCS is deployed or not.

-	It should be possible that one SNPN can take the role of both Onboarding Network (ON) and SO (Subscription Owner), and it should be possible that the ON and SO are different SNPNs i.e. O-SNPN and SO-SNPN.

Editor's note:	DCS is potentially introduced to authenticate a UE with default UE credentials or provide means to another entity to do it. There are two potential mechanisms for DCS to authenticate the UE. 1) DCS interacts with O-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is not involved in the authentication procedure. As a result, the SO-SNPN is not directly involved with the authentication procedure but gets informed of its result and then performs remote provisioning. 2) DCS interacts with SO-SNPN and Network Function in SO-SNPN (Subscription Owner SNPN) is involved in the authentication procedure. As a result, the SO-SNPN is directly involved and aware of the result of authentication procedure and performs remote provisioning. SA WG3 needs to evaluate the two above mechanisms from security perspective and provide feedback.

Editor's note:	The decision on whether primary authentication is required during initial access to the O-SNPN is dependent on SA WG3 feedback; until this feedback is received, it is assumed that such authentication is required.

-	For onboarding registration, the following principles are applied:

-	Upon registration for Onboarding, the UE provides its Default Device Identifier (DDI) encoded in SUPI/SUCI format.

NOTE: The DDI is in the format of Network Access Identifier (NAI) i.e., username@realm. Further details on the DDI will be defined by CT4.

-	O-SNPN uses the information contained in the DDI encoded in SUPI/SUCI format to route the authentication request towards the DCS.

-	The device sends the onboarding SUCI (generated from the onboarding SUPI) in the Registration request for onboarding to the AMF.

-	AMF discovers and selects AUSF in the DCS domain based on existing mechanisms for AUSF discovery and selection as described in TS 23.501 [4] clause 6.3.4.

-	If in certain deployment scenarios AUSF deployed in the O-SNPN interacts directly with the DCS, the same mechanisms for AUSF discovery and selection as described in TS 23.501 [4] clause 6.3.4 are used.

-	If in other deployment scenarios AAA Server is seen as DCS, SUPI/SUCI can also be used for routing to an external AAA-S as described in TR 33.857 [x].

-	The NG-RAN of the Onboarding network includes information in the SIB so that the UE can discover and select an appropriate O-SNPN. The UE may or may not be pre-configured with O-SNPN network selection information (e.g. O-SNPN network identifiers).

-	Upon registration to an SNPN for Onboarding, the UE provides the information at RRC level which indicates the registration is for onboarding. This information will be specified only for SNPN and allows NG-RAN to select an appropriate AMF that supports onboarding procedures.

Editor’s note:	Handling of RAN-level congestion is FFS.

-	Upon registration to an SNPN for Onboarding, the UE provides the information at NAS level that the registration request is for onboarding to allow AMF to, e.g., select an appropriate SMF and perform other onboarding-related configuration.

-	Using PLMN credentials for UE onboarding and PLMN as Onboarding Network (ON) is already possible.

-	Onboarding network should support functionality to restrict usage to only on-boarding service.

-	When Onboarding network is O-SNPN, the information required to restrict the usage to only onboarding service is locally configured in the AMF, and the AMF restricts the usage when the UE indicates that the registration is for Onboarding (e.g., onboarding registration type) or NG-RAN indicates that the access is for Onboarding.

-	When Onboarding network is a PLMN, the functionality to restrict usage is activated for the UE by AMF based on received operator profile from the UDM.

Editor’s note:	Whether there is a need to extend subscription information to support a restriction to only on-boarding service is FFS.

-	The UE shall initiate de-registration from the on-boarding network after finishing the remote provisioning or the on-boarding network shall initiate the de-registration after successful completion of onboarding or based on timer configured for on-boarding service.

************ End of 2nd Change ************