SA WG2 Meeting #142-E	S2-2009049

16 - 20 Nov 2020, Electronic	(revision of S2-2007738r10)

Source:	Intel

Title:	KI#1 conclusion for UEs with SNPN subscription

Document for:	Approval

Agenda Item:	8.2

Work Item / Release:	FS_eNPN / Rel-17

Abstract of the contribution: Builds on top of the technically endorsed S2-2007738r10 by adding architecture figures.

# 1	Introduction

This contribution is based on the technically endorsed S2-2007738r10, with the following changes:

-	It adds architecture figures on top of the endorsed text.

-	The separate entity-controlled prioritized list of preferred SNPNs and Group IDs is proposed to be merged in a single list.

-	Service continuity for services of the separate entity when the UE moves between two SNPNs (this is based on presumed SA1 LS reply to the SA2 LS on service continuity (S2-2007828).

# 2	Proposal

It is proposed to agree the text proposal below for inclusion in TR 23.700-07.

*** Start of changes ***

### 8.1.X	Conclusions for scenario where the SNPN offers connectivity for UE(s) with credentials owned by separate entity offering AUSF and UDM

It is recommended to enhance Nudm, Nausf, Namf and Nsmf services to support the following procedures for a UE in an SNPN, when the AMF and SMF belong to the SNPN, and the UDM and AUSF belongs to a separate entity owning the credentials:

-	Registration procedures, and

-	UE Configuration Update procedure for access and mobility management related parameters.

### 8.1.Y Conclusions on SNPN selection for UEs with SNPN subscription

-	The following enhancement are recommended to be performed on SIB, for SNPN only:

-	Indication that "access using credentials from a separate entity is supported"

-	Optionally, supported Group IDs (GIDs)

-	Optionally, an indication whether the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN

Editor's note:	Need for additional SIB information is FFS.

NOTE 1:	This refers to UEs that do not have an entry in the user-controlled or the separate entity-controlled lists of preferred SNPNs or Group IDs matching the SNPN's network identity and any of the Group IDs supported by the SNPN.

NOTE 2:	The number of supported Group IDs that can be broadcast will be determined by RAN2.

-	UE configuration

-	User-controlled prioritized list of preferred SNPNs

-	Separate entity controlled prioritized list of preferred SNPNs or Group IDs

NOTE 4:	If the UE has multiple subscriptions owned by different entities separate from the SNPN it is assumed that the subscription to use is selected by implementation specific means (e.g. based on user input) prior to network selection.

-	For a UE configured to use SNPNs, automatic SNPN selection is performed in the following order:

-	UE selects and attempts to register with the available SNPN identified by a PLMN ID and NID for which the UE has SUPI and credentials (i.e. as in Rel-16).

-	UE selects an available and allowable SNPN, which broadcasts "access using credentials from a separate entity is supported" indication and an SNPN ID contained in the user-controlled list (if available)

-	UE selects an available and allowable SNPN which broadcasts "access using credentials from a separate entity is supported" indication and an SNPN ID or Group ID contained in the separate entity-controlled list (if available)

-	If the UE has not found an available and allowable SNPN based on the above, the UE selects an available and allowable SNPN which broadcasts an indication that the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN:

-	If the UE detects a plurality of such SNPNs, the order in which the UE selects and attempts a registration with an SNPN is implementations specific

-	If the UE is unable to find a suitable cell of any allowable and available SNPN based on the above, the UE attempts to camp on an acceptable cell of any available SNPN supporting emergency calls (irrespective of SNPN ID), or of any available PLMN (irrespective of PLMN ID), in limited service state.

-	The UE continues automatic SNPN selection based on the above until a suitable cell is found or until the user stops the automatic selection procedure (in which case the user may proceed with manual network selection).

NOTE 5:	The details of how to prevent a UE from automatically repeatedly selecting an SNPN that rejects the UE’s registration (i.e. handling of permanently and temporarily forbidden SNPNs) can be defined by CT1.

-	For manual SNPN selection the UE presents all available SNPNs, which broadcast the "access using credentials from a separate entity is supported" indication.

NOTE 6:	The order in which available SNPNs are presented can be defined by CT1.