3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009055

Elbonia, November 16 – 20, 2020, 2020 	(revision of S2-2008742)

Source:	Huawei, HiSilicon, Lenovo, Motorola Mobility, Broadcom

Title:	Update Evaluation of steering modes and Conclusion on KI#1

Document for:	Approval

Agenda Item:	8.6

Work Item / Release:	ATSSS / Rel-17

Abstract: This paper is to update the evaluation and conclusion part for the steering mode KI#1.

- Introduction/Discussion

This paper is to update the evaluation and conclusion part for the steering mode KI#1, and remove the following EN.

Editor’s Note: It is FFS whether new steering mode(s) need to be defined in Rel-17 that support the following list of performance criteria.

2. Text Proposal

It is proposed to capture the following changes in the TR 23.700-93.

# * * * * First change * * * *

7.1	Evaluation for KI#1: Steering Modes

General:

The following steering modes are proposed in Rel-17 for addressing KI#1:

-

-	Autonomous Steering Mode (Solution #2).

-	Autonomous Steering Mode with Advanced PMF (Solution #3).

-	Redundant Steering Mode (Solution #4).

-	RTT Difference based Steering Mode (Solution #11).

-	UE Assisted Traffic Steering Mode (Solution #12).

They can be divided into four categories depending on what they are trying to achieve:

A.	Enable more flexibility in UE/UPF to select best traffic distribution (Sol #2, 3) in order to maximize the bandwidth/throughput.

B.	Take additional performance parameters into account, such as packet loss rate of the link vs the maximum packet loss rate for the service, the RTT differences and other thresholds etc as defined in Sol #2, 3, 11 and the steering modes suggested in Sol #1.

C.	Add packet duplication (bi-casting) as a way to reduce impacts of packet loss (Sol #4).

D.	Add a possibility for the UE to influence the steering mode and/or steering mode parameters such as the weight (Sol #12).

Category A, B proposals:

These type of solutions aims to address some limitations where each rel-16 steering mode is focusing too much on one specific aspect (e.g. fixed load-balancing weight or lowest RTT).

The additional performance parameters of the new steering modes described for Categories B can also be applied to the existing Rel-16 steering modes where thresholds can be added e.g. for maximum packet loss and packet delay budget and then, providing a room of freedom for scheduler implementations. That would allow each steering mode covers a larger set of behaviours, instead of being limited to only consider a single characteristic (e.g. RTT).

Solutions 2/3:

There are three main aspects proposed in solutions 2/3. They are analysed below:

1)	Autonomous steering by UE and UPF in order to maximize the bandwidth/throughput:

-	Such autonomous steering mode (in order to maximize the bandwidth/throughput) provides flexibility to both the UE and the UPF to maximize the resource usage of the two accesses considering the situation of the link and UE/UPF status, so that the user can get best service experience / throughput, which cannot be reached by Rel-16 steering modes which focus on a single characteristic (e.g. RTT or weight) and fixed splitting rate.

To be more specific, even with the extension of thresholds on the existing R16 steering mode, where the thresholds define the conditions on the access availability, the UE and UPF shall follow the R16 steering modes in most cases (i.e. normally both accesses are available according to the threshold value). For example, if the R16 load-balancing weight factors are set to fixed rates 50% and 50% per each access for UL, and in the case the transport bandwidth is different between 3GPP and non3GPP, e.g. 50Mbps over 3GPP, and 100Mbps over non3GPP, the UE cannot fully take advantage of 100Mbps via non 3GPP access as not satisfying the fixed 50% and 50% weight factors requirement.

2)	Thresholds:

Thresholds provided to the UPF may be a useful addition as proposed in Solution #2/#3:

-	Sol#3 suggest that PCF provides thresholds in PCC rules and that thresholds are set per IP Flow and per access.

-	There is no explicit relation between the thresholds described in Sol#3 and the QoS requirements for an SDF/application but the PCF when setting the threshold needs to consider the QoS requirements for an SDF/application. Thresholds parameter defined in Sol#3 are independent parameters per access, but can also be defined to be access independent, considering the QoS requirements are independent of access type. Sol#3 also proposes jitter thresholds which can be configured based on the application type. It should be noted however that currently there are no jitter requirements in the 5G QoS framework. It is also proposed to introduce loss rate thresholds for non-GBR traffic. Although for now the Maximum Packet Loss Rate value is currently only applied to GBR, it doesn't mean the packet loss impact can be ignored for the non-GBR traffic. Obviously, the more packets are lost, the worse service quality will be, not only for GBR but also for non-GBR.

3)	Enhanced PMF measurements:

-	The benefits of performing RTT measurements per QoS flow improves the precision of RTT measurements and has small impacts to the PMF protocol.

-	Jitter measurements reuses RTT measurement message for Jitter calculation. Some services such as game, IMS voice are sensitive to jitter, therefore it is possible that PCF can configure such threshold. It should be noted however that currently there are no jitter requirements in the 5G QoS framework.

-	Packet loss rate measurements may be useful in case loss rate thresholds are introduced. In case a PMF echo packet is lost, the UE or the UPF can resend PMF request or carry the lost information in the subsequent PMF message. Details can be left to stage 3.

Solution 11:

Solution #11 proposes a new steering mode to take RTT difference into account. The functionality of RTT difference based steering mode can however be achieved through proper implementation of other steering modes by UE/UPF implementation simply taking the RTT difference into account when making traffic switching/splitting decisions.

Category C proposal:

Solution 4:

The Solution states that it is targeting "loss rate sensitive traffic, such as IMS signalling, video, and some TCP-based traffic". This redundant transport is only initiated when one access cannot satisfy the loss rate so it could be useful, which is much better than retransmission after packet loss. For QUIC or TCP based applications, the value of redundant transmission is not clear since those applications anyway rely on retransmissions by the transport layer. However, in the case of QUIC with unreliable transmission where retransmission mechanism is not supported, the packet loss can still be a problem and should be reduced as much as possible. Taking video as an example, the increase in the packet loss rate may cause the video quality deteriorated.

Category D proposal:

Solution 12 (UE-assisted traffic steering mode) proposes to let the UE influence the steering mode and/or steering mode parameters. This may be based e.g. on UE battery consumption and battery level. Getting such feedback from the UE may be valuable to achieve a good user experience for ATSSS, but it needs to be ensured that operator control is maintained. The network controls whether or not the UE is allowed to use this mode/operation. Whether additional network control is needed while the UE is operating in the UE assisted steering mode (e.g., support for the PCF to set boundaries of the steering decisions by UE and UPF) needs further study.

# * * * * Next change * * * *

8.1	Conclusions for KI#1: Steering Modes

The following new steering modes shall be specified..

1)	An Autonomous steering mode, which shall be based on the following principles:

-	If authorized by the PCF, the UE and the UPF may autonomously determine how to steer, switch or, if allowed, split traffic over each access for UL and DL directions, respectively, subject to steering method and other criteria such as link status, access conditions and UE states (e.g., power saving).

-	The UE steers/switches/splits UL traffic by taking into account UE's preference on traffic split ratio or preferred access for UL transmission.

-	For the DL, the UE may request the UPF that the DL transmission matches the UL transmission traffic distribution. The UPF will take the UE's request into account when deciding the DL transmission traffic distribution.

-	When the network selects the Autonomous steering mode, the network may also provide a network-preferred steering mode. For example, the network may provide Active/Standby mode as a network-preferred steering mode, to indicate to UE that Active/Standby should be applied when the UE does not want to autonomously route the traffic.

NOTE 1:	How the UE requests the UPF to match DL transmission with UL transmission can be supported with MPTCP and MPQUIC is to be determined during normative phase.

This autonomous steering mode is applicable to all steering functionalities, including the steering functionalities defined in Rel-16

2)	A Redundancy steering mode, which shall be based on the following principles:

-	If authorized by the PCF, the UE and UPF shall transport every packet on both accesses in order to maximize the transmission reliability.

NOTE 2:	How the redundant packets are identified by the receiver will be determined during the normative phase.

This steering mode is applicable to all steering functionalities, including the steering functionalities defined in Rel-16, except ATSSS-LL.

In addition, the following steering modes enhancements shall be specified.

1)	It shall be possible to associate a steering mode with one or more conditions.

-	A condition with a steering mode indicates whether a measured parameter is above or below a threshold.

-	A steering mode shall be applied only when its associated condition is true, i.e. a measured parameter is below a threshold.

-	The UE and the UPF will switch the traffic to the other access for UL and DL directions, respectively, in case its associated condition is not true, i.e. a measured parameter is above a threshold.

2)	The thresholds can be based on the QoS parameters applied in the existing QoS model for meeting the QoS requirements of the applications/SDFs. Such thresholds may include (a) the Maximum RTT (derived from PDB) and (b) the Maximum Packet Loss Rate (derived from the Maximum Packet Loss Rate (MPLR) or the PER), (c) Jitter. The thresholds will be the same for both 3GPP and non-3GPP accesses since QoS requirements are per SDF/service.

Furthermore, the PMF is enhanced to support RTT, packet lost and jitter measurements per QoS Flow, as defined in subclause 6.3.2.1, for the steering modes where PMF need to be applied.

# * * * * End of changes * * * *