3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009059r03

Elbonia, November 16 – 20, 2020	(revision of S2-xxxxx)

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	KI #2: Solutions Conclusion

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This paper proposes conclusion to KI#2 solutions that has strong L4 assumptions

# Introduction

Solutions 30, 33, 38, and 40 assumes L4/TCP/TLS/QUIC context transfer (between EAS). Some of these underlying assumptions as required in these solutions are un-realistic as it requires touching the baseline layers below the application. Such context transfer between old EAS and new EAS may require, for example: synchronizing the TCP states such as sequence numbering, session context mirroring, window size, security contexts possibly including Initialization vector changing on packet per packet basis and so on. Additionally, there is no corresponding IETF or other specifications and thus it is not clear how to achieve such context transfer and if this is feasible and/or too complicated to implement and doable, especially when the Edge Environment is hosted by a third party, hence when the MNO and 3GPP do not control the underlying layers in the EAS.

Some of the above mentioned underlying assumptions required to make these respective solutions work, are highlighted below:

Solution 30: “AF can manage the runtime context mirroring between Local EAS and Anchor EAS and between source Local EAS and target Local EAS in order to guarantee the service continuity. The runtime context mirroring between source EAS and target EAS remains during the period of EAS migration (from EAS migration preparation to migration completion) to prevent UL/DL packet loss, which means during the transition period, the source Local EAS continues to serve the UE, synchronization of such knowledge between source and target Local EAS is assumed to be supported by the Edge Hosting Environment.”

Solution 33: This has assumptions such as: the application context including L4 network context can be transferred between the EAS(es). Application context relocation from EAS#1 to EAS#2 is supported. The application context relocation includes the upper layer network context (e.g. TCP context or high layer context).

Solution 34: ”If the UE receives the Local DN binding context control information, the UE forwards the control information to the High Layer OS within the UE to handle the control information properly. For example, if the control information includes Layer 4 context refresh indication, the UE removes the Layer 4 context (e.g. the TCP context) that was established with the local DN1”: this basically means that the solution assumes the 3GPP layer in the UE will remove a TCP context in the OS. Unlike these other solutions, solution 34 however does not require these L4/TCP/TLS/QUIC context transfer between EAS(s) i.e. here the requirement is only to UE internal implementation, if provided with corresponding indication by 5GC.

Solution 38: “NOTE 1: This solution assumes the UE related contexts, including application layer contexts and transporting layer contexts (e.g. TCP), if any, can be transferred from old EAS to new EAS by EAS relocation mechanisms, to ensure the packets can be handled seamlessly by the new EAS

Solution 40: Seamless change of Edge Application Sever for stateful applications by caching application status information in NEF. NEF retrieves stateful application status information from S-AF. The S-AF retrieves application status information from S-EAS. NEF sends the application status information cached from S-AF, to configure in T-AF. The status information may include: UE GPSI, UE IP address @ S-UPF, application ID, application address @ S-EAS, application context.

# Proposal

It is proposed to include the following in the TR 23.748.

* * * * Start of Changes * * * *

9. 	Conclusions

## 9.2	Conclusions for Key Issue #2

### 9.2.1	Conclusions for Key Issue #2: Reducing packet loss during EAS relocation

Buffering uplink packets in the target PSA until receiving the indication of successful EAS relocation as proposed in Solution #27 is recommended for normative phase.

### 9.2.2	Conclusions for Key Issue #2: UE DNS cache

For UE DNS cache related issue, it is concluded:

-	For session breakout case, the SMF may provide DNS cache clear indication within NAS message to UE as described in Solution #32 and #34 (only DNS cache related part). Based on the received DNS cache clear indication, the UE cleans the related DNS cache, which do not impact the application client.

Editor's note:	It is FFS when SMF sends the DNS cache clear indication to the UE, i.e. before the EAS relocation or after EAS relocation.

Editor's note:	It is FFS whether the SMF send the associated information with DNS cache clear to the UE so the UE can clear a part of DNS cache or all DNS cache.

NOTE 1:	If connectivity is available to the source EAS, then the application can select a new EAS after the DNS caching is flushed by the applications or cache timer expires.

NOTE 2:	If the UE does not support DNS cache clear triggered by 5GC, the application will continue to be served by the old EAS until the UE Application Layer DNS cache timer expires or the applications determines to flush the DNS cache.

NOTE 3:	This DNS cache clear indication does not impact the UE Application Layer DNS caching.

-	For SSC mode 2/3 case, the UE can removes the cached DNS records and can reselect a new EAS after it is allocated with a new IP address.

### 9.2.3	Conclusions for Key Issue #2: AF based EAS rediscovery

The AF may reselect a new EAS for UE due to UP path change notification or by its internal trigger, e.g. load balance. When the new EAS is reselected, the UE is informed with the new EAS address via the application layer signalling, which is out of scope of 3GPP.

Sending EAS IP address by 5GS to UE is not recommended to be specified in normative work.

### 9.2.4	Conclusions for Key Issue #2: Edge relocation considering user plane latency

Regarding the issues for edge relocation considering the user plane latency requirements, the solution #35 is recommended as baselines for the normative phase.

### 9.2.5	Conclusions for Key Issue #2: EAS IP address replacement in 5GC

To solve the issue of change of EAS with EAS IP address change with or without PSA change, for the UE unawareness solution, it is concluded that no solution can be recommended.

### 9.2.6	Conclusions for Key Issue #2: Other sub-issues for edge relocation

-	Solution #26 are not recommended in normative work.

-	Whether AF context transferring can be done via NEF is to be decided during normative phase after SA WG6 solution is stable.

## 9.3	Conclusions for Key Issue #5

The SMF uses Nsmf_PDUSession_SMContextStatusNotify operation to send the target DNAI(s) to the AMF to assistant the I-SMF/SMF selection. The AMF then uses the target DNAI(s) for I-SMF/SMF selection for existing or subsequent PDU Session. Clause 6.12.2.2 in Solutions #12 and solution#50 are recommended into normative with below considerations:

-	Normative work for KI5 shall only impact N11 (AMF-SMF interface) and N16a (to relay information). No PCF impact and no N4 impact are to be considered.

-	There is no I-SMF chaining induced by these solutions.

-	SMF decides whether it is required to send the target DNAI to the AMF for triggering I-SMF (re)selection.

-	AMF may retrieve from NRF, the SMF profiles that indicates supported DNAI. , and considers the last DNAI received from the SMF to decide the proper I-SMF/SMF. The AMF does not need to have policies related to potential conflicts if a UE would need to be simultaneously served by different DNAI(s).

-	Nsmf_PDUSession_SMContextStatusNotify operation is used by normative extensions due to these solutions. The notification is used to inform AMF: DNAI for the current PDU Session for I-SMF selection (situation of sol 50)/ DNAI for the subsequent PDU Session for SMF selection  (situation of solution 12).

* * * * Next Change * * * *

9.x	Conclusions for KI#2

Solutions 30, 33, 38, and 40 assume L4/TCP/TLS/QUIC context transfer between edge application servers. Some of these underlying assumptions as required in these solutions are un-realistic as it requires touching the baseline layers below the application. Such context transfer between old EAS and new EAS may require, for example: synchronizing the TCP states such as sequence numbering, session context mirroring, window size, security contexts possibly including Initialization vector changing on packet per packet basis and so on. Additionally, there is no corresponding IETF or other specifications and thus it is not clear how to achieve such context transfer and whether this is feasible and/or too complicated to implement and doable, especially when the Edge Environment is hosted by a third party, hence when the MNO and 3GPP do not control the underlying layers in the EAS. Therefore, solutions 30, 33, 38, and 40 are not recommended to be specified in normative work.

* * * * End of Change * * * *