3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009059r03

Elbonia, November 16 – 20, 2020	(revision of S2-xxxxx)

Source:	Nokia, Nokia Shanghai Bell, Ericsson

Title:	KI #2: Solutions Conclusion

Document for:	Discussion/Approval

Agenda Item:	8.3

Work Item / Release:	FS_enh_EC / Rel-17

Abstract: This paper proposes conclusion to KI#2 solutions that has strong L4 assumptions

# Introduction

# Proposal

It is proposed to include the following in the TR 23.748.

* * * * Start of Changes * * * *

### 9.2.5	Conclusions for Key Issue #2: EAS IP address replacement in 5GC

To solve the issue of change of EAS with EAS IP address change with or without PSA change, for the UE unawareness solution, it is concluded that no solution can be recommended in this release considering L4 context migration is not mature yet.

### 9.2.6	Conclusions for Key Issue #2: Other sub-issues for edge relocation

-	Solution #26 are not recommended in normative work.

-	Whether AF context transferring can be done via NEF is to be decided during normative phase after SA WG6 solution is stable.

Solutions 30, 33, 38 (parts of forwarding tunnel and ender marker), and 40 assume L4/TCP/TLS/QUIC context transfer between edge application servers. Some of these underlying assumptions as required in these solutions are considered as immature as it requires touching the baseline layers below the application. Such context transfer between old EAS and new EAS may require, for example: synchronizing the TCP states such as sequence numbering security contexts possibly including Initialization vector changing on packet per packet basis and so on. It is not clear whether achieving such context transfer is feasible  based on existing implementation, especially when the Edge Environment is hosted by a third party, hence when the MNO and 3GPP do not control the underlying layers in the EAS. Therefore, solutions 30, 33, 38, and 40 are not recommended to be specified in normative work in this study.

* * * * End of Change * * * *