SA WG2 Meeting #142E	S2-2009070

November 16 - 20, 2020, Elbonia	(was S2-200xxxx)

# Source:	Nokia, Nokia Shanghai Bell

# Title:	Cover Sheet of TR 23.700-20 for presentation to TSG SA for information

# Document for:	Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT/ Rel-17

# Presentation of Specification to TSG

# Presentation to:	TSG SA Meeting #90

Document for presentation:	TR 23.700: "Study on Enhanced support of Industrial Internet of Things (IIoT) (Release 17)", Version 1.3.0

Presented for:	Information

# Abstract of document:

The document is the outcome of FS_IIoT study. The study would enable enhanced support of IEEE TSN, also enhancements for Time Sensitive Communication to support deterministic applications.

Following Key Issues were prioritized:

- KI#1: Uplink Time Synchronization

- KI#2: UE-UE TSC communication

- KI#3A: Exposure of QoS and related enhancements

- KI#3B: Exposure of Time Synchronization and support for PTP time sync

- KI#5: Use of Survival Time for Deterministic Applications in 5GS

# Changes since last presentation to SA:

Solutions have been updated, open items have been resolved and conclusions for all key issues have also been enhanced.

The TR comprises of conclusions for all Key issues targeted for Rel-17. The TR has reached 95% completion status.

Outstanding Issues:

Resolution of 5 Editor’s notes regarding the following items:

- Key Issue #3A Exposure of deterministic QoS: Need for Burst Spread from AF.

- Key Issue #3A Exposure of deterministic QoS: Need for Jitter measurement and reporting to AF.

- Key Issue #3B Exposure of TSC Services: Need to include validity time for AF time sync request.

- Dependency on SA1 input for H&F (KI#3A)

- Dependency on RAN2 for Communication service availability (KI#5).

Items 4) and 5) can be resolved during normative work.

# Contentious Issues:

- Key Issue #3A Exposure of deterministic QoS

  - Need for Burst Spread from AF.

  - Need for Jitter measurement and reporting to AF

- Key Issue #3B Exposure of TSC Services.

  - Need to include validity time for AF time sync request.