SA WG2 Meeting #142E	S2-2009081r03

Nov 16 – 20, 2020

Source:	Nokia, Nokia Shanghai Bell, Ericsson, ZTE, Samsung

Title:	KI #5: conclusion update

Document for:	Discussion/Approval

Agenda Item:	8.5

Work Item / Release:	FS_IIoT / Rel-17

Abstract of the contribution: This contribution gives the update of conclusion for KI#5.

1		Discussion

Regarding the following two survival time defintions from SMF to NG-RAN via NGAP, definition i) has the semantics of survival time with less ambiguity to be used over NGAP.

- as a maximum time in units of “time” where each unit corresponds to the data burst periodicity defined in TSCAI in Rel-16; or

- as a maximum number of consecutive data burst transmission failures, where a data burst corresponds to a single application message

Proposal 1: Survival time is defined over NGAP as a maximum time in units of “time” where each unit corresponds to the data burst periodicity defined in TSCAI.

Regarding support of different communication service availability (CSA) target associated with the same survival time, it is understood that the reliability related parameters (e.g. PER) in 5QI associated with the QoS flow can be configured accordingly and used in NG-RAN to meet the corresponding CSA target. For instance, for the QoS flows with the same survival time and different CSA, the reliability related parameter in 5QI associated with the QoS flow can be configured differently according to the corresponding CSA. In this way, NG-RAN may apply the different operation (e.g. different weight or priority for scheduling, different link adaptation target etc.) to meet the survival ime target and reliability target that corresponds to CSA. Similar ideas were also discussed in RAN2 Tdocs [1] [2]. Thus, it is proposed not to have CSA as another parameter over NGAP to NG-RAN.

Proposal 2: Survival time is sufficient and CSA is not needed over NGAP to NG-RAN.

[1] R2-2008985	RAN Enhancements to Support New QoS Parameters for TSN; Intel Corporation

[2] R2-2009179	Concept and use of survival timer; Samsung

2		Proposal

The following change is proposed for TR 23.700-20.

# * * * * First change * * * *

8.4	Key Issue #5: Use of Survival Time for Deterministic Applications in 5GS

Principles for the normative work:

-	Survival Time is transferred as part of the TSCAI parameter but the TSCAI may not always comprise of Survival time.

-	Survival Time information is specified by the AF in units of "time" with respect to burst periodicity or as the maximum number of consecutive message transmission failures (i.e. whose loss can be tolerated). Note that there is a single message per burst periodicity and the burst contains the application message. It is conveyed together with TSCAI Periodicity parameter (the time between periodic TSC bursts) and burst size (e.g. MDBV).

If the Survival Time information  is specified by the AF in units of "time" with respect to TSN working domain burst periodicity, the Survival Time information needs to be mapped to the 5GS time domain by the SMF based on latest cumulative rateRatio between the TSN time and 5G time.

When survival time information is provided for a TSN stream, then it should not be aggregated with other TSN streams into a single QoS flow, or if they are aggregated, then the survival time parameter shall not be provided as part of TSCAI.

-	Survival Time information  is included in the TSC Assistance Container and delivered to PCF in an AF request by NEF or TSN AF.

-	The PCF provides the Survival Time information to SMF in the TSC Assistance container.

-	The SMF determines Survival Time and sends it to the NG-RAN as part of TSCAI without requiring AN or N1 specific signalling exchange with the UE.  If Survival Time information is the maximum number of consecutive message transmission failures, SMF translates the maximum number of consecutive message transmission failures to “time” unit based on TSCAI periodicity parameter and determines Survival time.

-	Survival time specified in unit of “time”  will be supported over NGAP.