SA WG2 Meeting #142-E		S2-2009082

Online, 16 - 20 November, 2020	      	(revision of xxx)

Source:	Sandvine

Title:	KI #7, Evaluation, conclusion

Document for:	Approval.

Agenda Item:	8.1

Work Item / Release:	FS_eNA_ph2 / Rel-17

Abstract: This contribution proposes conclusions and some updates to the evaluation for KI#7.  Add a new procedure to clarify how the TopN parameter is used when PCF is a service consumer.

# 1. Discussion

Only one solution i.e. Solution #61 is captured for the Key Issue #7: Adding Application attributes and KPIs as the Input data in some services described in TS 23.288 [5].

Solution #61 covers enhancement for two Analytics-ID, QoS Sustainability and Observed Service Experience Analytics.

Enhacement of QoS Sustainability Analytics ID evaluation

For QoS Sustainability the enhacement introduces the TopTen concept and add Application-ID and their KPIs in the data imput. In the Rel-16 version of this Analytics-ID, in case of an Outcome results as a prediction indicating that the QoS is not going to be sustained; the consumer does not have any clue of what to do; therefore, for critical use cases such as self-driving vehicles, this analytics ID is not indicated due to the lack of alternatives in case of a negative outcome.

For Rel-17, the enhancement introduces the Top-Ten concept; in case of a negative Outcome, the Service consumer receives the top-ten Application-IDs that are causing the issue of QoS sustainability and can react in real-time to request for the PCF actions to manage the traffic that is impacting the QoS sustainability. In the case of the PCF being the service consumer, the reaction is right forward performed with minimum disturbance in the QoS and permanently assuring the SLAs for critical services. In this scenario, the AF interacts (directly or through the NEF) with the PCF as described in TS 23.503 clause 5.3.1 Interactions between PCF and AF, in this case the PCF is the service consumer of the QoS sustainability Analytic ID. There is a need to define a procedure to clarify how this Analytics output is used in particular for the PCF that is explicitly mentioned as consumer.

Enhacement of Observed Service Experience Analytics ID evaluation

For Observed Service Experience the enhacement introduces the TopN concept as a parameter in the Analytic Filter and add Application-ID and their KPIs that are present in the traffic as additional data imput. In the Rel-16 version of this Analytics-ID, it focuses only on Application IDs served by AFs that could interact with the NEF and NWDAF. As a reference in a typical 4G network in 2020, Application IDs served by AF are less than 5% of the total Application-IDs present in the traffic. To have more context, it is possible to find in the network 3000 different Application-ID served from the internet.

As a conclusion is stated in 23.700-91 clause 8.4, ensure that slice SLA is guaranteed and leverage the Analytics Id on Observed Service Experience and Slice Load. Enhancement of Observed Service experience for this purpose is critical; otherwise, the usages of this Analytics ID will use less than 5% of the application-IDs present in the traffic to assure that slice SLA is guaranteed; therefore, the system would not offer SLA assurance for traffic served from the Internet.

For Rel-17, the enhancement introduces the TopN concept; with this concept, the service consumer can target the Top N Application-IDs that are experiencing the poorest service experience among applications served from the Internet and Applications served by local AFs. This mechanism is essential to find straight forward the Applications experiencing QoE issues. In the Rel-16 version of this Analytics-ID, this mechanism is not present; how to target the right applications that are experiencing QoE issues among 3000 Applications when internet traffic is considered, it is an important challenge.

This clause mentions also solutions different than Solution #61 that uses or proposes enhancement to the Observed Service Experience Analytics ID no related to the solution #61.

## 2. Text Proposal

It is proposed the following changes to TR 23.700-91.

# * * * * First change * * * *

### 6.61.4	Procedures improvement

They are performed changes in the clause 6.9.4, TS 23.288 [5], Procedures Figure 6.9.4-1 and steps descriptions. The purpose of this change is to include the part where the NWDAF receives Data available at the UPF. It is also added the follow note

NOTE:	How NWDAF receives data available at UPFs is not defined in this TS.

The following new step is added to TS 23.288 [5] Figure 6.9.4-1 and respective description.

Figure 6.61.4-1: "QoS Sustainability" analytics provided by NWDAF

1.	The consumer requests or subscribes to analytics information on "QoS Sustainability" provided by NWDAF. The parameters included in the request are described in clause 6.9.1, TS 23.288 [5].

The consumer may include multiple sets of parameters in order to provide different combinations of "Location information" and "Analytics target period" when requesting QoS Sustainability analytics for monitoring for a path of interest.

2.	NWDAF receives Data available at the UPF.

3.	The NWDAF collects the data specified in clause 6.9.2, TS 23.288 [5] from the OAM, following the procedure captured in clause 6.2.3.2, TS 23.288 [5].

4.	The NWDAF verifies whether the triggering conditions are met and derives the requested analytics. The NWDAF can detect the need for notification based on comparing the requested analytics of the target 5QI against the Reporting Threshold(s) provided by consumer in any cell over the requested Analytics target period.

5.	The NWDAF provides response or notification on "QoS Sustainability" to the consumer.

NOTE:	How NWDAF receives data available at UPFs is not defined in this specs.

Procedure to clarify how QoS sustainability Analytics output is used when an outcome prediction is negative and the QoS is no become sustained. In this particular case, the PCF is the service consumer.

Figure 6.61.4-2: "QoS Sustainability" PCF as service consumer with Outcome negative

This procedure is similar to the procedure in figure 6.61.4-1, with the following differences.

Step 0.a. AF-PCF Interaction with Ncp_PoliceAuthorization AF session with required QoS as described in 23.503 clause 6.1.3.22

Step 0.b Npcf_EventExposure_Subscribe with QoS targets can no longer be fulfilled. This subscription allow the AF to get aware in case QoS is not going to be guarantee.

Step 6. Policy Decision. PCC Rule installation to detect and report Application start/stop for those TOPTEN Application that belongs to background traffic

The following takes place when any SMF Notify to the PCF the Start detection of an application based on subscription in step 6

Step 7. Based on subscription on Step 6. Nsmf Application ID Start is detected by UPF-SMF. Application ID which belongs to background traffic category,

Step 8. PCF initiated SM Policy Association Modification

Based on Step 7 outcome, PCF update policies to manage QoS assuring the QoS Sustainability for the Targeted UE. Previously it was predicted QoS Sustainability will be compromised due to one or more of reported TOPTEN Application-IDs

The following takes place when NWDAF Notify to the PCF with positive Outcome based on subscription in step 1

Step 9. Nnwdaf_AnalyticsInfo_Response Notify returning QoS Sustainability with TOPTEN Application the most impacting the throughput. With Positive outcome prediction QoS Sustainability will become compliant with SLA

Step 10. PCF Remove PCC Rule to detect and report Application start/stop of background traffic

# * * * * End of the change* * * *

# * * * * Next change * * * *

# 7	Overall Evaluation

Editor's note:	This clause will provide evaluation of different solutions.

# 7.7	Key Issue #7 Adding Application attributes and KPIs as the Input data in some services described in TS 23.288 [5]

This clause evaluates some aspects of solution#61 as regards to:

Two Analytics-ID are covered in this solution:

- QoS Sustainability

- Observed Service Experience.

This clause mentions also solutions different than Solution #61 that uses or proposes enhancement to the Observed Service Experience Analytics ID no related to the solution #61.

Enhacement of QoS Sustainability Analytics ID

For QoS Sustainability the enhacement introduces the TopTen Application information and add Application-ID and their KPIs in the data imput. In the Rel-16 version of this Analytics-ID, in case of an Outcome results as a prediction indicating that the QoS is not going to be sustained; the consumer does not have any clue of what to do; therefore, for critical use cases such as self-driving vehicles, this analytics ID is not indicated due to the lack of alternatives in case of a negative outcome.

For Rel-17, the enhancement introduces the Top-Ten concept; in case of a negative Outcome, the Service consumer receives the top-ten Application-IDs that are causing the issue of QoS sustainability and can react in real-time to request for the PCF actions to manage the traffic that is impacting the QoS sustainability. In the case of the PCF being the service consumer, the reaction is right forward performed with minimum disturbance in the QoS and permanently assuring the SLAs for critical services. In this scenario, the AF interacts (directly or through the NEF) with the PCF as described in TS 23.503 clause 5.3.1 Interactions between PCF and AF, in this case the PCF is the service consumer of the QoS sustainability Analytic ID.

Enhacement of Observed Service Experience Analytics ID

There are two kind of enhancement in this solution for this Analytics ID, the first enhancement aims that the observed Service experience includes Application IDs that are present in the traffic mix and are not necessarily linked to the AFs, also improving statistics reporting. The second enhacement introduces the TopN concept as a parameter in the Analytic Filter and add Application-ID and their KPIs that are present in the traffic as additional data imput. In the Rel-16 version of this Analytics-ID, it focuses only on Application IDs served by AFs that could interact with the NEF and NWDAF. As a reference in a typical 4G network in 2020, Application IDs served by AF are less than 5% of the total Application-IDs present in the traffic. To have more context, it is possible to find in the network 3000 different Application-ID served from the internet.

As a conclusion is stated in 23.700-91 clause 8.4, ensure that slice SLA is guaranteed and leverage the Analytics Id on Observed Service Experience and Slice Load. Enhancement of Observed Service experience for this purpose is critical; otherwise, the usages of this Analytics ID will use less than 5% of the application-IDs present in the traffic to assure that slice SLA is guaranteed; therefore, the system would not offer SLA assurance for traffic served from the Internet.

For Rel-17, the enhancement introduces the TopN concept; with this concept, the service consumer can target the Top N Application-IDs that are experiencing the poorest service experience among applications served from the Internet and Applications served by local AFs. This mechanism is essential to find straight forward the Applications experiencing QoE issues. In the Rel-16 version of this Analytics-ID, this mechanism is not present; how to target the right applications that are experiencing QoE issues among 3000 Applications when internet traffic is considered, it is an important challenge.

# * * * * End of the change* * * *

# * * * * Next change * * * *

## 8.x	Key Issue #7 Conclusion: Adding Application attributes and KPIs as the Input data in some services described in TS 23.288 [5]

Solution #61 with Enhacement of QoS Sustainability Analytics ID introduces a mechanism to guarantee the QoS for any UE on one Application-ID therefore ensure the SLA for one specific Application-ID. With Enhacement of Observed Service Experience Analytics ID Solution #61 introduces a mechanism that target multiple Application-IDs that are experiencing QoE issues; therefore ensure the SLA for multiples Application-IDs. Both mechanism are complementary, applies for different scenarios and requirements.

The Solution #61 shall progress to the normative work and shall include the Analytics ID QoS sustainability and Service Observed Experience with both enhancements, includes Application IDs that are present in the traffic mix and are not necessarily linked to the AFs, and the TopN concept as a parameter in the Analytic Filter

The following procedures shall be progressed to normative work:

For QoS sustainability procedures describe in clause 6.61.4 figure 6.61.4-1 enhancement of the procedure and figure 6.61.4-2 "QoS Sustainability" PCF as service consumer (described above in this contribution)

For Observed Service experience procedures described in clause 6.61.7.3 figure 6.61.7.3-1, 6.61.7.3-2 and figure 6.61.7.3-3 (described in S2-2008382)

# * * * * End of the change* * * *