3GPP TSG-WG SA2 Meeting #142E e-meeting 	S2-2009086r01

Elbonia, November 16 – 20, 2020, 2020	(revision of S2-200xxxx)

Source:	Nokia, Nokia Shanghai-Bell

Title:	KI#1: Conclusion update

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	FS_5MBS / Rel-17

Abstract: This contribution updates solution 33 with details how to handle HTTP streaming.

# Introduction

At the last SA2 meetings concerns were raised that the MBSF should also handle the ingestion of content using HTTP streaming and that this is not considered in solution 33. It was also found desirable not to speak about stage 3 protocols but rather about the stage 2 basis for the work

# 2. Text Proposal

It is proposed to agree the following changes for TR 23.757.

## 6.33	Solution #33: Defining interactions between MBSF-C and MBSF-U based on N4 session management

### 6.33.1	Functional Description

#### 6.33.1.1	System Architecture

Baseline architectures Architecture 1 contains an MSF-U controlled by an MSF-C, and architecture 2 contains a MBSU controlled by an MBSF for similar tasks; the MBSF is integrated into the NEF. The converged architecture also contains an MBSF-U controlled by an MBSF_C. It is expected that proposed solution can be applied to baseline architecture 1, baseline architecture 2, and the converged architecture.

; The terms CP function and UP function will be used in what follows. A CP function that could be MSF-C, NEF, or MBSF-C, and a UP function that could be a MSF-U or MBSF-U

Editor's note:	It is FFS if N4 interface as specified in TS 29.244 [17] is suitable for the interaction between MSF-C and MSF-U. Collaboration with SA WG4 is needed.

#### 6.33.1.1	High Level Proposal

The procedures between UP function and CP function will be an based on extension on N4 session management parameters in subclause 5.8.2.11 of TS 23.501 [2] . N4 session management is used successfully in the 3GPP ecosystem and provides capabilities e.g. for packet filtering, forwarding, repacketization of PDUs (i.e. manipulation of transport layer), event notifications and billing. It lacks media manipulation capabilities (for transcoding, FEC and ROHC) and will require related extensions; solution 33 contains proposals for such extensions

This has the advantage that N4 session management is well known in the 3GPP ecosystem and that implementations collocating the UPF (that is already controlled using N4 session management) and the MBSU/MSF-U to minimize the user plane processing resources could also become possible in that manner.

#### 6.33.1.2	Requirements

The UP function will need to support transcoding, Forward Error Correction (FEC), and Robust Header Corrections (ROHC). It will also need to support receiving data via HTTP streaming

For FEC, two outgoing streams are created for one incoming stream: a main stream and a repair stream. If several content streams are received, they will be transmitted as several outgoing streams, but a single FEC repair stream can be send for all those content streams.

For ROHC, several outgoing content or FEC repair streams may be combined in one ROHC stream.

For HTTP streaming, an HTTP requests needs to be sent by the UPF before obtaining content in HTTP replies.

For Transcoding:

-	it may be necessary to receive an incoming RTCP stream with feedback for an outgoing RTP stream and adjust the transcoder accordingly

-	it may be necessary to send an outgoing RTCP stream with feedback derived by the transcoding for an incoming RTP stream.

Figure 6.33.1.2-1: Incoming ang outgoing data streams at the UP Function

#### 6.33.1.3	N4 session management extensions

It is suggested that the CP function supplies newly introduced "Packet Processing Rules" (PPR), extended Packet Detection Rules (PDR), and Forwarding action rules (FAR) to the UP Function as part of a PFCP session establishment or modification

The UP Function processes and modifies incoming media according to the received instructions in the PPR:

-	It transcodes the media as requested by the CPF.

-	It applies Robust header compression to outgoing media when requested by the CPF.

-	It applies Forward Error Correction to outgoing media and generates an associated outgoing FEC repair stream when requested by the CPF.

-	It monitors the incoming media stream and generates an associated RTCP feedback stream when requested by the CPF.

-	It receives an incoming RTCP feedback stream and may adjust the media processing based on the RTCP feedback when requested by the CPF.

-	It provides Event notifications about media processing related Events to the CPF when requested by the CPF.

-	For HTTP streaming, it generates HTTP requests to obtain streamed HTTP contents.

A PPR contains:

-	An PPR identifier.

-	Information about the outgoing media format (e.g. CODEC, FEC, ROHC, and possible parameters for the outgoing media format:

-	Type (e.g. CODEC, FEC, ROHC).

-	Codec specific parameter, e.g. for AMR parameters as defined in clause 8.1 of IETF RFC 4867 [19].

-	Transport, e.g. RTP profile.

-	For ROHC (see IETF RFC 5795 [20] and IETF RFC 3095 [21]): Either profile, target periodicity for full header packets, MAX_CID parameter.

-	For FEC framework configuration information (see clause 5.5 of IETF RFC 6363 [22]).

-	opional FAR ID for the outgoing media stream.

-	optional subsequent PPR ID.

-	Optional information about the incoming media format:

-	Type (e.g. CODEC, FEC, ROHC).

-	Codec or type specific parameter(s), e.g. for AMR parameters as defined in clause 8.1 of IETF RFC 4867 [19].

-	Transport, e.g. RTP profile, HTTP streaming.

-	Optional information about associated outgoing media stream:

-	An FAR identifier and/or an PPR ID.

-	Type (e.g. RTCP feedback for incoming media format, FEC repair flow, ROHC flow, HTTP request).

-	Optional media format or type specific parameter(s).

-	Optional information about associated incoming stream:

-	Type (e.g. RTCP feedback for outgoing media format, ROHC feedback).

-	Optional media format specific parameter.

-	Events to be reported to CP function, e.g.:

-	Unexpected Input media.

-	Resource shortage.

-	For FEC and ROHC: Session establishment success or failure.

The Packet detection rule (PDR) is used as follows:

-	An optional IE to contain PPR ID to be applied for traffic matching the PDR. Several PPR IDs may be provided and all related PPR shall then be applied in consecution, in the order the PPR IDS are provided.

-	An indication of the usage of the traffic within the PPR (e.g. associated stream, RTCP, ROHC, media), can be omitted for media usage.

-	The existing FAR ID in the PDR applies for the main output stream of the PPR.

-	If the PDR contains an associated PPR ID or indicates that the traffic is not to be used for media (but e.g for RTCP) within the PDR, it does not contain a FAR ID.

Figure 6.33.1.2-1: Description of packet processing in the UPF using Packet Detection Rules (PDRs), Packet Processing Rules (PPRs), and Forwarding Action Rules (FARs)

### 6.33.2	Procedures

Figure 6.33.2-1: session management interactions to configure UP Function packet processing

1.	CPF sends PFCP Session Establishment Request and includes PDRs (with PPR ID, and possible PPR usage information), PPRs (with information about incoming and outgoing encoding and a possible application of FEC ROHC,or HTTP streaming, about associated incoming streams, and about associated outgoing streams with FAR IDs, and media processing related Events to be reported), and FARs).

2.	UP Function is configured accordingly.

3.	UP Function may send an HTTP request for HTTP streaming

4.	UP Function receives an incoming packet.

5.	UP Function processes the incoming packet:

-	It searches a matching PDR.

-	It select PPR(s) according to PPR ID(s) in PDR.

-	It transcodes and applies ROHC and/or FEC according to PPR(s).

-	If sends outgoing packets as described by FARs.

6.	UP Function sends outgoing packet.

7.	UP Function generates RTCP feedback based on PPR instructions and sends it in backward direction.

8.	UP Function may report media processing related events to CPF based on CPF subscriptions.

9.	UP Function receives an incoming RTCP packet in backward direction.

10.	UP Function processes the incoming RTCP packet:

-	It searches a matching PDR.

-	It select PPR(s) according to PPR ID(s) in PDR and forwards the stream as RTCP stream to PPR based on usage information in PDR.

-	It may adjust the media processing base4d on the received feedback.

To enable that several media streams are obtaining a single outgoing FEC repair stream, several media streams point to the same PPR for FEC. The pointer may be the PPR ID in the PDR of each media stream. A PPR for FEC may thus have several input media streams and will then produce an output media stream for each input media stream (and send this stream to the FAR ID in the corresponding PDR), and in addition an FEC repair stream (to be sent to the FAR ID for the outgoing media streams. The PPPR for FEC can be invoked after other PPRs for transcoding.

Figure 6.33.2-2: Description of packet processing in the UP Function when several media streams are obtaining a single outgoing FEC repair stream

To enable that several media streams are grouped together in a single ROHC flow, several media streams point to the same PPR for ROHC. The pointer may be the PPR ID in the PDR of each media stream and the PDR will then not contain a FAR ID. A PPR for FEC may thus have several input media streams and will then produce only a single ROHC stream (to be sent to the FAR ID for the outgoing media stream in the ROHC PPR). The PPR for FEC can be invoked after other PPRs for transcoding or FEC.

Figure 6.33.2-3: Description of packet processing in the UP Function when several outgoing media streams are encapsulated together in a single outgoing ROHC flow

### 6.33.3	Impacts on services, entities and interfaces

MBSF-C:

-	Support N4 session management extended by Packet Processing Rules.

MBSF-U:

-	Support N4 session management extended by Packet Processing Rules.

-	Support media processing such as transcoding, FEC and ROHC.

-	Support media reception using HTTP streaming