SA WG2 Meeting #S2-143E	S2-210xxxx

Feb. 24 – March 9, 2021, Elbonia

Source:	Interdigital Inc., LG Electronics

Title:	TS 23.304: Clause 6.1.1.2 Control Plane UE-UE

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to add the Control Plane stack for UE-UE in TS 23.304

# 1	Introduction

This contribution adds the Control Plane stack for UE-UE communication in TS 23.304.

1) Discovery plane PC5 interface

The following conclusion was made in TR 23.752.

## 8.1 Key Issue #1: ProSe Direct discovery For Key Issue #1 (ProSe Direct discovery), the following aspects are concluded: - For discovery procedure over PC5 for commercial services and public safety, both model A and model B as defined in TS 23.303 [9] are recommended to be standardized. NOTE 1: Mechanism for discovering a UE-to-Network Relay and UE-to-UE Relay can be concluded in KI#3 and KI#4. - PC5 communication channel is used to carry the discovery message over PC5 and discovery message over PC5 is differentiated with other PC5 messages by AS layer. NOTE 2: Whether PC5-S signalling or any other new signalling in upper layer is used will be decided during the normative phase based on the protocol stack and the message structures/formats to be defined for PC5 direct discovery. …

Regarding NOTE 2, actually, conclusion was already made as seen in S2-2009229 (Reply LS to Reply LS on Direct Discovery and Relay) sent to RAN2 at SA2#142E.

Q1: Whether discovery message could be taken as PC5-S signalling or other new signalling in upper layer? A1: SA2 agreed that direct discovery message will be taken as new signalling in ProSe layer separately from PC5-S signalling. ProSe layer will indicate to AS layer whether the signalling is discovery message or PC5-S signalling.

Therefore, "ProSe protocol" over PDCP/RLC/MAC/PHY can be taken as a discovery plane for NR PC5.

2) PC5 Signalling Protocol

The PC5 Signalling Protocol stack specified in clause 6.1.2 of TS 23.287 [2] can be referred while mentioning the clauses about procedures for unicast, UE-to-NW Relay and UE-to-UE Relay.

# 2	Proposal

It is proposed to update section 6.1.1.2 in TS 23.304 with the following modifications.

* * * * First Change * * * *

- 6.1.1.2	UE – UE

##### 6.1.1.2.1	Discovery plane PC5 interface

The PC5 communication channel is used to carry the discovery messages over PC5 which are differentiated from other PC5 messages by the AS layer.

Figure 6.1.1.2.1-1 depicts a discovery plane for NR PC5 reference point.

Legend:

-	PC5-D: The PDCP/RLC/MAC/PHY functionality is specified in TS 38.300 [xx].

-	The "ProSe protocol" is used for handling ProSe Direct Discovery as specified in clause 6.3.2.

Figure 6.1.1.2.1-1: Discovery Plane PC5 Interface

##### 6.1.1.2.2	PC5 Signalling Protocol

The PC5 Signalling Protocol stack specified in clause 6.1.2 of TS 23.287 [2] is used. The protocol used for the control plane signalling over the PC5 reference point for the secure layer-2 link is specified in clauses 6.4.3, 6.4.4 and 6.4.5.

* * * * End of Change * * * *