SA WG2 Meeting #143E e-meeting	S2-210xxxx

Feb 24 – Mar 09, 2021, Elbonia	(revision of S2-210xxxx)

Source:	Interdigital Inc.

Title:	TS 23.304: 6.3.2.1 General ProSe Direct Discovery procedures over PC5 reference point

Document for:	Approval

Agenda Item:	8.8.2

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution provides a description of General ProSe Direct Discovery procedures over PC5 reference point into TS 23.304 subclause 6.3.2.1.

# 1.	Introduction

Based on the conclusions of TR 23.752, this contribution adds a description on General ProSe Direct Discovery procedures over PC5 reference point into TS 23.304 subclause 6.3.2.1.

# 2.	Proposal

It is proposed to capture the following changes into TS 23.304.

- * * * * First change * * * *

### 6.3.2	ProSe Direct Discovery procedures over PC5 reference point

#### 6.3.2.1	General

A PC5 communication channel is used to carry the discovery message over PC5 and the discovery message over PC5 is differentiated from other PC5 messages by AS layer.

Both Model A and Model B discovery as defined in TS 23.303 [X] are supported:

-	Model A uses a single discovery protocol message (Announcement).

-	Model B uses two discovery protocol messages (Solicitation and Response).

Depicted in 6.3.2.1-1 is the procedure for ProSe Direct Discovery with Model A.

Figure 6.3.2.1-1: ProSe direct discovery with Model A

1.	The Announcing UE sends an Announcement message. The Announcement message may include the Type of Discovery Message (i.e. Announcement), Destination Layer 2 ID, Source Layer 2 ID, ProSe Application Code or ProSe Restricted Code, ProSe Application ID, User Info ID, security protection element, metadata information.

Depicted in 6.3.2.1-2 is the procedure for ProSe Direct Discovery with Model B.

Figure 6.3.2.1-2: ProSe direct discovery with Model B

- The Discoverer UE sends a Solicitation message. The Announcement message may include Type of Discovery Message (i.e. Solicitation), Destination Layer 2 ID, Source Layer 2 ID, ProSe Query Code, ProSe Application ID, User Info ID, security protection element.

- The Discoveree UE that matches the solicitation message responds to the Discoverer UE with the Response message. The Response message may include Type of Discovery Message (i.e. Response), Destination Layer 2 ID, Source Layer 2 ID, ProSe Query Code, ProSe Application ID, User Info ID, security protection element, metadata information.

NOTE X:	Details of security protection element will be defined by SA WG3.

Editor's note: How Destination Layer 2 ID and Source Layer 2 ID are carried in the discovery message, e.g. as part of MAC header, depends on RAN WG design.

- * * * * End of changes * * * *