SA WG2 Meeting #143e	S2-2100085

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-2100085)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2461 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of AF guidance to PCF determination of proper URSP rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implementing TR 23.748 conclusion clause 9.1.1 to allow AF to guide PCF determination and delivery to UE(s) of proper URSP rules in TS 23.502: - In order to allow AF to guide PCF determination and delivery to UE(s) of proper URSP rules, clause 6.13.1.2 of solution 13 are also endorsed for normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 4.15.6.X describing that PCF may use may use information stored in the UDR using the Nnef_ServiceParameter service operation to create URSP rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 EC feature not fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.15.6.X., 5.2.12.2.1 ; 5.2.6.11.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 0504 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR also relates to a 23.758 PCR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.X	DN priorities for application

##### 4.15.6.X.1	Overview

This clause describes the procedures to allow an AF to configure DN priorities for the application to 5G system via NEF and therefore influence PCF decisions for URSP rules. The AF may belong to the operator or to an external party. The PCF considered in this clause is in the Home PLMN as it is the PCF that determines the URSP for the UE.

NOTE 1:	The operator can negotiate with external party (typically a Corporate represented by an AF) dedicated DNN(s) and/or S-NSSAI(s) for the traffic of UE(s) of this external party. UE(s) of the external party can be identified by a group identifier.

An AF may provide information about the relative precedence of (DNN, S-NSSAI), used to reach different applications represented by application filters. This relative precedence may depend on the DNAI where traffic of this PDU Session to a (DNN, S-NSSAI) may be offloaded. The request may be associated with a specific UE, or a group of UE(s) or any UE.

##### 4.15.6.X.2	DN priorities for application

Information on the DN priorities for application  consists of a list of rules that each associates an application filter with requested features for the PDU session to use to reach the application matching this application filter:

-	An application filter, whose definition corresponds to that of the URSP Traffic Descriptors (as defined for the URSP rule in TS 23.503 Table 6.6.2.1-2) and is associated with a filtering rule priority.

-	one or more set of prioritized DN information that each may correspond to:

-	DN information i.e. a (DNN, S-NSSAI). This may be provided by the AF or determined by the NEF based on the AF Identifier when it is not provided by the AF and the AF provides only one instance of prioritized DN information.

.

-	a DN information priority with a corresponding spatial validity condition that indicates where the DN information is to apply. This may correspond to a geographical area (i.e. geographical zone identifier) or to a (set of) DNAI.

NOTE 2:	The different sets of prioritized DN information indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application filter. Each set is meant to apply for a specific (set of) spatial validity condition. Each set is associated with a priority to cope with the case where multiple spatial validity conditions overlap.

NOTE 3: 	The purpose of " prioritized DN information" is to allow AF to influence priorities, for instance for overlapping DNAI when traffic destination may be reachable both in private/corporate DNN and general internet DNN.

If the AF provides a geographical area as spatial validity condition, it is up to the NEF to transform this information into 3GPP identifiers (e.g. TAI(s)).

NOTE 4:	DN priorities for appDomains information are not related with 5G VN group

The PCF composes the URSP rules for the UE as follows for each DN priorities for application rule:

-	the application filter is used to set the Traffic descriptor in the URSP rule (defined in TS 23.503 Table 6.6.2.1-2);

-	the filtering rule priority is used to set the Rule precedence value of the URSP rule (defined in TS 23.503 Table 6.6.2.1-2),

-	each prioritized DN information is used to set a Route Selection Descriptor as follows:

-	DNN and S-NSSAI from the prioritized DN information are used to set the DNN selection and Network Slice selection Route selection components in the Route Selection Descriptor of the URSP rule, respectively (defined in TS 23.503 Table 6.6.2.1-3); The DNN and S-NSSAI provided may be mapped in order to isolate AF from changes on the system configuration;

-	the DN information priority is used to set the Route Selection Descriptor Precedence in the Route Selection Descriptor (defined in TS 23.503 Table 6.6.2.1-3);

-	DNAI or geographical zone identifier in the spatial Validity conditions, if any, are mapped to the list of Cell IDs, RAN node IDs or TAI list in the Location Criteria in the Route Selection Descriptor of the URSP rule (defined in TS 23.503 Table 6.6.2.1-3).

The PCF stores the composed URSP rules of the UE to the UDR as part of UE's Policy Set entry and updates the UE as described in clause 4.2.4.3 in TS 23.502 and clause 6.1.2.42.2 in TS 23.503.

##### 4.15.6.X.3	Configuring PCF with DN priorities for application

AF

NEF

UDR

PCF

UE

6. UE policy delivery as specified in TS 23.503 clause 6.6

1. Nnef_ServiceParameter_ Create Request

2. Nudr_DataRepository_ Create Request

3. Nudr_DataRepository_ Create Response

5. Nudr_DataRepository_ Notify

4. Nnef_ServiceParameter_ Create Response

0. Nudr_DataRepository_ Subsribe

Figure 4.15.6.X.3-1: Configuring DN priorities for application

1.	An Application Function (AF) invokes Nnef_ServiceParameter service to communicate DN priorities for application (as defined in clause 4.15.6.X.2) to the NEF. This information may be associated ("target") with an individual UE or with a group of UE (for example all UE(s) of a corporate) or with any UE.

AF may update or remove an existing request as specified in clause 4.15.6.7

2.	The NEF may perform parameter translation such as translating the AF identity into a target (DNN, S-NSSAI). The NEF stores the information (after possible translation ) in the UDR (the corresponding data set and subset are indicated in step  5).

3.	The UDR responds to NEF.

4.	The NEF responds to the AF and provides a Transaction Reference ID allowing the AF to later on update or remove the rule.

5.	The PCF may acquire (GET or SUBSCRIBE to notifications on change of) Application Data from UDR for the DN priorities for appDomains information.

The PCF(s) that have subscribed to notification on modifications of AF requests (Data Set = Application Data; Data Subset = DN priorities for application,) receive(s) a Nudr_DM_Notify notification of data change from the UDR. Internal Group Identifier or SUPI may be used as Data Key to cover the case where the AF requests "targets" an individual UE or a group of UE.

6.	The PCF applies DN priorities for application to build URSP. The PCF stores the URSP rules to the UDR as part of UE's Policy Set entry as specified in clause 6.1.2.4 in TS 23.503 and updates the UE as specified in clause 4.2.4.3 in TS 23.502.

NOTE 1:	NEF may receive different policies from different AF (e.g. receive policies from external party AF corresponding a (corporate) group of users and apply policies associated to any UE received from different AF(s) managing Edge computing application deployments). The PCF needs to build an unique map of relative priorities between (DNN, S-NSSAI) for a given application (e.g. range of FQDN) (to be sent to the UE as URSP,). For this purpose, the NEF and PCF consider local policies to resolve the potential conflicts: these local policies take into account the identity of the AF that has provided a DN priorities for application policy. The PCF and the NEF can also use local policies to determine URSP parameters not provided in the AF requests.

NEXT CHANGE (2)

##### 5.2.12.2.1	General

The operations defined for Nudr_DM service use following set of parameters defined in this clause:

-	Data Set Identifier: uniquely identifies the requested set of data within the UDR (see clause 4.2.5).

-	Data Subset Identifier: it uniquely identifies the data subset within each Data Set Identifier. As specified in the procedures in clause 4, e.g. subscription data can consist of subsets particularised for specific procedures like mobility, session, etc.

-	Data Keys defined in Table 5.2.12.2.1-1

For Nudr_DM_Subscribe and Nudr_DM_Notify operations:

-	The Target of Event Reporting is made up of a Data Key and possibly a Data Sub Key both defined in Table 5.2.12.2.1-1. When a Data Sub Key is defined in the table but not present in the Nudr_DM_Subscribe this means that all values of the Data Sub Key are targeted.

-	The Data Set Identifier plus (if present) the (set of) Data Subset Identifier(s) corresponds to a (set of) Event ID(s) as defined in clause 4.15.1

An NF Service Consumer may include an indicator when it invokes Nudr_DM Query/Create/Update service operation to subscribe the changes of the data, to avoid a separate Nudr_DM_Subscribe service operation.

Depending on the use case, it is possible to use a Data Key and/or one or multiple Data sub keys to further identify the corresponding data, as defined in Table 5.2.12.2.1-1 below.

Table 5.2.12.2.1-1: Data keys

| Data Set | Data Subset | Data Key | Data Sub Key |
| --- | --- | --- | --- |
|  | Access and Mobility Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMF Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | UE context in SMF data | SUPI | PDU Session ID or DNN |
| Subscription Data (see clause 5.2.3.3.1) | SMS Management Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | SMS Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Session Management Subscription data | SUPI | S-NSSAI |
|  |  |  | DNN |
|  |  |  | Serving PLMN ID and optionally NID |
|  | Slice Selection Subscription data | SUPI | Serving PLMN ID and optionally NID |
|  | Group Data | Internal Group Identifier or External Group Identifier | - |
|  | Identifier translation | GPSI |  |
|  |  | SUPI | Application Port ID |
|  | Intersystem continuity Context | SUPI | DNN |
|  | LCS privacy | SUPI | - |
|  | LCS mobile origination | SUPI | - |
|  | UE reachability | SUPI | - |
|  | Group Identifier Translation | Internal Group Identifier or External Group Identifier | - |
|  | UE context in SMSF data | SUPI | - |
|  | V2X Subscription data | SUPI | - |
| Application data | Packet Flow Descriptions (PFDs) | Application Identifier | - |
|  | AF traffic influence request information | AF transaction internal ID |  |
|  | (See clause 5.6.7 and clause 6.3.7.2 in TS 23.501 [2]). | S-NSSAI and DNN and/or Internal Group Identifier or SUPI |  |
|  | Background Data Transfer (NOTE 3) | Internal Group Identifier or SUPI |  |
|  | DN priorities for appDomains | S-NSSAI and DNN and/or Internal Group Identifier or SUPI or indication for Any UE |  |
|  |  | Transaction Reference ID |  |
|  | Service specific information (See clause 4.15.6.7) | S-NSSAI and DNN or Internal Group Identifier or SUPI |  |
| Policy Data | UE context policy control data (See clause 6.2.1.3 in TS 23.503 [20]) | SUPI |  |
|  | PDU Session policy control data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 in TS 23.503 [20]) |  | DNN |
|  | Policy Set Entry data (See clause 6.2.1.3 in TS 23.503 [20]) | SUPI (for the UDR in HPLMN) |  |
|  |  | PLMN ID (for the UDR in VPLMN) |  |
|  | Remaining allowed Usage data | SUPI | S-NSSAI |
|  | (See clause 6.2.1.3 in TS 23.503 [20]) |  | DNN |
|  | Sponsored data connectivity profiles (See clause 6.2.1.6 in TS 23.503 [20]) | Sponsor Identity |  |
|  | Background Data Transfer data (See clause 6.2.1.6 in TS 23.503 [20]) | Background Data Transfer Reference ID. (NOTE 2) |  |
|  |  | None. (NOTE 1) |  |
| Exposure Data | Access and Mobility Information | SUPI or GPSI | PDU Session ID or |
| (see clause 5.2.12.1) | Session Management information | SUPI or GPSI | UE IP address or DNN |
| NOTE 1: Retrieval of the stored Background Data Transfer References for all ASP identifiers in the UDR requires Data Subset but no Data Key or Data Subkey(s). NOTE 2: Update of a Background Data Transfer Reference in the UDR requires a Data key to refer to a Background Data Transfer Reference as input data. NOTE 3: The Background Data Transfer includes the Background Data Reference ID and the ASP id that requests to apply the Background Data Reference ID to the UE(s). Furthermore, the Background Data Transfer includes the relevant information received from the AF as defined in clause 6.1.2.4, TS 23.503 [20]. |  |  |  |

The content of the UDR storage for (Data Set Id= Application Data, Data Subset Id = AF TrafficInfluence request information) is specified in TS 23.501 [2], clause 5.6.7, Table 5.6.7-1. This information is written by the NEF and read by the PCF(s). PCF(s) may also subscribe to changes onto this information.

NEXT CHANGE (3)

##### 5.2.6.11.2	Nnef_ServiceParameter_Create operation

Service operation name: Nnef_ServiceParameter_Create

Description: The consumer stores service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an application identifier)

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available, indication that the parameters are to apply to any UE)

Outputs, Required: Transaction Reference ID, operation execution result indication.

Outputs, Optional: None.

Service Parameters corresponding to DN priorities for appDomains is defined in clause 4.15.6.X.2. The Service Descriptor corresponding to DN priorities for appDomains is an AF Identifier.

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES