SA WG2 Meeting #143e	S2-2100085r13

# Feb 24th – March 9th, 2021 ; Elbonia                   		 	 				(revision of S2-2100085)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2461 | rev | - | Current version: | 16.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of AF guidance to PCF determination of proper URSP rules |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-01-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implementing TR 23.748 conclusion clause 9.1.1 to allow AF to guide PCF determination and delivery to UE(s) of proper URSP rules in TS 23.502: - In order to allow AF to guide PCF determination and delivery to UE(s) of proper URSP rules, clause 6.13.1.2 of solution 13 are also endorsed for normative phase. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New clause 4.15.6.X describing that PCF may use may use information stored in the UDR using the Nnef_ServiceParameter service operation to create URSP rules. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 EC feature not fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 4.15.6.X, 5.2.6.11.2 |  |  |  |  |  |  |  |  |
| ; |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.503 CR 0504 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | This CR also relates to a 23.548 PCR |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

4.15.6.X	Application guidance for URSP determination

This clause describes the procedures to allow an AF to provide guidance for URSP determination to 5G system via NEF. The AF may belong to the operator or to an external party. The PCF considered in this clause is in the Home PLMN as it is the PCF that determines the URSP for the UE.

NOTE 1:	The operator can negotiate with external party (typically a Corporate represented by an AF) dedicated DNN(s) and/or S-NSSAI(s) for the traffic of UE(s) of this external party. UE(s) of the external party can be identified by a group identifier.

The guidance for URSP determination may be used to provide 5GC with guidance for the URSPs depending on the UE location. This is further described in TS 23.548 [xx].

For providing guidance for URSP determination, the procedure defined in clause 4.15.6.7 is performed with the following considerations:

1)	Service Description indicates an AF Identifier.

2)	Service Parameters.

Information on the AF guidance for URSP determination which consists of a list of rules that associate an application traffic descriptor with requested features for the candidate PDU sessions the application traffic may use:

-	An application traffic descriptor, whose definition corresponds to that of the URSP Traffic Descriptors (as defined for the URSP rule in TS 23.503[4] Table 6.6.2.1-2).

-	one or more sets of Route selection parameters, each parameter may correspond to:

-	(DNN, S-NSSAI). This may be provided by the AF or determined by the NEF based on the AF Identifier when it is not provided by the AF and the AF provides only one instance of AF guidance for URSP determination

Editors Note: it is FFS whether the AF can provide 	SSC mode.

-	a default Route selection precedence value to be used for the application traffic when Route selection precedence with a corresponding spatial validity condition is not provided.

-	Route selection precedence with a corresponding spatial validity condition that indicates where the Route selection parameters apply. This may correspond to a geographical area (i.e. geographic zone identifier).

Editors Note: it is FFS whether the spatial validity condition may include DNAI(s). DNAI would be translated into identifiers that the UE can handle such as Cell Id(s) and TAI(s)

NOTE 2:	The different sets of Route selection parameters indicate different sets of PDU Session information (DNN, S-NSSAI) that can be associated with applications matching the application traffic descriptor. Each set is meant to apply for a specific (set of) spatial validity condition. Each set is associated with a Route selection precedence to cope with the case where multiple spatial validity conditions overlap.

If the AF provides a geographical area as spatial validity condition, it is up to the NEF or PCF  to transform this information into 3GPP identifiers (e.g. TAI(s)).

Editors Note: it is FFS whether the spatial would be translated by NEF or PCF

NEF may, based on local configuration, complement missing service parameters.

NOTE 3:	AF guidance for application traffic is not related with 5G VN group

3)	a specific UE, or a group of UE(s) or any UE that the AF request may be associated with.

The usage of the AF guidance for application traffic is described in clause 6.2.4 in TS 23.548 [x].

NEXT CHANGE (2)

5.2.6.11.2	Nnef_ServiceParameter_Create operation

Service operation name: Nnef_ServiceParameter_Create

Description: The consumer stores service specific parameters in the UDR via the NEF.

Inputs, Required: Service Descriptor (e.g. the combination of DNN and S-NSSAI, an AF-Service-Identifier or an application identifier)

Inputs, Optional: Service Parameters and Target UE identifiers (e.g. the address (IP or Ethernet) of the UE if available, GPSI if available, External Group Identifier if available)

If identifiers of target UE(s) or a group of UEs are not provided, then the Service Parameters shall correspond to any UEs using the service identified by the Service Description.

Outputs, Required: Transaction Reference ID, operation execution result indication.

Outputs, Optional: None.

NEXT CHANGE (4)

END OF CHANGES