3GPP TSG-SA2 Meeting #143-E	S2-2100094r01

Elbonia, February 24 – March 09, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0194 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  |  |  | UICC apps |  |  |  | ME |  |  | Radio Access Network |  |  |  |  |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Dispersion Analytics |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Spirent, Ericsson, Sandvine, AT&T, Huawei, Samsung |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_ph2 |  |  |  |  |  |  |  |  |  |  |  | Date: |  | 2021-02-18 |  |  |  |  |
| Category: | B |  |  |  |  |  |  |  |  |  |  |  | Release: |  | Rel-17 |  |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |  |  |  |  |
| Reason for change: |  | Introduction of Dispersion Analytics feature as captured in the conclusion of Key Issue #9: Dispersion Nalytics output provided by NWDAF in TR 23.700-91 Dispersion analytics characterizes the interaction of a UE, or a group of UEs, with the network and identifies at what locations (i.e. area of interest, TAs, cell) users dispersed most (if not all) their data volume and sessions transactions. Same concept applies to dispersion at a given network slice; at which slice UEs dispersed most (if not all) their data and session transactions. As part of the characterization, a UE or a group of UEs, are evaluated and classified as either fixed users, campers or travellers at the Area of Interest. This classification is determined per operator's defined thresholds. For example, if a UE disperses, during the period of observation, most (threshold = 95%) of its data volume at a location, the dispersion data-classification of the UE is fixed for that location. Likewise, different thresholds are defined for campers and travellers categories at the location. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new section for the new Rel 17 Dispersion Analytis feature is provided with clauses that cover General information, Dispersion Analytics attributes, Input data, Output data and a dispersion analytics procedure. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Delays in the introduction of the Dispersion Analytic new feature, which was successfully concluded in TR 23.700-91 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.x.1, 6.x.2, 6.x.3, 6.x.3.1, 6.x.3.2, 6.x.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  | X |  |  | Other core specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| affected: |  |  | X |  | Test specifications |  |  |  |  |  |  | TS/TR 23.502 CR xxx |  |  |  |  |  |  |  |
| (show related CRs) |  |  | X |  | O&M Specifications |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Changes – New Subclause 6.x* * * *

### 6. x  Dispersion Analytics

#### 6.x.1	General

Dispersion analytics identifies the characteristics  that a UE , or a group of UEs  disperse most (if not all) their data volume and sessions transactions (i.e. MM and SM messages) at a given location (i.e. areas of interest, TAs, cells) or in given network slices.

The NWDAF shall be able to provide dispersion analytics statistics or predictions and shall be able to collect a UE dispersion related information from NFs and perform analytics to provide dispersion statistics or predictions.

Dispersion - The percentage of activity that a UE, or group of UEs, generated at a location or a slice during a period of interest. This enables the operator to rank hot locations or slices by the various activities and to identify the top contributors (UEs) for that activity. When heavy UEs at a particular location or slice are detected and reported to a policy entity, their QoS attributes, or access to an area of interest or a slice can then be limited.

-	Data dispersion - The percentage of data traffic volume that a UE, or a group of UEs , generated at the location or a slice during the period of interest.

-	Transaction dispersion - The percentage of MM and SM messages that a UE, or a group of UEs, generated at the location or a slice during the period of interest.

The operator may classify and assign one of the three mobility classes per dispersion characteristics for a UE or a group of UEs as fixed, camper or traveller. The classification is based on either Data or Transaction.

-	Data-Classification - fixed, camper, traveller data-classification per thresholds assigned by the operator. As an  example, when a UE disperses, during the period of observation, most (threshold = 95%) of its data at a location or a slice, the dispersion data-classification of the UE at that location or slice is fixed.

-	Transaction-Classification - fixed, camper, traveller transactions-classifications per thresholds assigned by the operator.As an example, when a UE disperses, during the period of observation, (threshold=) 40% of its session transactions at a location or a slice, the dispersion transaction-classification of the UE is camper at that location or slice.

Through dispersion analytics, the NWDAF can determine that a data hot spot is formed when the amount of data or amount of transactions dispersed by most of the UEs at the area of interest or slice exceeds a certain threshold established through statistical trending or operator policy.

The consumer of dispersion analytics may indicate in its request:

-	Analytics ID set to "UE Dispersion Analytics"

-    A Dispersion Analytics (DA) type set to Data Disperison Analytics (DDA) or Transaction Dispersion Analytics (TDA)

-	The Target of analytics reporting which is a single UE, any UE, or a group of UEs.

-	Analytics Filter Information optional list of TA(s), Area(s) of Interest, Cells, or S-NSSAI.

-	An Analytics target period indicating the time period over which the statistics or predictions are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address.

NOTE:	Care must be taken with regards to load when requesting Any UE. This could be achieved via utilization of some event filters (e.g. Area of Interest for AMF) and possible analytic filters (e.g. top-heavy users or fixed campers) and/or Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

#### 6.x.2	Input Data

The NWDAF shall be able to collect UE dispersion information from NF(s) and AFs. The detailed information collected by the NWDAF is network data from 5GC  NFs and service data AFs:

-	Data related to UE dispersion and bound by location collected from 5GC are defined in the Tables 6.x.2-1 and 6.x.2-2 ;

Table 6.x.2-1: Location based UE transaction dispersion information collected from serving AMF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| Type Allocation Code | AMF | Terminal model and vendor information of the UE. |
| UE locations (1..max) | AMF | UE positions |
| >UE location |  | Area of Interest (TA or cells that the UE enters) |
| >Timestamp |  | A time stamp when the AMF detects the UE entering this location |
| >Transactions | AMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged at the location. |

Table 6.x.2-2: UE transction dispersion information collected from serving SMF

| Information | Source | Description |
| --- | --- | --- |
| UE IP | SMF | UE IP address |
| UE ID | SMF | SUPI |
| >Timestamp |  | A time stamp associated with the collected information |
| >Transactions | SMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged. |

Data volume can be collected from the AF per UE across all applications or per UE per application(s) when the application IDs are requested.

- Data related to UE data dispersion and bound by location or slice collected from the AF and UPF are defined in tables 6.x.2-3 through 6.x.2-6. In table 6.x.2-3, the data volume is collected per UE from the AF. The collected UE information is applicable across all the applications used by the UE during the period.

Table 6.x.2-3:  UE Data dispersion information collected from the AF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AF | Could be external UE ID (i.e. GPSI) |
| Timestamp | AF | A time stamp of the collected information |
| >Data Volume UL/DL | AF | Amount of UE data volume exchanged per application during the period of observation, if application ID is provided in the Analytics request. |

- In table 6.x.2-4, The AF reports volume per UE per application in relation to the start and stop of the application as indicated by the application duration.

Table 6.x.2-4: UE data dispersion collected from the AF

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | AF | UE IP address |
| IP 5-tuple | AF | IP address 5-tuple |
| Timestamp | AF | A timestamp of the collected information |
| Application ID | AF | Identify the application at the AF |
| Location of Application | AF/NEF | Location of application represented by a list of DNAI(s). The NEF may map the AF-Service identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Data Volume UL/DL | AF | amount of UE data volume exchanged per application during the period. |
| Application duration | AF | (Duration for the application (e.g. Voice talk time) |
| NOTE1: NOTE 2: |  | Application ID and IP 5-tuple are mutually exclusive. Multiple outputs are provided by the AF when multiple applications are running at the AF for the same UE and time period. |

Data volume can also be collected from the UPF per UE IP address across all applications or per UE for each  application(s).

- In table 6.x.2-5, the data volume is collected per UE from the UPF. The collected UE information is applicable across all the applications used by the UE between start and stop of the PDU session.  The UPF reports volume per UE IP address across all applications.

Table 6.x.2-5: UE data dispersion collected from serving UPF

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | UPF | UE IP address |
| Timestamp |  | A time stamp of the collected information |
| Data Volume UL/DL | UPF | Amount of UE data volume exchanged per UE across all applications. |
| NOTE |  | The Data volume can be reported either as total volume of the PDU session or periodically. It refers to the Data volume exchanged between the start and stop of the PDU session. When reported periodically, the period can be specified in the requested analytic target period or otherwise it is a default value configured in the UPF. |

- In table 6.x.2-6, The UPF reports volume per UE for each  application(s) in relation to the start and stop of the application as indicated by the application duration.

Threre are two modes of data collection:

- Non periodical-  A mode where the data volume is exchanged between the start and stop of an application. If there are multiple application sessions for the UE PDU session, the NWDAF aggregates the volume across all applications to obtain per UE information.

- Periodical- A mode where data volume is exchanged periodically between start and stop of an application. The period can be specified in the requested analytic target period or otherwise it is a default value configured in the UPF.

Table 6.x.2-6: UE data dispersion collected from serving UPF

| Information | Source | Description |
| --- | --- | --- |
| UE IP address | UPF | UE IP address |
| Timestamp | UPF | A timestamp of the collected information |
| Application ID | UPF | Identify the application at the UPF |
| IP 5-tuple | UPF | IP 5-tuple |
| Location of Application | UPF | List of Internet applications represented by DNAI(s). |
| Data Volume UL/DL | UPF | amount of UE data volume exchanged per application during the period. |
| Application duration | UPF | (Duration for the application (e.g. Voice talk time) |
| NOTE1: NOTE 2: NOTE 3: |  | Application ID and IP 5-tuple are mutually exclusive. Multiple outputs are provided by the UPF when multiple applications are running at the UPF for the same UE and time period The Data volume can be reported either as total volume of the PDU session or periodically. It refers to the Data volume exchanged between the start and stop of the PDU session. When reported periodically, the period can be specified in the requested analytic target period or otherwise it is a default value configured in the UPF. |

The focus of the NWDAF is to calculate the volume of data and amount of transactions dispersed by the UE at each visited location during a period of interest. The entry time and exit time to a location is provided by the AMF.

If the optional application ID or IP 5-tuple is requested, the volume per UE per application can be provided by the UPF or AF.

The Correlation Information as defined in Table 6.2.4-1  is used to correlate the input data from AMF and SMF, the input data from SMF and UPF, the input data from UPF and AF.

-	Slice information related to UE transaction dispersion from NF(s) are defined in the Tables 6.x.2-7 and 6.x.2.-8.

-	The Slice data volume collection is the same as listed in tables 6.x.2.3 through 6.2.3.6.

Table 6.x.2-7: Slice based UE transaction dispersion information collected from serving AMF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | AMF | SUPI |
| Type Allocation Code | AMF | Terminal model and vendor information of the UE. |
| Slice (1..max) | AMF | UE assigned slice |
| >S-NSSAI |  | Slice (up to 8 per UE at any given instance) |
| >Timestamp |  | A time stamp when the slice was assigned |
| >Transactions | AMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged at the slice |

Table 6.x.2-8: Slice based UE transactin dispersion information collected from serving SMF

| Information | Source | Description |
| --- | --- | --- |
| UE IP | SMF | UE IP address |
| UE ID | SMF | SUPI |
| Slice (1..max) |  | UE assigned slice |
| >S-NSSAI |  | Slice (up to 8 per UE at any given instance) |
| >Timestamp |  | A time stamp associated with the collected information |
| >Application ID | SMF | Identify the application for the UE at the slice |
| >Transactions | SMF | Either per UE transaction or optionally if subscription with periodic notification is requested an amount of UE transactions exchanged at the slice |

The focus of the NWDAF is to calculate the volume of data and amount of transactions dispersed by the UE at each slice during a period of interest. A time stamp of a slice assignment is provided by the AMF.

The Correlation Information as defined in Table 6.2.4-1  is used to correlate the input data from AMF and SMF, the input data from SMF and UPF, the input data from UPF and AF.

Data collection may be provided on samples (e.g. spatial subsets of UEs or UE group, temporal subsets of UE location information).

NOTE:	How the data from UPF is retrieved is not specified in this release.

NOTE: The list of monitored transactions (MM and SM messages) is the subset of the messages listed in TS 24.501 [yy] clauses 8.2 and 8.3.

Editor’s note: The Namf_EventExposure and Nsmf_EventExposure services are enhanced in a different CR to provide transaction information or transaction count in Tables 6.x.1.2-1/2/7/8.

#### 6.x.3	Output Analytics

The NWDAF  provides UE dispersion analytics, which can be statistics and/or prediction for data and/or transaction disperiosns, to consumer NFs or AFs..

##### 6.x.3..1	Data Dispersion Analysis

The data dispersion analytics results provided by the NWDAF  can be UE data dispersion statistics as defined in Table 6.x.3.1-1 and/or UE data dispersion predictions as defined in Table 6.x.3.1-2:

Table 6.x.3.1-1: Data dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics |
| >> UE location | TA or cells where the UE dispersed its data |
| >> Application ID (1..max) | To identify the application for the UE |
| >>> Data dispersed | Data volume dispersed at this location and application, if application ID is provided. |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller |
| >> Data ranking | Percentile ranking of data usage at this location (i.e. % data dispersed at this location) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

Table 6.x.3.1-2: Data dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location during the analytics target period |
| >> UE location | TA or cells where the UE is predicted to disperse its data |
| >> Application ID (1..max) | To identify the application for the UE |
| >>> Data dispersion | Data volume dispersion prediction at this location and application, if application ID is provided. |
| >> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location) |
| >> Data classification | Data mobility classification for this location: fixed, camper, traveller |
| >> Confidence | Confidence of this prediction (i.e. mobility classification at this location) |
| >> Data ranking | Percentile ranking of data usage at this location (i.e. % data dispersed at this location) |
| >> Confidence | Confidence of this prediction (i.e. percentile ranking at this location) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

The data dispersion analytics results provided by the NWDAF can also be for a UE or group of UEs data dispersion statistics at a given slice as defined in Table 6.x.3.1-3 and/or dispersion predictions as defined in Table 6.x.3.1-4:

Table 6.x.3.1-3: Data dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > Slice (1..max) | Observed slice statistics |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed data |
| >> Application ID (1..max) | To identify the application for the UE at the slice |
| >>> Data dispersed | Data volume dispersed at this slice and application, if application ID is provided. |
| >> Data classification | Data mobility classification for slice: fixed, camper, traveller |
| >> Data ranking | Percentile ranking of data usage at this slice (i.e. % data dispersed at this slice) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

Editor Note: Consider the addition of “>>Bandwidth” into tables 6.x.3.1-3 and 6.x.3.1-4, which is the utilized bandwidth at the slice calculated by the NWDAF as the volume over the period. See also SA5 Liasion letter S2-2008361 (S5-205436).

Table 6.x.3.1-4: Data dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > Slice (1..max) | Predicted slice during the Analytics target period |
| >> S-NSSAI | Slice where the UE is predicted to disperse its data |
| >> Application ID (1..max) | To identify the application for the UE at the slice |
| >>> Data dispersion | Data volume dispersion prediction at this slice and application, if application ID is provided. |
| >> Confidence | Confidence of this prediction (i.e. data to be dispersed at this slice) |
| >> Data classification | Data mobility classification for this slice: fixed, camper, traveller |
| >> Confidence | Confidence of this prediction (i.e. mobility classification at this slice) |
| >> Data ranking | Percentile ranking of data usage at this slice (i.e. % data dispersed at this slice) |
| >> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

NOTE 1:	When target of analytics reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the dispersion analytics result of one or a  list of  time slots to the NF service consumer(s) for the UE.

NOTE 2:	The Application ID in the Data Dispersion Analytics is optional. When the Application ID is missing, the Data Dispersion Analytics is applied across all the applications in an AoI or a slice.

##### 6.x.3.2	Transactions Dispersion Analysis

The transaction (MM and MS messages) dispersion analytics results provided by the NWDAF can be UE transaction dispersion statistics as defined in Table 6.x.3.2-1 and/or UE transaction dispersion predications as defined in Table 6.x.3.2-2:

Table 6.x.3.2-1: Transaction dispersion statistics bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Observed location statistics |
| >> UE location | TA or cells where the UE dispersed its transactions |
| >> Transaction dispersed | Transactions amount dispersed at this location |
| >> Transaction classification | Transactions mobility classification for this location: fixed, camper, traveller |
| >> Transaction ranking | Percentile ranking of transactions volume at this location (i.e. % transaction dispersed at this location) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

Table 6.x.3.2-2: Transaction dispersion prediction bound by location

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of predicted time slots |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > UE location (1..max) | Predicted location during the analytic target period |
| >> UE location | TA or cells where the UE is predicted to disperse its transactions |
| >> Transaction dispersion | Transaction amount predicted to be dispersed at this location |
| >> Confidence | Confidence of this prediction (i.e. data to be dispersed at this location) |
| >> Transaction classification | Transaction mobility classification for this location: fixed, camper, traveller |
| >> Confidence | Confidence of this prediction (i.e. mobility classification at this location) |
| >> Transaction ranking | Percentile ranking of transactions at this location (i.e. % transaction dispersed at this location) |
| >> Confidence | Confidence of this prediction (i.e. percentile ranking at this location) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

The transaction dispersion analytics results provided by the NWDAF  can be UE or group of UEs transaction dispersion statistics at a given slice as defined in Table 6.x.3.2-3 and/or dispersion predictions as defined in Table 6.x.1.4.2-4:

Table 6.x.3.2-3: Transaction dispersion statistics bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of time slots during the Analytics target period |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > Slice (1..max) | Observed slice statistics |
| >> S-NSSAI | Slice where the UE or group of UEs dispersed its transactions |
| >> Transaction dispersed | Transactions volume dispersed at this location |
| >> Transaction classification | Transaction mobility classification for slice: fixed, camper, traveller |
| >> Transaction ranking | Percentile ranking of transactions at this slice (i.e. % transaction dispersed at this slice) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

Table 6.x.3.2-4: Transaction dispersion prediction bound by slice

| Information | Description |
| --- | --- |
| UE group ID or UE ID | Identifies a UE or a group of UEs, e.g. internal group ID defined in TS 23.501 [2] clause 5.9.7, SUPI (see NOTE). |
| Time slot entry (1..max) | List of predicted time slots |
| > Time slot start | Time slot start within the Analytics target period |
| > Duration | Duration of the time slot |
| > Slice (1..max) | Predicted slice during the Analytics target period |
| >> S-NSSAI | Slice where the UE is predicted to disperse its transactions |
| >> Transaction dispersion | Transaction volume to be dispersed at this slice |
| >> Confidence | Confidence of this prediction (i.e. transactions to be dispersed at this slice) |
| >> Transaction classification | Transaction mobility classification for this slice: fixed, camper, traveller |
| >> Confidence | Confidence of this prediction (i.e. mobility classification at this slice) |
| >> Transaction ranking | Percentile ranking of transactions at this slice (i.e. % transaction dispersed at this slice) |
| >> Confidence | Confidence of this prediction (i.e. percentile ranking at this slice) |
| >> Ratio | Percentage of UEs in the group (in case of UE group) |

NOTE:	When target of analytics reporting is an individual UE, one UE ID (i.e. SUPI) will be included, the NWDAF will provide the dispersion analytics result for one or a list of time slots to the NF service consumer(s) for the UE.

### 6.x.4	Dispersion Analytic Procedure

The NWDAF can provide  Dispersion analytics, in the form of statistics or predictions or both, to another NF. If the NF is an AF, and when the AF is untrusted, the AF will request analytics via the NEF, and the NEF will then convey the request to the NWDAF.

Figure 6.x.4-1: UE Dispersion Analytics provided to an NF

1.	The NF sends a request to the NWDAF for dispersion analytics on a specific UE, any UE, or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF can request statistics or predictions or both. The type of analytics is set to “UE Dispersion Analytics”, the Dispersion Analytic (DA) type is set to “Data Dispersion Analytics” (DDA) or  ”Transactions Dispersion Analytics” (DTA) and analytic filter information = (Area of Interest, slice, target period). The NF provides the UE id or Internal Group ID in the Target of Analytics Reporting.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs, SMFs of the requested UE(s) for notification of location changes or a slice change (a slice change can be an additional slice or a deletion). This step may be skipped when e.g. the NWDAF already has the requested analytics available.

The NWDAF subscribes to application service data from AF(s) by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (via NEF).

NOTE:	The NWDAF determines the AMF serving the UE, any UE, or the group of UEs as described in TS 23.288 [5] clause 6.2.2.1.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provides requested UE dispersion analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Request Response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for UE dispersion analytics provided by NWDAF are described in clause 6.x.1.4. The provided analytics enables the consumer to predict changing network conditions such as data volume change at a location or a slice, excessive signalling conditions at a location or a slice, etc.

5-6.  If at step 1, the NF has subscribed to receive notifications for UE dispersion analytics, after receiving event notification from the AMFs (e.g. location change) or SMFs (e.g. slice change add/delete) subscribed by NWDAF in step 2, the NWDAF may generate new dispersion analytics

7.	The NWDAF provides the newly generated dispersion analytics to the NF. The details for UE dispersion analytics provided by NWDAF are described in clause 6.x.3.

* * * * End of Changes * * * *