3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100116r02

Elbonia, February 24 – March 09, 2021

Source:	Huawei, Nokia, Nokia Shanghai Bell, China Mobile, ZTE, Futurewei, NTT DOCOMO, Lenovo, InterDigital Inc.

Title:	EAS Discovery for Session Breakout connectivity model

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract of the contribution: This contribution proposes adding into TS 23.548 DNS-based EAS discovery procedure over a session breakout connectivity model, according to conclusion in clause 9.1.4 in TR 23.748.

- Discussion

This paper is to incorporate solution 22 of TR 23.748 in TS 23.548 clause 6.2.3.

NOTE: LDNSR used in solution 22 of TR 23.748 is proposed to be replaced by EASDF (Edge Application Server Discovery Function) in this paper.

The following discussion is to address the open issues of solution 22 in TR 23.748:

Open Issue 1: How does the EASDF maintain a PDU Session context bound to the PDU Session?

According to clause 6.22.1 of TR 23.748, this issue is left for decision in normative phase. There are 2 possible ways being discussed:

Alternative 1: The EASDF acting as AF subscribes to PDU Session Status event exposure from the SMF, and SMF notifies the PDU Session Status to EASDF

Alternative 2: The SMF invokes a new EASDF service to provide PDU Session Status to EASDF

For the first alternative, as the SMF event exposure service is used, the assumption is that no EASDF service needs to be defined. In addition, according to TR 23.748, clause 6.22.2, after the SMF selects an EASDF, e.g., based on local configuration of suing NRF-based NF selection mechanisms, the PDU Session context “may also be created at this stage” and be communicated to EASDF. It is unclear, however, when and how, once the PDU Session is established, the SMF communicates the PDU Session context to the EASDF. After the PDU Session is established, and the initial PDU Session context is communicated, the EASDF, acting as an AF, may subscribe to SMF User Plane management event notification for PDU Session Status notifications. When a PDU Session is modified, the SMF sends PDU Session Status notification to the EASDF according to the subscription, the PDU Session Status notification includes the IP address of the PDU Session and the SMF ID of the PDU session. Later, if the EASDF needs to send the new EAS information to the SMF (e.g., due to Application Relocation), the EASDF invokes SMF service based on SMF ID. For this alternative, it is unclear:

- When and how, the PDU Session context is communicated to the EASDF, after the PDU Session is established and the EASDF is selected by the SMF..

- How to update the DNS Response notification trigger. E.g., Several potential optimizations for DNS Response handling have been discussed, e.g. the SMF provides the condition to trigger the EASDF to notify it about certain characteristics in the DNS Response, e.g., when the IP Address (of the EAS) returned by the DNS, is within certain range. The condition may need to be updated after UE mobility, or after UL CL/BP insertion. Without the EASDF service, it is difficult to update information stored in the EASDF for the PDU Session.

- Furthermore, for option 2b, the SMF needs to send DNS Response to EASDF. A NF providing the EASDF service can used existing Service-Based principle as those specified in 23.501, clause 6.3. Thereby enabling consistence interaction between the SMF and EASDF.

The alternative 2 will define a EASDF service. During PDU Session establishment, the SMF discover and select one EASDF. After that, it invokes the EASDF service to create a PDU Session status context in the EASDF. The SMF may provide callback URI and notification condition to the EASDF, when the notification condition is matched, the EASDF sends notification to SMF and provides the EAS information (IP address, FQDN) by using the callback URI. The SMF can update the context created in EASDF for the PDU Session, e.g. update the notification condition for the EAS information notification.

Based on the above analysis, the alternative 2 is a more flexible solution. Hence, it is recommended that the alternative 2 is adopted as a way forward.

Proposal 1: A new EASDF service is defined. The SMF invokes a new EASDF service to create a context at EASDF.

Open Issue 2: How does the EASDF inform the SMF of the EAS information?

In NOTE 3 of clause 6.22.1 in TR 23.748, 2 options are listed:

Alternative1: use SMF service.

Alternative2: use AF influence on traffic routing procedure.

This proposal suggest yet another option, which is:

Alternative 3: Assuming a new NF, i.e., EASDF, the SMF can use the notification service of EASDF to inform SMF about the EAS Address provided in the DNS response, i.e. SMF subscribes to the EASDF notification service.

Based on proposal 1, option 2 is excluded.

Both option 1 and option 3 are possible. Option 3 is proposed, because it doesn’t need new SMF service, and it is aligned with current SBI interface of SMF.

With Option 3, the DNS Query call flow is depicted as following:

Only a EASDF service is defined. The service includes create, update, delete, notify operation. The subscription may be piggybacked in create or update operation.

Proposal 2: EASDF service is defined to notify the DNS Response, i.e. new EAS information, to SMF.

- Proposal

It is proposed to add the following solution to TS 23.548.

************************ 1st Change, all new text************************

### 6.2.3	EAS (re-)discovery over Session Breakout connectivity model

#### 6.2.3.1	General

This clause describes the EAS discovery and re-discovery procedures for PDU Session with Session Breakout connectivity model.

Editor's Note: The following C-DNS/L-DNS description will be moved to other more generic clause in future meeting since it applies to all connectivity models.

Central DNS (C-DNS) server is centrally deployed by MNO or 3rd party and is responsible for resolving the UE DNS queries into suitable Edge Application Server (EAS) IP address(es).

Local DNS (L-DNS) resolvers/servers may be locally deployed by MNO or 3rd parties within the edge hosting environment, and is responsible for resolving UE DNS queries into suitable EAS IP address(es) within the local DN. The L-DNS resolvers/servers may or may not have connectivity with C-DNS depending on the deployment.

NOTE 1:	The C-DNS server and/or L-DNS resolvers/servers can use an anycast address.

NOTE 2:	The C-DNS server or L-DNS resolvers/servers can contact any other DNS servers for recursive queries, which is out of scope of this specification.

The following Session breakout models are defined:

-	Dynamic session breakout: ULCL/BP/Local PSA (and their associated traffic filters and forwarding rules) are inserted based on DNS Response provided by the EASDF.

-	Pre-established session breakout: ULCL/BP/Local PSA (and their associated traffic filters and forwarding rules) are inserted without dependency on the UE sending out DNS queries or data traffic. They are typically inserted based on local configuration or per AF request.

#### 6.2.3.2	EAS discovery procedure

##### 6.2.3.2.1	General

For PDU Session with Session Breakout connectivity model, based on UE subscription (e.g. DNN) and/or the operator’s configuration, the DNS Query sent by UE may be handled by an EASDF, or by a local or central DNS resolver/server.

For the case that the UE DNS Query is to be handled by EASDF, the following applies.

-	During the PDU Session establishment procedure, the SMF selects an EASDF and provides its address to the UE as the DNS Server to be used for the PDU Session. The UE sends DNS Query to the EASDF. The SMF configures the EASDF with rules to handle DNS messages of the UE: If the FQDN in a DNS Query matches the FQDN(s) provided by the SMF, based on instructions by SMF, one of the following options is executed by the EASDF:

-	Option A: The EASDF adds the ECS option into the DNS Query message as defined in RFC 7871[y], and sends the DNS Query message to the DNS server. The DNS server resolves the EAS IP address considering the ECS option, and sends the DNS Response to the EASDF.

-	Option B: The EASDF forwards the DNS Query message to a suitable Local DNS server which is responsible for resolving EAS within the corresponding Local DN, and receives DNS Response message from Local DNS server.

-	Option C: The EASDF sends the DNS Query message to SMF and the SMF forwards the DNS Query message to a suitable Local DNS server via a UPF. The Local DNS server sends back the DNS Response message to the EASDF via the UPF and the SMF. This option applies to the scenario where the Local DNS server is on a Local DN that has no direct user plane connectivity with Central DN.

Editor's Note: The Option C related scenario is to be clarified in a general clause.

The SMF instructions for a matching FQDN may as well indicate EASDF to contact SMF. SMF may then provide instructions to EASDF to execute one of the above Options (A, B or C) or to simply forward the DNS Query towards a preconfigured DNS server/resolver for DNS resolution.

-	If the DNS Query from the UE does not match a notification condition set by the SMF, then the EASDF may simply forward the DNS Query towards a preconfigured DNS server/resolver for DNS resolution.

-	When the EASDF receives a DNS Response message, the EASDF may notify the EAS information (i.e. EAS IP address and optionally the EAS FQDN) to the SMF if the notification condition provided by the SMF. The notification condition corresponds to IP address range(s) or FQDN range(s). When the EAS IP address in the DNS Response is within the IP address range(s), or the FQDN is matched, the EASDF sends the EAS information to SMF. The SMF may trigger UL CL/BP and L-PSA insertion based on the EAS IP address and other information for UPF selection as specified in clause 6.3.3 in TS 23.501[2].

The ECS option or the Local DNS server address provided by the SMF to the EASDF are part of the rules to handle DNS queries from the UE. They are related to a candidate DNAI for that FQDN for the UE location. The SMF may provide rules to handle DNS queries from the UE to the EASDF either when the SMF establishes the association with the EASDF for the UE, and may update the rules at any time when the association exists. For the selection of the candidate DNAI for an FQDN for the UE, the SMF may consider the UE location, network topology and information of EAS deployment.. After the UE mobility, if the provided ECS option or the Local DNS server need be updated, the SMF notify them to the EASDF.

Editor's Note: The sentence above means that the EASDF gets the FQDN(s) from SMF. How the SMF getting the FQDN(s) with DNS related information e.g. the ECS option or the Local DNS server address needs to be clarified in AF influence on traffic routing procedure specified in 3GPP TS 23.502[3].

Once the UL CL/BP and L-PSA have been inserted, the SMF may decide that the DNS messages for the FQDN are to be handled by local DNS resolver/server from now on. This option is further described in clause 6.2.3.2.3. Whereas the EAS discovery procedure with EASDF, is further described in clause 6.2.3.2.2

For the case that the DNS message is to be handled by local DNS resolver/server, the DNS Query is routed to the local DNS resolver/server corresponding to the DNAI where the L-PSA connects as described in clause 6.2.3.2.3. The SMF is provisioned with the local DNS server address based on configuration or per AF request. Based on the operator’s configuration, one of the following options may apply when UL CL/BP and Local PSA have been inserted (during or after PDU Session Establishment):

-	Option D: The SMF chooses a local DNS server, and configures it to the UE as new DNS server. SMF may obtain the IP address of the local DNS server via local configuration. The local DNS server is determined based on the DNAI supported by the L-PSA. In addition, the SMF also configures the UL CL with the traffic routing rule (including e.g. Local DNS server address) to route traffic destined to the local DN including the DNS Query messages to the L-PSA. The local DNS server resolves the DNS Query either locally or recursively by communicating with other DNS servers.

-	Option E: If the SMF has configured that DNS Queries for an FQDN can be locally routed on the UL CL, then the subsequent DNS queries for the FQDN will be locally routed to the local DNS resolver. The local DNS resolver receives and handles the DNS Query that is addressing EASDF: modifies the packet's destination IP address (corresponding to EASDF) to that of the L-DNS and stores the original IP address (EASDF IP) and the packet's source IP address (corresponding to UE's IP address) to its own (i.e. the local DNS resolver's) IP address and stores the original source IP address (UE IP) for later processing. The local DNS resolver then forwards the modified DNS Query to the L-DNS, if the L-DNS can resolve the IP address, it responds to the local DNS resolver with the IP address of EAS; otherwise, it does recursive DNS resolution.

NOTE:	 Option E assumes that either ULCL steering is based on L4 information (i.e. DNS port number) or ULCL has visibility of the DNS traffic (i.e. FQDN in the DNS Query message) The UPF may be instructed by the SMF to apply different forwarding of non-ciphered UL DNS traffic based on the target domain of the DNS Query. Option E requests modification of destination IP address of DNS messages, whether this is allowed or not is subject to local regulations. Option E does not work when UE uses DoH or DoT.

##### 6.2.3.2.2	EAS discovery procedure with EASDF

This procedure applies to Option A, B and C in clause 6.2.3.2.2.

Figure 6.2.3.2.2-1: EAS discovery procedure with EASDF

1.	UE sends PDU Session Establishment Request to the SMF as shown in step 1 of clause 4.3.2.2.1 of TS 23.502[3].

2.	The SMF selects EASDF as described clause 6.3.x (TBD) of TS 23.501[1]. This selection may use NRF discovery or may be based on SMF local configuration. The EASDF may have been registered onto the NRF.

3.	The SMF invokes Neasdf_DNSContext_Create Request (UE IP address, callback URI , rules to handle DNS messages from the UE) to the selected EASDF. The rules to handle DNS messages from the UE may include [ECS information], [local DNS server information], [need to use SMF to forward traffic to the local DNS server], notification condition (when to notify the SMF).

This step is performed before step 11 of PDU Session Establishment procedure in clause 4.3.2.2.1 of TS 23.502 [3].

The EASDF creates a DNS context for the PDU Session, and stores the UE IP address, the callback URI and rules to handle DNS messages from the UE into the context.

The notification condition is the condition for the EASDF to send EAS related information to SMF when it receives DNS Query or DNS Response.

-	DNS Query for ECS options or local DNS server address handling, the SMF may set the notification condition to instruct the EASDF to send the EAS FQDN(s) to the SMF if the EAS FQDN in the DNS Query message matches with the FQDN(s) in notification condition.

-	DNS Query for EAS unreachable by the EASDF, the SMF sets the notification condition to instruct the EASDF to send the DNS Query message and AS FQDN in DNS Query message to the SMF if the AS FQDN matches with the FQDN(s) in notification condition.

-	DNS Response for specific IP address or FQDN ranges, the SMF sets the notification condition to instruct the EASDF to send EAS IP address/FQDN to the SMF if the EAS IP address in the DNS Response message is within one of the IP address range(s) of the notification condition, or the FQDN in the DNS Response matches one of the FQDNs of the notification condition.

The SMF may send the mapping information between ECS information or local DNS server and FQDN for a selected DNAI to the EASDF before the DNS Query message is received at the EASDF. For each FQDN which is to be discovered at a specific DNAI, the corresponding ECS information or local DNS server information is provided to the EASDF.

4.	The EASDF invokes the service operation Neasdf_DNSContext_Create Response (IP address of the EASDF) and with information allowing later the SMF to update or delete the context.

TThe IP address of the EASDF is the address which is to be used by the UE to reach the EASDF as a DNS Server for the PDU Session.

5.	The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502[3]. The UE configures the EASDF as DNS server for that PDU Session.

6.	The SMF may at any time invoke Neasdf_DNSContext_Update Request (PDU Session Context ID, rules to handle DNS queries from the UE) to EASDF.

The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a notification by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE.

7.	The EASDF responds with Neasdf_DNSContext_Update Response.

8.	The UE sends DNS Query message to the EASDF.

9.	If the DNS Query message matches the DNS Query notification condition in a rule to handle DNS queries from the UE, the EASDF sends the notification to SMF by invoking Neasdf_DNSContext_Notify Request (AS FQDN, [DNS Query message]).

The EASDF sends the AS FQDN, which is the one UE requested to be resolved in the DNS Query message.

For Option C, the EASDF also includes the DNS Query message in the request.

10.	The SMF responds with Neasdf_DNSContext_Notify Response.

For Option A, the SMF includes corresponding ECS option in the response message. For Option B, the SMF includes corresponding local DNS Server IP address in the response message. The EASDF may as well be instructed to simply forward the DNS Query to a pre-configured DNS server/resolver.

Step 11 are skipped for option C:

11a.	The EASDF handles the DNS Query message received from the UE as the following:

-	For Option A, the EASDF adds the ECS option into the DNS Query message as specified in RFC 7871[y] and sends it to C-DNS server;

-	For Option B, the EASDF sends the DNS Query message to the Local DNS server.

If neither notification condition nor ECS option/Local DNS server provided by the SMF matches the requested FQDN in the DNS Query, the EASDF may simply forward the DNS Query to a pre-configured DNS server/resolver.

11b.	EASDF receives DNS Response from the DNS server.

Steps 12 are performed for option C only:

12a.	If the UPF used to forward DNS messages between the Local DNS Server and the SMF has not been selected, the SMF selects an UPF and configures the UPF with proper rules for such purpose.

NOTE: 	The UPF that forwards the DNS Query message can be a UPF different from the Local PSA UPF inserted in step 17.

12b.	The SMF forwards the DNS Query message to the Local DNS Server via the UPF.

12c.	The local DNS Server sends the DNS Response message to the SMF via UPF.

12d.	The SMF invokes Neasdf_DNSContext_Update Request (DNS Context ID, DNS Response) to forward the DNS Response message to the EASDF.

12e.	The EASDF responds with Neasdf_DNSContext_Update response.

At this point the EASDF has received the DNS response for the UE request.

13.	The EASDF may send an EASDF notification to SMF by invoking Neasdf_DNSContext_Notify request (EAS information).

If the EAS IP address or the FQDN in the DNS Response message matches the notification condition, the EASDF sends the notification to SMF. The EAS information includes EAS IP address(es). The EASDF does not  send the DNS Response message to the UE but waits for SMF instructions (step 16).

14.	The SMF invokes Neasdf_DNSContext_Notify Response service operation. SMF instructs EASDF to cache the DNS Response.

15.	The SMF may perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

Based on received EAS information and other UPF selection criteria as specified in clause 6.3.3 in TS 23.501[2], the SMF may perform UL CL/BP and Local PSA selection and insertion as described in TS 23.502[3].

16.	The SMF invokes Neasdf_DNSContext_Update Request (forward DNS response indication).

The forward DNS response indication is used to indicate the EASDF to forward the cached DNS Response to UE.

17.	The EASDF responds with Neasdf_DNSContext_Update Response.

18.	The EASDF sends the DNS Response to UE.

##### 6.2.3.2.3	EAS discovery procedure with local DNS server/resolver

This procedure applies to Option D and E in clause 6.2.3.2.1.

Figure 6.2.3.2.3-1: EAS discovery with local DNS server/resolver

1.	The SMF inserts UL CL/BP and Local PSA.

UL CL/BP/Local PSA insertion can be triggered by DNS messages as described in clause 6.2.3.2.2. Or, the SMF may pre-establish the UL CL/BP and Local PSA before the UE sends out any DNS Query message. The UL CL/BP and Local PSA are inserted as described in TS 23.502 [3].

When the UL CL/BP and Local PSA are inserted, the SMF configure the UL CL/BP based on which option is to be used for DNS Query handling:

-	For Option D, the SMF configures the UL CL/BP with the traffic routing rule (including e.g. Local DNS server address) to forward UE packets destined to the local DN to the Local PSA. The packets destined to local DN includes DNS Query messages destined to local DNS Server.

-	For Option E, the SMF configures the UL CL/BP to forward the DNS Query associated with the certain FQDN to the Local PSA, and configure the Local PSA to forward the DNS queries to the dedicated local DNS resolver.

Steps 2 and 3 are performed for option D, and skipped for option E:

2.	The SMF sends PDU Session Modification Command (Local DNS Server Address) to UE.

If, based on operator’s policy, the Local DNS Server IP Address in the local Data Network needs to be notified to UE, the SMF sends PDU Session Modification Command (Local DNS Server Address) to UE.

3.	The UE responds with PDU Session Modification Complete.

The UE configures the Local DNS Server as DNS server for the PDU Session. The UE sends the following DNS Query message to the indicated Local DNS Server.

4.	UE sends DNS Query message.

5.	The DNS Query message is forwarded to the local DNS Server and handled as described in following:

-	For Option D, the target address of the DNS Query is the IP address of the Local DNS Server. The DNS Query is forwarded to the Local DNS Server by UL CL/BP and Local PSA.

-	For Option E:

a.	The local DNS resolver modifies the packet's destination IP address (corresponding to EASDF) to that of the L-DNS and stores the original IP address (EASDF-IP) and the packet's source IP address (corresponding to UE's IP address) to its own (i.e. the local DNS resolver's) IP address and stores the original source IP address (UE-IP) for later processing.

b.	The local DNS resolver then forwards the modified DNS Query to the L-DNS and processes as follows:

-	If the L-DNS can resolve the IP address for the requested FQDN of EAS, it responds to the local DNS resolver with the desired IP address of the local EAS.

-	If the L-DNS cannot resolve the IP address for the requested FQDN of EAS but it is connected to a C-DNS, it communicates with the C-DNS to recursively resolve the EAS IP address.

6.	The Local PSA receives DNS Response message from local DNS server, it forwards it to the UL CL/BP, and the UL CL/BP forwards the DNS Response message to UE based on destination IP address of the DNS Response message(i.e. UE IP address).

********************************* End Change ****************************