3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2100129

Elbonia, February 24 – March 09, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	KI#4: Conclusion update for the solutions

Document for:	Approval

Agenda Item:	8.4.1

Work Item / Release:	FS_eNS_ph2 / Rel-17

Abstract: This contribution is proposed to update the conclusion of KI#4.

1. Introduction/Discussion

As concluded last meeting, for KI#1 and KI#2, A new NF is defined to support the storing of network slice information (i.e., the maximum number of UEs/PDU Sessions allowed to be served by a network slice), and managing, updating and enforcing the network slice information for the purpose of network slice access control. The new NF can be deployed as standalone or co-located within existing NF.

It is proposed to update the KI#4 conclusion to clarify that the new defined NF(s) will expose the network slice information to the AF.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-40.

# * * * * First change * * * *

8.4	Conclusion for Key Issue #4

For Key Issue #4 (Support for network slice quota event notification in a network slice):

-	The new 5GC NF (Control Plane NF within the 5G Core Network) has an interface with AF (optionally via NEF) and manages event subscription/notification from the AF for the network slice related quota on the number of registered UEs, number of established PDU sessions and/or limitation of data rate per network slice. The AF is allowed to set the triggering condition per S-NSSAI (not per UE), e.g. threshold, in terms of number of registered UEs or number of established PDU Sessions for the notification sent by the new 5GC NF.

-	The new 5GC NF determines whether event notification triggering condition is met based on data received from other 5GC NFs via service based interface. If the triggering condition is met, e.g. a specified threshold is reached, the new 5GC NF sends notification to the AF.

NOTE:	Limitation of data rate per network slice aspect will be determined based on the conclusion of KI#5 in a normative phase.

# * * * * End of changes * * * *