e3GPP TSG-WG SA2 Meeting #143E e-meeting  	S2-2100133r01

# Elbonia, February 24 – March 09, 2021	(revision of S2-200xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0139 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 5GC-MO-LR Procedures for Periodic Location report to UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_eLCS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-02-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the current specificaiton, there are quite a few procedures for different use cases. However, for the MO-LR procdure, it is observed that when the UE intends to locate itself, the UE has to send one MO-LR request in order to get one MO-LR Response(i.e., one location estimation ). This is not suitable for the periodic location service, which the UE has to get its latest location for a certain period. - a worker is working in the dagerous zone, the UE has to get its location in for every 1s. - a AVG is moving with 3m/s , the UE has to get its location for every 300ms(i.e., moving with 1m). To better support the positioning case in industry, it is proposed to capture a new procedure for perodic location report. |  |  |  |  |  |  |  |  |
| Summary of change: |  | a new procedure for Periodic Location report to UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current procedure doesn’t fit the industry use case, i.e., waste signaling and increase latency |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.X	5GC-MO-LR  Procedures for Periodic Location report to UE

6.x.1	Initiation of the Periodic Location report to UE

Figure 6.x.1-1: Initiation of the Periodic Location report to UE

1.	If the UE is in CM-IDLE state, UE instigates the UE triggered Service Request as defined in clause 4.2.3.2 of TS 23.502 [19] in order to establish a signalling connection with the AMF. If the UE is in CM-Connected state, the step is skipped.

2.	The UE sends an MO-LR Request message included in a UL NAS TRANSPORT message. The MO-LR Request may optionally include an LPP positioning message.  Location service type is set to location estimate of the UE. LCS requested QoS information (e.g. accuracy, response time, LCS QoS Class) is included in this message. In addition, the duration of periodic report and the interval of the periodic report is included in this message.

3.	 The AMF selects an LMF as described in clause 5.1.

4.	The AMF invokes the Nlmf_Location_DetermineLocation service operation towards the LMF. The service operation includes an LCS Correlation identifier, the serving cell identity, an indication that location estimate is requested, the duration of periodic report, the interval of the periodic report and any embedded LPP message in the MO-LR Request. The service operation may include the requested QoS and Supported GAD shapes. Once an AMF has selected an LMF it must continue to use that LMF for the duration of the session.

5.	When periodic report can be supported, the LMF returns the Nlmf_Location_DetermineLocation Response towards the AMF. The service operation includes the LCS Correlation identifier, information about the positioning method, the accepted duration of periodic report and the accepted interval of the periodic report.

After the Nlmf_Location_DetermineLocation Response is sent, the LMF starts the step 7 and step 8.

6.	The AMF sends an MO-LR Response message included in a DL NAS TRANSPORT message. The response includes the the LCS Correlation identifier, the accepted duration of periodic report and the accepted interval of the periodic report. AMF may record charging information.

Step 7 and step 8 are executed until accepted duration of periodic report expires, with the interval of the periodic report as provided by the LMF in step 5.

7.  The procedure described in clause 6.11 is performed to estimate the location of the UE.

8.	The LMF periodically provides the latest estimation to the UE in the Location Update message. The Location Update message also includes, if this was obtained, the accuracy of the location estimation.

6.x.2	Cancellation of the Periodic Location report

Figure 6.x.2-1: Initiation of the Cancellation of the Periodic Location report

1.	The UE performs a UE triggered service request as defined in clause 4.2.3.2 of TS 23.502 [19] if in CM-IDLE state in order to establish a signalling connection with the AMF.

2.	The UE sends a Cancel Location request message to the LMF,  which is transferred to the serving AMF in the a UL NAS TRANSPORT message and is further delivered to the LMF using an Namf_Communication_N1MessageNotify service operation.

3.	The LMF returns an acknowledgment in the Namf_Communication_N1N2MessageTransfer the serving AMF. The AMF relays the acknowledgement in the DL NAS TRANSPORT message

4.	The LMF informs serving AMF about the cancellation of the Periodic Location report in the   Nlmf_Location_EventNotify message.

# * * * * Second change * * * *

8.3.1	General

The following table shows the LMF Services and LMF Service Operations.

Table 8.3.1-1: List of LMF Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nlmf_Location | DetermineLocation | Request/Response | AMF |
|  | EventNotify | Notify | GMLC, AMF |
|  | CancelLocation | Request/Response | AMF |
|  | LocationContextTransfer | Request/Response | LMF |
| Nlmf_Broadcast | CipheringKeyData | Notify | AMF |

# * * * * Third change * * * *

8.3.2.3	Nlmf_Location_EventNotify service operation

Service operation name: Nlmf_Location_EventNotify.

Service operation description: Allow the consumer NF to get notified about the geodetic and optionally civic location of a target UE when some certain events are detected,

-	the events implicitly subscribed by the AMF using Nlmf_Location_DetermineLocation service operation, or

-	 the cancellation of reporting of periodic or triggered location events, or

-	the cancellation of the 5GC-MO-LR  Procedures for Periodic Location report to UE.

Input, Required: Notification Correlation ID, UE (SUPI and if available GPSI), Type of event.

Input, Optional: Geodetic Location, Civic Location, Position Methods Used (in the case of success indication provided), Notification Target address, Serving LMF identification, Failure Cause (in the case of failure indication provided).

Output, Required: None.

Output, Optional: Success/Failure indication.

See clause 6.3.1 and clause 6.3.2 for examples of usage of this service operation.

# * * * * End of changes * * * *