3GPP TSG-SA WG2 Meeting #143E e-meeting	S2-2100162r02

Elbonia, February 24 – March 09, 2021 	(Revision of )

# Title:	Draft LS Response on the input parameters of the LCS subscription request from an AF via NEF

# Response to:	(S2-2100020/C3-205448) LS on the input parameters of the LCS subscription request from an AF via NEF

# Release:	Release-16

# Work Item:	5G_eLCS

Source:	SA WG2

To:	CT WG3

Cc:	CT WG4

Contact Person:

Name:	Åke Busin

Tel. Number:

E-mail Address:	ake.busin@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:  S2-2100164

- Overall Description:

This LS provides answers to three question issued by CT3. For convenience the background information from CT3 is also included below.

“CT3 has observed that the inclusion of the parameters "External Client Type" and "LCS Service Type" as input parameters to be provided by the AF to the NEF when creating a LCS monitoring event subscription is not clear enough in the current Stage 2 specifications.

Indeed, step 1 of clause 6.1.2 of 3GPP TS 23.273 (cf. extract below) states that the "LCS Service Type" should be part of the parameters provided by the AF to the GMLC via the NEF, when sending a location monitoring event subscription.

1.	The LCS Client or the AF (via NEF) sends a request to the (H)GMLC for a location and optionally a velocity for the target UE which may be identified by an GPSI or an SUPI. The request may include the required QoS, supported GAD shapes, LCS client type, LCS service type (see TS 22.071 [2]) and other attributes. (H)GMLC (for 1a) or NEF (for 1b) authorizes the LCS Client or the AF for the usage of the LCS service. If the authorization fails, step 2-23 are skipped and (H)GMLC (for 1a) or NEF (for 1b) responds to the LCS Client or the AF the failure of the service authorization in step 24. In some cases, the (H)GMLC derives the GPSI or SUPI of the target UE and possibly the QoS from either subscription data or other data supplied by the LCS Client or AF.

At the same time, this parameter is not listed in clause 5.5 of 3GPP TS 23.273, which normally lists all the input parameters that can be provided by an AF or an LCS client. The "External Client Type" parameter is also not present. In addition, Table 7.2.1-1 in clause 7.2.1 indicates that both the "LCS Service Type" and the "External Client Type" should be stored in the GMLC for an External LCS Client.

In addition, 3GPP TS 29.515 (i.e. Ngmlc API), under the remit of CT4, defines these parameters as possible input parameters to be provided by the NEF to the GMLC (as per Table 6.1.5.2.2-1 of 3GPP TS 29.515) as part of the Ngmlc_Location_ProvideLocation service operation. The "External Client Type" is even defined as a mandatory input parameter. In the meantime 3GPP TS 29.122, under the remit of CT3, currently does not define these parameters as possible input parameters to be provided by the AF when creating an LCS monitoring event subscription (as per Table 5.3.2.1.2-1 of 3GPP TS 29.122), which is a clear misalignment.

Therefore, CT3 has the following questions:”

Q1:	Should these parameters, i.e. "LCS Service Type" and the "External Client Type", be provided by an AF to the NEF when creating an LCS monitoring event subscription? This would enable the NEF to provide them to the GMLC in the Ngmlc_Location_ProvideLocation request.

ANSWER: The parameters, "LCS Service Type" and the "External Client Type" shall not be provided by AF. This has been corrected/clarified in attached CR.

Q2:	If the answer to the Q1 is yes, what is the behaviour of the NEF and GMLC when receiving these parameters? What are the checks, if any, that need to be performed?

ANSWER: N/A as answer to Q1 is no.

Q3:	If the answer to Q1 is no, then how these parameters are derived by the NEF? Should these parameters only be stored and derived by the GMLC? In this case, this would mean that these parameters should be removed from the list of possible input parameters to be provided by the NEF in the Ngmlc_Location_ProvideLocation request.

ANSWER:   The parameter "External Client Type" may be provisioned in the NEF per AF. It may be used for privacy authorization in GMLC (23.273 6.1.2) or NEF (23.273 6.5.1). The parameter needs to be included in Ngmlc_Location_ProvideLocation request.

The parameter “LCS Service Type" may be if used mapped in GMLC from attribute “Service Identity” (see (23.273 6.1.2) and may be used for privacy authorization of an LCS Client in GMLC (see 23.273 7.1).  The parameter does not need to be included in Ngmlc_Location_ProvideLocation request.

2. Actions:

To CT WG3 group.

ACTION: 	SA2 kindly requests CT3 to take response into consideration.

3. Date of Next SA2 Meetings:

3GPP TSG SA2 #144e	April 12-16                 2021	E-Meeting

3GPP TSG SA2 #145e	May 17-28                 2021	E-Meeting