SA WG2 Meeting SA2#143-e	S2-2100178r02

24 February - 9 March, 2021, Electronic meeting	(revision of S2-2009412)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.203 | CR | 1133 | rev | 1 | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multimedia Priority Service (MPS) Phase 2 support for Data Transport Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, CISA ECD, AT&T, T-Mobile USA, Nokia, Nokia Shanghai-Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS2 |  |  |  |  |  | Date: |  |  | 18 02 2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | MPS for Data Transport Service provides the Service User with priority for applications using the default bearer in the EPS, to one or more selected active Packet Data Networks (PDNs), in periods of severe network congestion during which normal commercial data service is degraded. This CR supports stage 1 requirements, TS 22.153, clause 9.3.1: The system shall support: - MPS for DTS for an authorized Service User using a UE with a subscription for MPS, and - MPS for DTS for an authorized Service User using a UE that does not have an MPS subscription. TS 23.203 currently does not support MPS for DTS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added clarifications in various MPS clauses with respect to the existing Priority EPS Bearer Service and IMS Priority service; and - Added new clause on Bearer Priority for MPS for Data Transport Service. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for MPS for Data Transport Service in Release 17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.11.2, 6.1.11.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | CR for TS 23.401 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR updates endorsed Draft CR S2-2009412 from meeting SA2#142-e, November 16-20, 2020 based on additional discussion with stakeholders. |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 6.1.11.2	PCC rule authorization and QoS rule generation

For MPS service, the PCRF shall generate the corresponding PCC/QoS rule(s) with the ARP/QCI parameters as appropriate for the prioritized service.

For non-MPS service, the PCRF shall generate the corresponding PCC/QoS rule(s) as per normal procedures, without consideration whether the MPS Priority EPS Bearer Service is active or not, but upgrade the ARP/QCI values suitable for MPS when the Priority EPS Bearer Service is invoked. When the Priority EPS Bearer Service is revoked, the PCRF shall change the ARP/QCI values modified for Priority EPS Bearer Service to appropriate values.

NOTE 1: The above statements for the Priority EPS Bearer Service are also applicable for the MPS for Data Transport Service.

Whenever one or more AF sessions of an MPS service are active within the same PDN connection, the PCRF shall ensure that the ARP priority level of the default bearer is at least as high as the highest ARP priority level used by any authorized PCC rules belonging to an MPS service. If the ARP pre-emption capability is enabled for any of the authorized PCC rules belonging to an MPS service, the PCRF shall also enable the ARP pre-emption capability for the default bearer.

NOTE 2:	This ensures that services using dedicated bearers are not terminated because of a default bearer with a lower ARP priority level or disabled ARP pre-emption capability being dropped during mobility events.

NOTE 3:	This PCRF capability does not cover interactions with services other than MPS services.

SECOND AND LAST CHANGE

#### 6.1.11.X	Bearer priority for MPS for Data Transport Service

MPS for Data Transport Service enables the prioritization of all traffic on the default bearer and other bearers upon AF request. The QoS modification to the default bearer and other bearers is done based on operator policy and regulatory rules by means of local PCRF configuration.

NOTE 1: If no configuration is provided, MPS for Data Transport Service applies only to the default bearer.

The AF authorizes MPS for Data Transport Service requests. Subsequently, the AF forwards the MPS for Data Transport Service request to the PCRF and indicates to the PCRF whether the request is for invoking or revoking MPS for Data Transport Service. The PCRF shall respond to the AF indicating that it received the AF request. The PCRF shall not perform any subscription check for MPS for Data Transport Service requests.

The PCRF shall at the activation of MPS for Data Transport Service over Rx, perform the same steps as described in clause 6.1.11.3 for the activation of the Priority EPS Bearer Service.

NOTE 2:  	To keep the PCC rules bound to the default bearer, the PCRF can either modify the ARP/QCI of these PCC rules accordingly or set the Bind to Default Bearer PCC rule attribute.

The PCRF shall inform the AF that it successfully acted upon the MPS for Data Transport Service invocation/revocation request.

The PCRF shall at the deactivation of MPS for Data Transport Service over Rx perform the same steps described in clause 6.1.11.3 for the deactivation of the Priority EPS Bearer Service.

If the bearers are deactivated for other reasons than an AF request, the PCRF shall notify the AF by terminating the Rx session.

The AF may also request an SDF for priority signalling between the UE and the AF, where the AF includes the Priority indicator over Rx, in order to enable the PCRF to set appropriate QoS values for the signaling bearer.

END OF CHANGES