SA WG2 Meeting #143E (e-meeting)	S2-2100212r7

Feb 24 – Mar 09, 2021, Elbonia

Source:	Ericsson, Intel

Title:	KI#4: Update Conclusion for UE-to-UE relay

Document for:	Approval

Agenda Item:	8.8.1

Work Item / Release:	FS_5G_ProSe/Rel-17

Abstract of the contribution: This contribution proposes to clean some inconsistencies in the conclusion of KI#4 UE-to-UE relay.

# r02: remove duplicated statement “The reselection criteria are to be coordinated with RAN2 WG.” since  a new NOTE is added.

# Discussion

In TR 23.752-100, there are several inconsistencies in the conclusion of KI#4 UE-to-UE relay.

Inconsistency #1: there are recommended solutions for L3 UE-to-UE relay discovery. However, there is no recommended solutions for L2 UE-to-UE relay discovery, which may create confusion to other WGs, e.g. RAN2

Proposal #1: Since Sol#8 and Sol#11 can work for both L2 and L3 UE-to-UE relay, it is proposed to make Sol#8 and Sol#11 for recommended solutions for L2 UE-to-UE relay discovery.

Inconsistency #2: Regarding L3 UE-to-UE relay reselection, there exist such statement “The reselection criteria are to be coordinated with RAN2 WG”. However, in the conclusion of L2 UE-to-UE relay, the statement is written in a NOTE.

Proposal #2: Adding NOTE in the conclusion of L3 UE-to-UE relay stating that It is left to RAN WG2 to decide the radio criteria on Relay reselection.

# Proposal

It is proposed to include the following in TR 23.752-100.

* * * Start of changes * * *

## 8.4	Key Issue #4: Support of UE-to-UE Relay

The followings are taken as conclusion for Layer-3 UE-to-UE relay:

-	No showstopper has been identified by SA2 for L3 UE-to-UE solution. SA WG2 recommends L3 UE-to-UE Relay proceed into normative work, subject to RAN2 and SA3 conclusion.

-	L3 UE-to-UE relay solution can support relaying of IP and non-IP traffic. For IP traffic, the IP addresses of the UEs can be either assigned by the relay (as described in sol#10) or self-assigned (as described in sol#32). For Non-IP traffic, it can be either handled via IP encapsulation or without IP encapsulation (as described in sol#49).

NOTE 1:	It is left to SA WG3 to decide the details of how to support end-to-end security between the Source UE and Target UE for IP traffic and Non-IP traffic.

-	UE-to-UE Relay discovery and selection are supported by:

-	Model A discovery (as described in sol#11);

-	Model B discovery (as described in sol#8, Alt 2);

-	UE-to-UE Relay reselection

The relay reselection can be viewed just like redoing the relay selection as described in Sol#8 or be performed as described in Sol#50.

NOTE 2: It is left to RAN WG2 to decide the radio criteria on Relay reselection.

-	QoS support

End-to-end QoS support for Remote UE is provided by splitting the QoS between the two PC5 links between the source UE and target UE. QoS splitting configuration can be provided from PCF as part of policy to both Remote UE and Relay UE or the QoS splitting can be managed by the Relay UE based on the end-to-end QoS needs. For QoS handling, Sol#31 can be considered as the starting point for the normative work.

The following are taken as conclusions for the L2 UE-to-UE Relay:

-	No showstopper has been identified by SA WG2 for L2 UE-to-UE solution. SA WG2 recommends L2 UE-to-UE Relay proceed into normative work.

NOTE  2:	The operation procedures for supporting the L2 UE-to-UE Relay need coordination with RAN2 to decide how the UE-to-UE Relay performs the data/signalling routing.

-	UE-to-UE Relay discovery and selection are supported by:

-	Model A discovery (as described in sol#11);

-	Model B discovery (as described in sol#8, Alt 2);

-	The connection establishment in Sol#9 can be taken as baseline.

-	For QoS handling, Sol#31 can be taken as baseline.

NOTE 3:	It is left to RAN WG2 to support the QoS enforcement in AS layer.

-	For Relay reselection, the negotiated UE-to-UE Relay reselection in Sol#50 and the Relay selection in Sol#8 can be used under different conditions. Both Sol#50 and Sol#8 can be taken as baseline.

NOTE 4:	It is left to RAN WG2 to decide the radio criteria on Relay reselection.

NOTE 5:	It is left to RAN WG2 and SA WG3 to decide the details of how to support end-to-end security between the Source UE and Target UE for IP traffic and Non-IP traffic.

NOTE 6: It is left to RAN2 to decide the frame format defined in Sol#11 for L2 and L3 relay during normative work.

* * * End of changes * * *