SA TSG-SA2 Meeting #143e	S2-2100221

2021-02-24 – 2021-03-09, Elbonia

Source:	Ericsson, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Samsung, Broadpeak; Futurewei

Title:	Connectivity Models

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEdge_5GC /Rel-17

Abstract of the contribution: This contribution adds the connectivity models in the new EC TS.

# 1	Introduction

This contribution adds the connectivity models in the new EC TS.

# 3	Proposal

*******************  Start changes  ********************

4.3	Connectivity models

5GC supports the following connectivity models to enable Edge Computing:

-	Distributed Anchor Point: The PDU Session Anchor UPF is in a local site, i.e. close to the UE location. The PSA UPF may be changed due to e.g. UE mobility.

-	Session Breakout: The PDU Session has a PDU Session Anchor UPF in a central site and a PDU Session Anchor UPF in the local site. The Central PSA UPF provides the IP Anchor Point in case of UL Classifier is used. The Edge Computing application traffic is selectively diverted to the Local PDU Session Anchor UPF using UL Classifier or multi-homing Branching Point technology. The Local PSA UPF may be changed due to e.g. UE mobility.

-	Multiple PDU Sessions: Edge Computing applications use PDU Session with the PDU Session Anchor UPF in the local site. The rest of applications use PDU Session with Central PDU Session Anchor UPF. The mapping between applications and PDU Sessions is steered by the URSP rules. The Local PSA UPF may be changed due to e.g. UE mobility.

These three connectivity models are illustrated in the figure below:

Figure 4.3-1: 5GC Connectivity Models for Edge Computing

********************  End  changes  ********************