SA TSG-SA2 Meeting #143e	S2-2100221r03

2021-02-24 – 2021-03-09, Elbonia

Source:	Ericsson, Nokia, Nokia Shanghai Bell, Huawei, HiSilicon, Samsung, Broadpeak; Futurewei

Title:	Connectivity Models

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEdge_5GC /Rel-17

Abstract of the contribution: This contribution adds the connectivity models in the new EC TS.

# 1	Introduction

This contribution adds the connectivity models in the new EC TS.

# 3	Proposal

*******************  Start changes  ********************

4.3	Connectivity models

5GC supports the following connectivity models to enable Edge Computing:

-	Distributed Anchor Point: The PSA UPF is in a local site, i.e. close to the UE location. The PSA UPF may be changed due to e.g. UE mobility.

-	Session Breakout: The PDU Session has a PSA UPF in a central site and a PSA UPF in the local site. The C-PSA UPF provides the IP Anchor Point if UL Classifier is used. The Edge Computing application traffic is selectively diverted to the L-PSA UPF using UL Classifier or multi-homing Branching Point technology. The L-PSA UPF may be changed due to e.g. UE mobility.

-	Multiple PDU Sessions: Edge Computing applications use PDU Session with the L-PSA UPF in the local site. The rest of applications use PDU Session with C-PSA UPF. The mapping between applications and PDU Sessions is steered by the URSP rules. The L-PSA UPF may be changed due to e.g. UE mobility.

These three connectivity models are illustrated in Figure 4.3-1:

Figure 4.3-1: 5GC Connectivity Models for Edge Computing

********************  End  changes  ********************